/*******************************************************************************

*

*    Service Processor Module

*

*    The Service Processor Module (SPM) attaches to the system bus and

*    provides various service functions for the system. These functions

*    include system console interfaces, front panel interface, real time

*    clock, system clock interrupt service, power supply controls, floppy

*    disk controller, system wide interrupt dispatcher, various system

*    monitor functions (power, temperature), and system diagnostic capabil-

*    ities.

*    The SPM consists of two boards, the main board which plugs into the CSS

*    bus backplane, and the real world interface board which contains all

*    connectors for the serial ports, the power supply control connectors,

*    and the temperature sensor connectors. The real world interface board

*    is connected to the main board via a third 96pin din connector at the

*    bottom of the main board. The real world interface board resides behind

*    the CSS backplane in the interface slot behind the Service Processor

*    Main board.

*

******************************************************************************/

/*******************************************************************************

*                        Service Processor Memory Map

********************************************************************************

*

* 31302928 27262524 23222120 19181716 15141312 111098 7654 3210 - address bits

*

*-------------------------------------------------------------------------------

* 0 x x x  0 0 0 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 Eprom

* 0 x x x  0 0 0 0  0 0 0 0  0 0 0 0  1 1 1 1  1 1 11 1111 1111 - 512k <----

* 0 x x x  0 0 0 0  0 0 0 0  0 0 0 1  1 1 1 1  1 1 11 1111 1111 - 1M

*-------------------------------------------------------------------------------

* 0 x x x  0 0 0 0  0 0 0 0  0 0 1 0  0 0 0 0  0 0 00 0000 0000 Spare

* 0 x x x  0 0 0 0  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

*    0x01000000

*    0x0103ffff

* 0 x x x  0 0 0 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 Sram (256k)

* 0 x x x  0 0 0 1  0 0 0 0  0 0 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  0 0 0 1  0 0 0 0  0 1 0 0  0 0 0 0  0 0 00 0000 0000 Spare

* 0 x x x  0 0 0 1  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  0 0 1 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 ACRW

* 0 x x x  0 0 1 0  0 0 0 0  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 reserved

* 0 x x x  0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  0 0 01 0000 0000 local A/D conv

* 0 x x x  0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  0 0 10 0000 0000 local CIO

*-------------------------------------------------------------------------------

* 0 x x x  0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  0 0 11 0000 0000 reserved for

* 0 x x x  0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  0 1 11 0000 0000 local devices

*-------------------------------------------------------------------------------

* 0 x x x  0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  1 0 00 0000 0000 aux. SCC 0

* 0 x x x  0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  1 0 01 0000 0000 aux. SCC 1

* 0 x x x  0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  1 0 10 0000 0000 aux. CIO

* 0 x x x  0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  1 0 11 0000 0000 aux. A/D conv

*-------------------------------------------------------------------------------

* 0 x x x  0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  1 1 00 0000 0000 reserved for

* 0 x x x  0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  1 1 11 0000 0000 aux. devices

*-------------------------------------------------------------------------------

* 0 x x x  0 0 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 real time clock

* 0 x x x  0 0 1 1  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111 and NV ram

*-------------------------------------------------------------------------------

* 0 x x x  0 1 0 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 reserved

* 0 x x x  0 1 0 0  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  0 1 0 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 clock control

*-------------------------------------------------------------------------------

* 0 x x x  0 1 0 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0001 reserved

* 0 x x x  0 1 0 1  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  0 1 1 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 reserved

* 0 x x x  0 1 1 0  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 Wr cntl 0

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 Wr cntl 0 (read)

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0010 0000 Wr cntl 1

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0010 0000 Wr cntl 1 (read)

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0100 0000 Wr cntl 2

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0100 0000 Wr cntl 2 (read)

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0110 0000 Reserved

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 1000 0000 CSS command reg

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 1010 0000 Status reg

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 1100 0000 CSS error reg

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 1110 0000 Dispatcher error

*                                                               register

*-------------------------------------------------------------------------------

* 0 x x x  0 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 1110 0001 Spare

* 0 x x x  0 1 1 1  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 0 0 0  1 0 0 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 Map RAM loc 0

* 0 0 0 1  1 0 0 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000  "   "   "  1

* 0 0 1 0  1 0 0 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000  "   "   "  2

* 0 0 1 1  1 0 0 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000  "   "   "  3

* 0 1 0 0  1 0 0 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000  "   "   "  4

* 0 1 0 1  1 0 0 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000  "   "   "  5

* 0 1 1 0  1 0 0 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000  "   "   "  6

* 0 1 1 1  1 0 0 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000  "   "   "  7

*-------------------------------------------------------------------------------

* 0 x x x  1 0 0 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 Floppy Disk

* 0 x x x  1 0 0 0  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  1 0 1 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 reserved

* 0 x x x  1 0 1 0  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  1 0 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 I.D. Idle Queue

* 0 x x x  1 0 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 1111

*-------------------------------------------------------------------------------

* 0 x x x  1 0 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0001 0000 unused

* 0 x x x  1 0 1 1  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  1 1 0 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 I.D. Queue Ram

* 0 x x x  1 1 0 0  0 0 0 0  0 0 0 0  0 0 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  1 1 0 0  0 0 0 0  0 0 0 0  0 1 0 0  0 0 00 0000 0000 unused

* 0 x x x  1 1 0 0  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  1 1 0 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 I.D. Pointer Ram

* 0 x x x  1 1 0 1  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

* 0 x x x  1 1 1 0  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 Int Dispatcher

* 0 x x x  1 1 1 0  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111 Misc. Rams

*-------------------------------------------------------------------------------

* 0 x x x  1 1 1 1  0 0 0 0  0 0 0 0  0 0 0 0  0 0 00 0000 0000 reserved

* 0 x x x  1 1 1 1  1 1 1 1  1 1 1 1  1 1 1 1  1 1 11 1111 1111

*-------------------------------------------------------------------------------

/******************************************************************************

*

*    EPROM

*

*    The EPROM is 8 bits wide and can be read by the processor on data bits

*    24 to 31. There are 2 sockets for the EPROM, one or both can be populat-

*    ed with either 27256s (32k x 8 each), or 27512s (64k x 8 each) or

*    271001s (128k x 8 each). Address bits Add.00 to Add.15 or Add.16 or

*    Add.17 are used to address the EPROM. The decoding of the high bit is

*    controlled by the Promsize field in control register 2. This allows the

*    two EPROM chips to be contiguous regardless of their size. (See Control

*    register 2 description for details).

*

******************************************************************************/

#define PROMSIZ    0x10000           /* cpu monitor prom*/

#define PROMSTART  0x00000000        /* start of prom area */

/*******************************************************************************

*

*    Static Ram

*

*    The SRAM is 32 bits wide and consists of 8 or 4 32k x 8 static RAM chips

*    for 64k x 32 or 32k x 32 bits of memory. It is addressed with address

*    bits Add.02 to Add.17.

*

******************************************************************************/

#define SRAMSTART 0x01000000

#define SRAMSIZE  0x40000

/* NOVRAM...

*/

#define NOVRAM	((struct novram *)(0x03000000))

#define NEW_NOVRAM	((struct new_novram *)(0x03000000))

/*******************************************************************************

*

*   Real Time Clock

*

*   The real time clock is a MK48T12 chip. This chip has a built in crystal

*   oscillator and a lithium battery. In addition to a real time clock, it

*   has 2K bytes of nonvolatile RAM. This RAM can be used to keep config-

*   uration information. The service processor does not have any switches.

*

******************************************************************************/

#define RTC ((struct rtc *)(0x030007f8))

/*******************************************************************************

*

*   Local A/D converter

*

*   The local A/D converter is an MC14442 and a TL431A voltage reference.

*   It is used to measure the temperature on the Service Processor (i.e. the

*   cardcage) and the 6 voltages present on the SPM(i.e. +5V main, +5V aux.,

*   +12V main, +12V aux., -12V main, and -12V aux.)

*

******************************************************************************/

#define ADC_CNTL ((unsigned short*)(0x02800102))

#define ADC_ADATA ((unsigned short*)(0x02800100))

#define ADC_SC    0x0100

#define ADC_CH0   0x0000        /* measures +5v for master system */

#define ADC_CH1   0x0001        /* reference voltage 4.5v must read FF */

#define ADC_CH2   0x0002        /* Measure +5v. aux */

#define ADC_CH3   0x0003        /* Measure +12v */

#define ADC_CH4   0x0004        /* Measure +12v aux */

#define ADC_CH5   0x0005        /* Measure -12v */

#define ADC_CH6   0x000e        /* Measure -12v aux */

#define ADC_CH7   0x000f        /* measure on board temperature. */

#define ADC_EOC   0x8000

#define ADC_MASK  0x00ff        /* mask for valid data. */

/*******************************************************************************

*

*    Local CIO

*

*    The local CIO is a Zilog 8036, used for various timing functions includ-

*    ing the CSS bus timeout and the main system clock interrupt.

*

******************************************************************************/

#define LOCCIO ((struct cio *)(0x02800200))

/*******************************************************************************

*

*    SCC

*

******************************************************************************/

#define ASCC ((struct ascc *)(0x02800800))

#define AUXASCC0B ((struct ascc *)(0x02800800))

#define AUXASCC0A ((struct ascc *)(0x02800820))

#define AUXASCC1B ((struct ascc *)(0x02800900))

#define AUXASCC1A ((struct ascc *)(0x02800920))

/*******************************************************************************

*

*    Clock Control (write only)

*

*    The Clock control port is a single bit port which is written with data

*    bit 0. Data.00 = 0 means the CPU is using its on-board 10Mhz clock,

*    Data.00 = 1 means the CPU is using the CSS bus clock divided by two as

*    its clock source. When switching from one clock source to the other,

*    approximately 1us after the write to the clock control port, the hard-

*    ware will issue a reset to the CPU, hold the reset for approximately

*    500us, and switch clocks during the reset. This is required to meet the

*    timing specifications of the 68020. In the process, the entire board

*    will be reset, so any VLSI chips will have to be reinitialized. The

*    memory, however, will keep its data valid. The state of the clock bit

*    (i.e. which clock is being used) can be read in the status register. If

*    the CSS bus clock is not running(e.g. the CSS bus is powered down), the

*    hardware will not select the CSS clock.

*

*      31 ---------- 1 0

*     +---------------+-+

*     |       x       |c|

*     +---------------+-+

*             |        |

*             |        +-------------- 0 - on board 10Mhz clock

*             |                        1 - CSS bus clock

*             +----------------------- unused

******************************************************************************/

#define CLKCNTL  ((unsigned *)(0x05000000))

/*******************************************************************************

*

*   CSS Command Register (read only)

*

*   The CSS command register is a 32 bit register which allows the local CPU

*   to read the value written to the SPM via a CSS write to addr 0xXXXXAAYY

*   by any other CSS module. X's are don't cares and AA will be captured by

*   the SPM and can be read in the Status register. YY must be of the form

*   000X XXXX. The SPM only decodes the top three bits of the least signif-

*   icant byte of CSS address. When another module writes to an address of

*   the form described above on the SPM, the 68020 recieves an interrupt

*   informing it that a CSS command has been received. The 68020 will read

*   the CSS command register and the status register to determine the data

*   of the command.

*

*      31 ---------- 0

*     +---------------+

*     |   command     |

*     +---------------+

*             |

*             +----------------------- command to SPM via CSS write

*

******************************************************************************/

#define CSSCMD ((unsigned *)(0x07000080))

/*******************************************************************************

*

*    Status Register (read only)

*

*    The status register is a 24 bit register, data bits 00 to 07 and 16 to

*    23 and 24 to 31 can be read by the 68020. Bits 24 to 27 are the bus slot

*    number given by the position of the SPM in the backplane. For diagnostic

*    purposes, the SPM does not have its slot id hardwired, so for the board

*    to function properly, the slot id must be read in the status register

*    and written in write control registers 0 and 1. Bit 31 in the Status

*    register is a status bit from the interrupt dispatcher state machine.

*    When this bit is 0, the interrupt dispatcher has suspended its dispatch-

*    ing function and the CPU is allowed to access the various RAMs in the

*    interrupt dispatcher. This is needed for initializing the interrupt

*    dispatcher and for diagnostic purposes. When the bit is a 1, the

*    interrupt dispatcher is in its normal operating mode and the CPU must

*    not access any of the dispatcher RAMs. Bit 30 is an indicator from the

*    floppy disk, floppy ready. Bit 29 is a status bit from the clock select

*    mechanism. When this bit is a 1, the 68020 is using the local clock,

*    when 0, the 68020 is using the CSS clock divided by 2. Bit 28 is reserv-

*    ed. Bits 00 to 07 are part of the CSS error register as follows:

*    Bit 00 has the state of the CSS bus nack signal at the time of the last

*    CSS bus error, bit 01 has the state of the CSS bus ack, bit 02 has the

*    state of the CSS arbiter grant error, bit 03 is the bad command bit.

*    This bit is a 0 if the SPM has detected a CSS bus protocol violation

*    (e.g. data parity is bad), it is a 1 if the SPM has not detected such

*    a violation but it has nevertheless detected a CSS bus error. Bit 04

*    is the state of the CSS arbiter burst signal. Bits 05 to 07 are reserv-

*    ed. Bits 16 to 23 are the CSS data byte 6 at the time of the most recent

*    CSS command write.

*

*      31  30  29 28 27 - 24 23 - 16

*     +--+----+--+--+-------+-------+

*     |ID|FRDY| c| x|slot # | byte 6|

*     +--+----+--+--+-------+-------+

*      |    |   |  |    |       |

*      |    |   |  |    |       +----- CSS data byte 6 at the time of the most

*      |    |   |  |    |              recent CSS command write.

*      |    |   |  |    +------------- bus slot # determined by position of

*      |    |   |  |                   SPM in backplane

*      |    |   |  +------------------ reserved

*      |    |   +--------------------- 1 = local clock

*      |    |                          0 = CSS clock

*      |    +------------------------- floppy ready

*      +------------------------------ from Interrupt Dispatcher state machine

*                                      0 - dispatcher has been suspended and

*                                          the cpu is allowed access to ID rams

*                                      1 - Int Dispatcher normal mode, cpu

*                                        should not access dispatcher RAM's

*

*      15-8 7-6     5       4      3      2    1    0

*     +----+---+--------+------+-------+-----+----+----+

*     |  x |   | active |burst*|bad cmd|gerr*|back|nack|

*     +----+---+--------+------+-------+-----+----+----+

*       |    |     |       |      |      |      |   |

*       |    |     |       |      |      |      |   +-state of CSS bus nack

*       |    |     |       |      |      |      |     signal at the last

*       |    |     |       |      |      |      |     CSS bus error

*       |    |     |       |      |      |      +-- state of CSS bus ack

*       |    |     |       |      |      +--------- state of CSS arbiter

*       |    |     |       |      |                 grant error

*       |    |     |       |      +---------------- bad command bit

*       |    |     |       |                    0 - SPM has detected a CSS bus

*       |    |     |       |                        protocol violation(data

*       |    |     |       |                        parity is bad)

*       |    |     |       |                    1 - SPM has not detected a

*       |    |     |       |                        violation, but detected a

*       |    |     |       |                        CSS bus error

*       |    |     |       +----------------------- state of CSS arbiter

*       |    |     |                                burst signal

*       |    |     +------------------------------- always a 1 except during

*       |    |                                      force ack,nack,grant errors

*       |    +------------------------------------- reserved

*       +------------------------------------------ unused

*

******************************************************************************/

#define STATUSREG ((unsigned *)(0x070000a0))

#define STAT_SLOTMASK  0x0f000000

#define STAT_SLOTSH    24

#define STAT_REG_IDRAM 0x80000000

#define STAT_IDLE_FULL 0x10000000		/* If set, idle-que is not full. */

#define STAT_FLOP_RDY 0x40000000		/* Mask for floppy ready. */

/*******************************************************************************

*

*   CSS Error Register (read only)

*

*   When the SPM detects an error on the CSS bus the hardware will interrupt

*   the 68020. The 68020 will read the CSS error information register to

*   determine the nature of the error. The conditions which cause such an

*   error are: bad data parity, bad type parity, destination error, source

*   error, invalid type, bus nack, or grant error. All error conditions

*   except the last two will latch in valid data for the CSS error informa-

*   tion register as shown below. Additional error information is captured

*   in the status register. When a bus error condition has been latched in

*   the error register no new error conditions can be latched until after

*   the bus error register has been read by the 68020.

*

*   31 - 27 26-25  24  23-20 19-16  15   14   13-8  7-0

* +--------+-----+----+-----+-----+----+-----+----+----+

* |CSS data|  x  |bpar|dest | src |derr|serr|btype|bpar|

* +--------+-----+----+-----+-----+----+-----+----+----+

*     |       |     |    |     |     |    |    |    |

*     |       |     |    |     |     |    |    |    +- bus data parity

*     |       |     |    |     |     |    |    |       0 - 7

*     |       |     |    |     |     |    |    +------ bus type 0 - 5

*     |       |     |    |     |     |    +----------- source error

*     |       |     |    |     |     +---------------- destination error

*     |       |     |    |     +---------------------- bus source 0 to 3

*     |       |     |    +---------------------------- bus dest 0 to 3

*     |       |     +--------------------------------- bus type parity

*     |       +--------------------------------------- reserved

*     +----------------------------------------------- CSS data bits 57,40-43

*

******************************************************************************/

#define CSSERROR    ((unsigned *)(0x070000c0))

#define DERR        15

#define SERR        14

#define BTYPE_SH    8

#define SRC_SH      16

#define DEST_SH     20

#define BPAR_SH     24

#define BPAR_MASK   0x01000000

#define DEST_MASK   0x00f00000

#define SRC_MASK    0x000f0000

#define BTYPE_MASK  0x00003f00

#define TYPE_SH     0x0b

#define TYPE_MASK   0x3800

#define SIZE_MASK   0x0700

#define SIZE_SH     0x08

/******************************************************************************* *

*  Dispatcher Error Register

*

*  When the Interrupt Dispatcher detects an error it will suspend its

*  operation and latch error information in this register.

*

*   31 - 24  23  22-20 19-18 17-16 15-12 11-8   7  6-0

* +--------+----+-----+-----+-----+-----+-----+---+---+

* | Int Vec|dint| err |  x  | pri |dest | src |int|src|

* +--------+----+-----+-----+-----+-----+-----+---+---+

*     |       |     |    |     |     |    |    |    |

*     |       |     |    |     |     |    |    |    +-- i/o bus source slot

*     |       |     |    |     |     |    |    |        (only valid if bit 7=1)

*     |       |     |    |     |     |    |    +------- i/o bus int

*     |       |     |    |     |     |    |             1 - int came from an

*     |       |     |    |     |     |    |                 i/o module

*     |       |     |    |     |     |    |             0 - int came from a CSS

*     |       |     |    |     |     |    |                 bus module

*     |       |     |    |     |     |    +------------ CSS bus source slot

*     |       |     |    |     |     +----------------- CSS bus dest slot

*     |       |     |    |     +----------------------- Interrupt priority

*     |       |     |    +----------------------------- reserved

*     |       |     +---------------------------------- error code

*     |       |                                         (see below for codes)

*     |       +---------------------------------------- Directed interrupt

*     +------------------------------------------------ Int vector number

*

*

*        error information bits 22 21 20

*                               -- -- --

*                                x  x  0 - request error

*                                1  0  x - ack error, bus error on ack response

*                                0  0  x - ack error, no previous requests

*                                x  1  1 - recieve error, fix pointers or reset

*                                          recieve

*

******************************************************************************/

#define DISPERROR     ((unsigned *)(0x070000e0))

#define DISP_ACK_ERR       1        /* insane ack, an ack for no good reason */

#define DISP_REQ_ERR       2        /* request with no int. pending */

/*      unused             3        /* recieve err  */

/*      unused             4        /* */

#define DISP_ACK_TOUT      5        /* timeout on ack response */

#define DISP_REQ_TOUT      6        /* timeout on request */

#define DISP_RCV_ERR       7

#define DISP_ERR_SH        20

/*******************************************************************************

*

*        Write Control Register 0

*

*   31-29  28    27    26    25    24   23-20  19-16

*  +-----+-----+-----+------+-----+-----+------+-----+

*  |     |inh7 |freez| dest |grant| ack |srcID*|srcID|

*  +-----+-----+-----+------+-----+-----+------+-----+

*     |     |     |     |     |     |      |     |

*     |     |     |     |     |     |      |     +------ out.src 0-3 source id

*     |     |     |     |     |     |      |             used by SPM when it is

*     |     |     |     |     |     |      |             issuing a command

*     |     |     |     |     |     |      +------------ out.src 0 to 3*

*     |     |     |     |     |     |                  complement of source id

*     |     |     |     |     |     +------------------- force ack/nak error.

*     |     |     |     |     |

*     |     |     |     |     +------------------------- force grant error.

*     |     |     |     |

*     |     |     |     +------------------------------- force bad dest.

*     |     |     |

*     |     |     +------------------------------------- Freeze CPU bit:

*     |     |                                            active 1 (Must have

*     |     |                                            CSS Reset rdy 1)

*     |     +------------------------------------------- Inhibit* CSS Bus

*     |                                                  errors, (level 7)

*     |												  capturing. CSS int's.

*     |

*     +------------------------------------------------- Reserved.

*

*

*

*     15-12  11-8  7-6  5-0

*   +------+------+---+-----+

*   |slotID|slotID|f10|Ftype|

*   +------+------+---+-----+

*      |      |    |    |

*      |      |    |    +---- forced type

*      |      |    |          (used if "use.forced.type" bit is active)

*      |      |    +--------- forced address bits 00 & 01

*      |      +-------------- slot id used by int dispatcher

*      |                      (tells SPM where to place pointers for

*      |                      non-dir ints 0-3)

*      +--------------------- slot id used with CSS backplane

*                             (to pretend to be a CPU)

*

******************************************************************************/

#define WRCNTL0 ((unsigned *)(0x07000000))

#define WR0_FTYPE_MASK    0x0000003f

#define WR0_FADD01_MASK   0x000000c0

#define WR0_FADD01_SH     0x06

#define WR0_PRETEND_MASK  0x0000f000

#define WR0_PRETEND_SH    12

#define WR0_FRC_ACK       0x01000000

#define WR0_FRC_GRANT     0x02000000

#define WR0_FRC_DEST      0x04000000

#define WR0_FRC_FREE      0x08000000

#define WR0_FRC_INH7      0x10000000

/*******************************************************************************

*

*        Write Control Register 1 (write only)

*

*   31  30  29   28    27   26   25   24

* +---+---+----+----+-----+----+----+----+

* |ack|rdy|rrdy|fmod|fdest|diag|diag|dreq|

* +---+---+----+----+-----+----+----+----+

*   |   |   |     |    |     |    |    |

*   |   |   |     |    |     |    |    +-- cpu.disp.req*

*   |   |   |     |    |     |    |        0 - indicates to int dispatcher that

*   |   |   |     |    |     |    |        68020 wants to access its rams.

*   |   |   |     |    |     |    |        1 - int dispatcher resumes normally.

*   |   |   |     |    |     |    +------- diag.frc.int.rec*

*   |   |   |     |    |     |             0 - decode a CSS cpu int request

*   |   |   |     |    |     |                 level as a CSS command.

*   |   |   |     |    |     |             1 - normal mode.

*   |   |   |     |    |     +------------ diag.any.type*

*   |   |   |     |    |                   0 - accept any CSS bus action as a

*   |   |   |     |    |                       valid response to a SPM

*   |   |   |     |    |                       initiated read request.

*   |   |   |     |    |                   1 - normal mode.

*   |   |   |     |    +------------------ use.fake.dest*

*   |   |   |     |                        0 - destination at arbiter is forced

*   |   |   |     |                            to be bits 8 to 11 of cntl reg 1

*   |   |   |     |                        1 - normal - dest at arbiter is the

*   |   |   |     |                            same as the dest which will be

*   |   |   |     |                            used on the CSS bus with the

*   |   |   |     |                            requested CSS bus transaction.

*   |   |   |     +----------------------- force.modify*

*   |   |   |                              0 - forces a modify cycle on the CSS

*   |   |   |                                  bus as long as the bit is 0

*   |   |   |                              1 - normal mode.

*   |   |   +----------------------------- cpu.res.ready*

*   |   |                                  1 to 0 resets the ready count in the

*   |   |                                    arbiter for the SPM.

*   |   |                                    Normally a 1.

*   |   +--------------------------------- cpu.inc.ready*

*   |                                      1 to 0 increments the ready count

*   |                                        in the arbiter for the SPM.

*   |                                        Normally a 1.

*   +------------------------------------- frc.reset.ack*

*                                          use to reset ACK request during ACK

*                                          error interrupt service.

*                                          Normally a 1.

*

*   23  22  21  20  19   18   17  16

* +---+---+---+---+----+-----+----+----+

* |rst|rst|rst|pol|type|idrst| m1 | m0 |

* +---+---+---+---+----+-----+----+----+

*   |   |   |   |    |    |    |    |

*   |   |   |   |    |    |    |    +------------ m_byte0.

*   |   |   |   |    |    |    +-------- m_byte1.

*   |   |   |   |    |    |                 0        1    8 byte read

*   |   |   |   |    |    |                 1        0   16 byte read

*   |   |   |   |    |    |                 1        1   32 byte read

*   |   |   |   |    |    |                 0        0    4 byte read (default)

*   |   |   |   |    |    +------------ int.disp.res*

*   |   |   |   |    |                  interrupt dispatcher reset(active low)

*   |   |   |   |    |                  if above 3 bits are 0 CSS bus reads are

*   |   |   |   |    |                  4 bytes. Normally a 1.

*   |   |   |   |    +----------------- Use.forced.type*

*   |   |   |   |                       0 - forced type from cntl reg 0 (0-5)

*   |   |   |   |                           is used as the bus type.

*   |   |   |   |                       1 - hardware will generate the type.

*   |   |   |   |                           Normally a 1.

*   |   |   |   +---------------------- Type.polarity

*   |   |   |                           sets the polarity of the type field

*   |   |   |                           for forcing errors on the type.

*   |   |   |                           Normally a 1.

*   |   |   +-------------------------- force.reset.recieve*

*   |   |                               resets the recieved flag to the int

*   |   |                               dispatcher (active low)

*   |   |                               Normally a 1.

*   |   +------------------------------ Idle.reset*

*   |                                   reset for the Idle FIFO (0 then 1)

*   |                                   Normally a 1.

*   +---------------------------------- CSS.reset

*                                       0 - normal.

*                                       1 - holds reset line to 0 on

*                                           CSS backplane.

*

*    15    14     13   12 11-8  7-0

* +-----+-----+-------+--+----+-----+

* |motor|fdack|frespon|tp|dest| pol |

* +-----+-----+-------+--+----+-----+

*    |     |      |     |   |    |

*    |     |      |     |   |    +---- out.data.parity 0 to 7

*    |     |      |     |   |          sets the polarity of the parity bits

*    |     |      |     |   |          for each byte of the CSS data bytes.

*    |     |      |     |   |          0-even, 1-odd

*    |     |      |     |   +--------- fake.arb.dest 0 to 3

*    |     |      |     |              these bits are presented to the arbiter

*    |     |      |     |              as the intended destination of a req'd

*    |     |      |     |              command if the fake dest bit is 0.

*    |     |      |     |              Default to our own slot id.

*    |     |      |     +------------- terminal count - used by the floppy

*    |     |      |                    controller.

*    |     |      +------------------- force.response*

*    |     |                           force the response signal at the arbiter

*    |     |                           (active low)

*    |     +-------------------------- fdack* used by floppy controller

*    +-------------------------------- floppy.motor*

*                                      motor on or off *

*

******************************************************************************/

#define WRCNTL1 ((unsigned *)(0x07000020))

#define WR1_DIAG_ANY_TYPE    0x04000000

#define WR1_FRC_RST_ACK      0x80000000

#define WR1_IRDY             0x40000000

#define WR1_RRDY             0x20000000

#define WR1_FMOD             0x10000000

#define WR1_FDEST            0x08000000

#define WR1_DATYPE           0x04000000

#define WR1_FINTREC          0x02000000

#define WR1_DREQ             0x01000000

#define WR1_CSSRST           0x00800000

#define WR1_IDLERST          0x00400000

#define WR1_FRC_RST_RCV      0x00200000

#define WR1_RSTREC           0x00200000

#define WR1_TPOL             0x00100000

#define WR1_UFTYPE           0x00080000

#define WR1_ID_RST           0x00040000

#define WR1_CSSRD_MASK       0x00030000

#define WR1_32BYTE_RD        0x00030000

#define WR1_RD32             0x00030000

#define WR1_16BYTE_RD        0x00020000

#define WR1_RD16             0x00020000

#define WR1_8BYTE_RD         0x00010000

#define WR1_RD8              0x00010000

#define WR1_4BYTE_RD         0x00000000

#define WR1_MOTOR            0x00008000

#define WR1_FDACK            0x00004000

#define WR1_FRES             0x00002000

#define WR1_TP               0x00001000

#define WR1_DEST_SH          8

#define WR1_POL              0x000000ff

/*******************************************************************************

*

*        Write Control Register 2 (write only)

*

*   31  30   29   28   27  26-25 24

* +---+----+----+----+----+----+----+

* |rst|led0|led1| grn| red|size|prom|

* +---+----+----+----+----+----+----+

*   |    |    |    |    |    |    |

*   |    |    |    |    |    |    +------ prom.at.1

*   |    |    |    |    |    |            0 - eprom at 0x00000000

*   |    |    |    |    |    |            1 - eprom at 0x01000000

*   |    |    |    |    |    +----------- promsiz.0 and promsiz.1

*   |    |    |    |    |                 prom size 00 - 27256 (32k x 8)

*   |    |    |    |    |                           01 - 27512 (64k x 8)

*   |    |    |    |    |                           10 - 27010 (128k x 8)

*   |    |    |    |    |                           11 - reserved

*   |    |    |    |    |

*   |    |    |    |    +---------------- red fault* led (top of board)

*   |    |    |    +--------------------- green led* (second from top)

*   |    |    +-------------------------- red led 1* (third from top)

*   |    +------------------------------- red led 0* (fourth from top)

*   +------------------------------------ bd.reset*

*                                         board reset when 0, cpu must set to

*                                         1 on power up

*

******************************************************************************/

#define WRCNTL2 ((unsigned char *)(0x07000040))

#define WR2_BDRST 0x80

#define WR2_LED0 0x40

#define WR2_LED1 0x20

#define WR2_GRNLED 0x10

#define WR2_REDLED 0x08

#define WR2_LED01_SH 0x05

/*******************************************************************************

*

*     Map Ram

*

*     When the SPM is performing a CSS access, add.31 = 1, the map ram is

*     read. CPU address bits 28 to 30 select one of eight locations in the

*     map ram. The top 4 bits of the map ram output give the destination

*     (i.e. the slot id of the board being accessed) and the bottom four bits

*     are mapped to address bits bus_data_44 to bus_data_47 for the CSS bus.

*     The lower 28 bits of the 68020s address are directly mapped to address

*     bits bus_data_40 to 43 and bus_data_50 to 77. Before accessing the CSS

*     bus, the 68020 must initialize the map ram.

*

*   31  30   29   28   27   26   25   24

* +---+----+----+----+----+----+----+----+

* | destination slot |  CSS add 44 - 47  |

* +---+----+----+----+----+----+----+----+

*

******************************************************************************/

#define	NUM_CSS_MAPS	8

#define MAPBASE (unsigned char *)0x08000000

#define MAP0    (unsigned char *)0x08000000

#define MAP1    (unsigned char *)0x18000000

#define MAP2    (unsigned char *)0x28000000

#define MAP3    (unsigned char *)0x38000000

#define MAP4    (unsigned char *)0x48000000

#define MAP5    (unsigned char *)0x58000000

#define MAP6    (unsigned char *)0x68000000

#define MAP7    (unsigned char *)0x78000000

/*******************************************************************************

*

*    Interrupt Idle Queue

*

*    The queue is a 16 character fifo, where the programmed character is an

*    "idle" cpu board.  This should cuase the next non-directed interrupt to

*    be passed off to this cpu, rather than the next cpu in the main queue.

*

*   31  30   29   28   27   26   25   24    23 - 0

* +---+----+----+----+----+----+----+----+-----------+

* |  Not used        |     idle cpu      |  Not used |

* +---+----+----+----+----+----+----+----+-----------+

*

******************************************************************************/

#define IDLEREG     (unsigned int *)0x0b000000

#define IDLESHFT    24

/*******************************************************************************

*

*      Interrupt Dispatcher Queue

*

*      The main queue is a 4k x 32 bit RAM which contains all the information

*      attached to an interrupt as it is defined on the CSS bus. It is

*      recommended that it is initialized with a known value for diagnostic

*      reasons. Below is a map describing where interrupt data is placed in

*      the Queue Ram.

*

*        loc                                interrupt type

*        ---                                --------------

*

*      +------------+--------------------------------------------------+ ----

*      | 0x0c000000 |                        non-directed level 0      | 128

*      | 0x0c0001fc |                                                  | ints

*      +------------+--------------------------------------------------+ ----

*      | 0x0c000200 |                        reserved                  |

*      | 0x0c0007fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c000800 |                        non-directed level 1      |

*      | 0x0c0009fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c000a00 |                        reserved                  |

*      | 0x0c000ffc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c001000 |                        non-directed level 2      |

*      | 0x0c0011fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c001200 |                        reserved                  |

*      | 0x0c0017fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c001800 |                        non-directed level 3      |

*      | 0x0c0019fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c001a00 |                        reserved                  |

*      | 0x0c001ffc |                                                  |

*      +------------+--------------------------------------------------+ ----

*      | 0x0c002000 |                        directed level 4, CPU 0   | 64

*      | 0x0c0020fc |                                                  | ints

*      +------------+--------------------------------------------------+ ----

*      | 0x0c002100 |                        directed level 4, CPU 1   |

*      | 0x0c0021fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002200 |                        directed level 4, CPU 2   |

*      | 0x0c0022fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002300 |                        directed level 4, CPU 3   |

*      | 0x0c0023fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002400 |                        directed level 4, CPU 4   |

*      | 0x0c0024fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002500 |                        directed level 4, CPU 5   |

*      | 0x0c0025fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002600 |                        directed level 4, CPU 6   |

*      | 0x0c0026fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002700 |                        directed level 4, CPU 7   |

*      | 0x0c0027fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002800 |                        directed level 4, CPU 8   |

*      | 0x0c0028fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002900 |                        directed level 4, CPU 9   |

*      | 0x0c0029fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002a00 |                        directed level 4, CPU A   |

*      | 0x0c002afc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002b00 |                        directed level 4, CPU B   |

*      | 0x0c002bfc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002c00 |                        directed level 4, CPU C   |

*      | 0x0c002cfc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002d00 |                        directed level 4, CPU D   |

*      | 0x0c002dfc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002e00 |                        directed level 4, CPU E   |

*      | 0x0c002efc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c002f00 |                        directed level 4, CPU f   |

*      | 0x0c002ffc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003000 |                        directed level 5, CPU 0   |

*      | 0x0c0030fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003100 |                        directed level 5, CPU 1   |

*      | 0x0c0031fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003200 |                        directed level 5, CPU 2   |

*      | 0x0c0032fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003300 |                        directed level 5, CPU 3   |

*      | 0x0c0033fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003400 |                        directed level 5, CPU 4   |

*      | 0x0c0034fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003500 |                        directed level 5, CPU 5   |

*      | 0x0c0035fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003600 |                        directed level 5, CPU 6   |

*      | 0x0c0036fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003700 |                        directed level 5, CPU 7   |

*      | 0x0c0037fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003800 |                        directed level 5, CPU 8   |

*      | 0x0c0038fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003900 |                        directed level 5, CPU 9   |

*      | 0x0c0039fc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003a00 |                        directed level 5, CPU A   |

*      | 0x0c003afc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003b00 |                        directed level 5, CPU B   |

*      | 0x0c003bfc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003c00 |                        directed level 5, CPU C   |

*      | 0x0c003cfc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003d00 |                        directed level 5, CPU D   |

*      | 0x0c003dfc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003e00 |                        directed level 5, CPU E   |

*      | 0x0c003efc |                                                  |

*      +------------+--------------------------------------------------+

*      | 0x0c003f00 |                        directed level 5, CPU f   |

*      | 0x0c003ffc |                                                  |

*      +------------+--------------------------------------------------+

*

*

******************************************************************************/

#define QUEUE_RAM (unsigned *)(0x0c000000)

#define QUEUE_LEN     0x4000

#define QUEUE_LWORDS  0x1000

#define VECTOR_MASK   0xff000000

#define VECTOR_SH     24

#define Q_DEST_MASK   0x0000f000

#define Q_DEST_SH     12

#define Q_SRC_MASK    0x00000f00

#define Q_SRC_SH      8

/*******************************************************************************

*

*      Interrupt Dispatcher Pointer

*

*      There are two pointer RAM's, each 256 x 7 bits, called the incoming

*      pointer RAM and the outgoing pointer RAM. These pointer RAM's allow the

*      main queue to function as multiple FIFOs. The incoming and outgoing

*      pointer associated with a particular long word in the Pointer RAM will

*      either point to a queue for directed level 4 or 5 interrupts for a CPU

*      in slot x, or if a SPM is in slot x, the pointers are used to maintain

*      interrupt info for non-directed level 0 to 3 interrupts.

*

*         31  30 29 28 27 26 25 24 23  22 21 20 19 18 17 16  15 - 0

*        +---+--------------------+---+---------------------+------+

*        | x |  incoming pointer  | x |   outgoing pointer  |   x  |

*        +---+--------------------+---+---------------------+------+

*

*

*             loc                        pointer type

*             ---                        ------------

*        +------------+--------------------------------------------------+

*        | 0x0d000000 |                directed level 4, CPU 0           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000004 |                directed level 4, CPU 1           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000008 |                directed level 4, CPU 2           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d00000c |                directed level 4, CPU 3           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000010 |                directed level 4, CPU 4           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000014 |                directed level 4, CPU 5           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000018 |                directed level 4, CPU 6           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d00001c |                directed level 4, CPU 7           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000020 |                directed level 4, CPU 8           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000024 |                directed level 4, CPU 9           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000028 |                directed level 4, CPU A           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d00002c |                directed level 4, CPU B           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000030 |                directed level 4, CPU C           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000034 |                directed level 4, CPU D           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000038 |                directed level 4, CPU E           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d00003c |                directed level 4, CPU F           |

*        |            |                or non-directed level 0           |

*        +------------+--------------------------------------------------+

*        | 0x0d000040 |                directed level 5, CPU 0           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000044 |                directed level 5, CPU 1           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000048 |                directed level 5, CPU 2           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d00004c |                directed level 5, CPU 3           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000050 |                directed level 5, CPU 4           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000054 |                directed level 5, CPU 5           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000058 |                directed level 5, CPU 6           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d00005c |                directed level 5, CPU 7           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000060 |                directed level 5, CPU 8           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000064 |                directed level 5, CPU 9           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000068 |                directed level 5, CPU A           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d00006c |                directed level 5, CPU B           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000070 |                directed level 5, CPU C           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000074 |                directed level 5, CPU D           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000078 |                directed level 5, CPU E           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d00007c |                directed level 5, CPU F           |

*        |            |                or non-directed level 1           |

*        +------------+--------------------------------------------------+

*        | 0x0d000080 |          non-directed level 2 (if SPM in slot 0) |

*        +------------+--------------------------------------------------+

*        | 0x0d000084 |          non-directed level 2 (if SPM in slot 1) |

*        +------------+--------------------------------------------------+

*        | 0x0d000088 |          non-directed level 2 (if SPM in slot 2) |

*        +------------+--------------------------------------------------+

*        | 0x0d00008c |          non-directed level 2 (if SPM in slot 3) |

*        +------------+--------------------------------------------------+

*        | 0x0d000090 |          non-directed level 2 (if SPM in slot 4) |

*        +------------+--------------------------------------------------+

*        | 0x0d000094 |          non-directed level 2 (if SPM in slot 5) |

*        +------------+--------------------------------------------------+

*        | 0x0d000098 |          non-directed level 2 (if SPM in slot 6) |

*        +------------+--------------------------------------------------+

*        | 0x0d00009c |          non-directed level 2 (if SPM in slot 7) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000a0 |          non-directed level 2 (if SPM in slot 8) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000a4 |          non-directed level 2 (if SPM in slot 9) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000a8 |          non-directed level 2 (if SPM in slot A) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000ac |          non-directed level 2 (if SPM in slot B) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000b0 |          non-directed level 2 (if SPM in slot C) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000b4 |          non-directed level 2 (if SPM in slot D) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000b8 |          non-directed level 2 (if SPM in slot E) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000bc |          non-directed level 2 (if SPM in slot F) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000c0 |          non-directed level 3 (if SPM in slot 0) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000c4 |          non-directed level 3 (if SPM in slot 1) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000c8 |          non-directed level 3 (if SPM in slot 2) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000cc |          non-directed level 3 (if SPM in slot 3) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000d0 |          non-directed level 3 (if SPM in slot 4) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000d4 |          non-directed level 3 (if SPM in slot 5) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000d8 |          non-directed level 3 (if SPM in slot 6) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000dc |          non-directed level 3 (if SPM in slot 7) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000e0 |          non-directed level 3 (if SPM in slot 8) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000e4 |          non-directed level 3 (if SPM in slot 9) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000e8 |          non-directed level 3 (if SPM in slot A) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000ec |          non-directed level 3 (if SPM in slot B) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000f0 |          non-directed level 3 (if SPM in slot C) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000f4 |          non-directed level 3 (if SPM in slot D) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000f8 |          non-directed level 3 (if SPM in slot E) |

*        +------------+--------------------------------------------------+

*        | 0x0d0000fc |          non-directed level 3 (if SPM in slot F) |

*        +------------+--------------------------------------------------+

*

*

******************************************************************************/

#define POINTER_RAM (unsigned *)(0x0d000000)

#define POINTER_LEN             0x100

#define POINTER_LWORDS          0x40

#define POINTER_MASK            0x7f7f0000

#define OUT_PNTR_SH             16

#define IN_PNTR_SH              24

/*******************************************************************************

*

*      Interrupt Dispatcher Miscellaneous RAMs

*

*      The miscellaneous RAMs consist of the counter RAM, the service pending

*      RAM, and the CPU configuration RAM. They are all simultaneously address-

*      ed as a 32 bit port, with different data bits going to the different

*      RAMs as follows:

*

*         31 - 28 27 - 24  23 - 16       15       14 - 10    9 - 0

*        +-------+-------+---------+------------+---------+---------+

*        |   x   | config|    x    | serv. pend |     x   | counter |

*        +-------+-------+---------+------------+---------+---------+

*                   16                  128                    8

*                  deep                 deep                  deep

*

*      The counter must be initialized to all 1s, the service pending must be

*      initialized to all 0s, and the configuration RAM must be initialized

*      with all slot ids of CPUs present in the CSS backplane which are avail-

*      able for recieving interrupts as follows:

*

*                address                        data

*

*                CPU slot id n                CPU slot id 1

*                CPU slot id 1                CPU slot id 2

*                      .                              .

*                      .                              .

*                      .                              .

*                CPU slot id n-1                CPU slot id n

*                    n+1                        CPU slot id 1

*                      .                              .

*                      .                              .

*                      .                              .

*                     16                        CPU slot id 1

*

*        Counter

*        -------

*        note:        negative true logic is used to store the count

*                (0x3ff corresponds to 0)

*

*            Address           Bits 9 - 0

*        +---------------+-----------------------+

*        | 0x0e000000    |     count level 0     |

*        +---------------+-----------------------+

*        | 0x0e000004    |     count level 1     |

*        +---------------+-----------------------+

*        | 0x0e000008    |     count level 2     |

*        +---------------+-----------------------+

*        | 0x0e00000c    |     count level 3     |

*        +---------------+-----------------------+

*        | 0x0e000010    |     count level 4     |

*        +---------------+-----------------------+

*        | 0x0e000014    |     count level 5     |

*        +---------------+-----------------------+

*        | 0x0e000018    |       reserved        |

*        +---------------+-----------------------+

*        | 0x0e00001c    |       reserved        |

*        +---------------+-----------------------+

*

*        Service Pending Ram

*        -------------------

*

*            Address                Bit 15

*        +---------------+-----------------------+

*        | 0x0e000000    |    CPU 0, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000004    |    CPU 1, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000008    |    CPU 2, level 0     |

*        +---------------+-----------------------+

*        | 0x0e00000c    |    CPU 3, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000010    |    CPU 4, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000014    |    CPU 5, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000018    |    CPU 6, level 0     |

*        +---------------+-----------------------+

*        | 0x0e00001c    |    CPU 7, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000020    |    CPU 8, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000024    |    CPU 9, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000028    |    CPU A, level 0     |

*        +---------------+-----------------------+

*        | 0x0e00002c    |    CPU B, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000030    |    CPU C, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000034    |    CPU D, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000038    |    CPU E, level 0     |

*        +---------------+-----------------------+

*        | 0x0e00003c    |    CPU F, level 0     |

*        +---------------+-----------------------+

*        | 0x0e000040    |    CPU 0, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000044    |    CPU 1, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000048    |    CPU 2, level 1     |

*        +---------------+-----------------------+

*        | 0x0e00004c    |    CPU 3, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000050    |    CPU 4, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000054    |    CPU 5, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000058    |    CPU 6, level 1     |

*        +---------------+-----------------------+

*        | 0x0e00005c    |    CPU 7, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000060    |    CPU 8, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000064    |    CPU 9, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000068    |    CPU A, level 1     |

*        +---------------+-----------------------+

*        | 0x0e00006c    |    CPU B, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000070    |    CPU C, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000074    |    CPU D, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000078    |    CPU E, level 1     |

*        +---------------+-----------------------+

*        | 0x0e00007c    |    CPU F, level 1     |

*        +---------------+-----------------------+

*        | 0x0e000080    |    CPU 0, level 2     |

*        +---------------+-----------------------+

*        | 0x0e000084    |    CPU 1, level 2     |

*        +---------------+-----------------------+

*        | 0x0e000088    |    CPU 2, level 2     |

*        +---------------+-----------------------+

*        | 0x0e00008c    |    CPU 3, level 2     |

*        +---------------+-----------------------+

*        | 0x0e000090    |    CPU 4, level 2     |

*        +---------------+-----------------------+

*        | 0x0e000094    |    CPU 5, level 2     |

*        +---------------+-----------------------+

*        | 0x0e000098    |    CPU 6, level 2     |

*        +---------------+-----------------------+

*        | 0x0e00009c    |    CPU 7, level 2     |

*        +---------------+-----------------------+

*        | 0x0e0000a0    |    CPU 8, level 2     |

*        +---------------+-----------------------+

*        | 0x0e0000a4    |    CPU 9, level 2     |

*        +---------------+-----------------------+

*        | 0x0e0000a8    |    CPU A, level 2     |

*        +---------------+-----------------------+

*        | 0x0e0000ac    |    CPU B, level 2     |

*        +---------------+-----------------------+

*        | 0x0e0000b0    |    CPU C, level 2     |

*        +---------------+-----------------------+

*        | 0x0e0000b4    |    CPU D, level 2     |

*        +---------------+-----------------------+

*        | 0x0e0000b8    |    CPU E, level 2     |

*        +---------------+-----------------------+

*        | 0x0e0000bc    |    CPU F, level 2     |

*        +---------------+-----------------------+

*        | 0x0e0000c0    |    CPU 0, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000c4    |    CPU 1, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000c8    |    CPU 2, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000cc    |    CPU 3, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000d0    |    CPU 4, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000d4    |    CPU 5, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000d8    |    CPU 6, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000dc    |    CPU 7, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000e0    |    CPU 8, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000e4    |    CPU 9, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000e8    |    CPU A, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000ec    |    CPU B, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000f0    |    CPU C, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000f4    |    CPU D, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000f8    |    CPU E, level 3     |

*        +---------------+-----------------------+

*        | 0x0e0000fc    |    CPU F, level 3     |

*        +---------------+-----------------------+

*        | 0x0e000100    |    CPU 0, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000104    |    CPU 1, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000108    |    CPU 2, level 4     |

*        +---------------+-----------------------+

*        | 0x0e00010c    |    CPU 3, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000110    |    CPU 4, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000114    |    CPU 5, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000118    |    CPU 6, level 4     |

*        +---------------+-----------------------+

*        | 0x0e00011c    |    CPU 7, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000120    |    CPU 8, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000124    |    CPU 9, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000128    |    CPU A, level 4     |

*        +---------------+-----------------------+

*        | 0x0e00012c    |    CPU B, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000130    |    CPU C, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000134    |    CPU D, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000138    |    CPU E, level 4     |

*        +---------------+-----------------------+

*        | 0x0e00013c    |    CPU F, level 4     |

*        +---------------+-----------------------+

*        | 0x0e000140    |    CPU 0, level 5     |

*        +---------------+-----------------------+

*        | 0x0e000144    |    CPU 1, level 5     |

*        +---------------+-----------------------+

*        | 0x0e000148    |    CPU 2, level 5     |

*        +---------------+-----------------------+

*        | 0x0e00014c    |    CPU 3, level 5     |

*        +---------------+-----------------------+

*        | 0x0e000150    |    CPU 4, level 5     |

*        +---------------+-----------------------+

*        | 0x0e000154    |    CPU 5, level 5     |

*        +---------------+-----------------------+

*        | 0x0e000158    |    CPU 6, level 5     |

*        +---------------+-----------------------+

*        | 0x0e00015c    |    CPU 7, level 5     |

*        +---------------+-----------------------+

*        | 0x0e000160    |    CPU 8, level 5     |

*        +---------------+-----------------------+

*        | 0x0e000164        |        CPU 9, level 5        |

*        +---------------+-----------------------+

*        | 0x0e000168        |        CPU A, level 5        |

*        +---------------+-----------------------+

*        | 0x0e00016c        |        CPU B, level 5        |

*        +---------------+-----------------------+

*        | 0x0e000170        |        CPU C, level 5        |

*        +---------------+-----------------------+

*        | 0x0e000174        |        CPU D, level 5        |

*        +---------------+-----------------------+

*        | 0x0e000178        |        CPU E, level 5        |

*        +---------------+-----------------------+

*        | 0x0e00017c        |        CPU F, level 5        |

*        +---------------+-----------------------+

*

*

******************************************************************************/

#define MISC_RAM (unsigned *)(0x0e000000)

#define MISC_LEN            0x200

#define MISC_LWORDS         0x80

#define MISC_MASK           0x0f0083ff

#define CNTR_MASK           0x000003ff

#define CNTR_LWORDS         8

#define CNTR_ZERO_BIT       0x00000400

#define SERV_MASK           0x00008000

#define SERV_LWORDS         0x60

#define CONFIG_LWORDS       16

#define CONFIG_MASK         0x0f000000

#define CONFIG_SH           24

#define PIPEDATA (unsigned *)(0x80000018)

#define PIPEADDR (unsigned *)(0x8000001c)

#define DATARESP 4

/*******************************************************************************

*

*        Translation table for CSS data bits to local address and data bits.

*

*                              CSS BUS DATA

*        +------+------+------+------+------+------+------+------+

*        |00  07|10  17|20  27|30  37|40  47|50  57|60  67|70  77|

*        +------+------+------+------+------+------+------+------+

*        |24  31|16  23|8   15|0    7|24  31|16  23|8   15|0    7|

*        +------+------+------+------+------+------+------+------+

*        |<-------- IN DATA -------->|<-------- IN ADDR -------->|

*

*                        IN DATA

*          31 - 24   23 22-18 17 16   15 - 12    11 - 8   7     6 - 0

*        +----------+-+------+-----+----------+----------+-+-------------+

*        | Vector # |D|   x  |level|Dest. Slot| Src. Slot|I|I/O Src. Slot|

*        +----------+-+------+-----+----------+----------+-+-------------+

*

*        Vector #

*        --------

*        read by the 68020 during the interrupt acknowledge cycle.

*

*        Directed Bit

*        ------------

*        directed interrupt if a 1, the interrupt must be sent to the

*        specified destination slot. If a 0, the interrupt may be sent to

*        any computational module accepting interrupts.

*

*        Int Priority

*        ------------

*        determines the priority of the interrupt, which increases with value.

*

*        Destination Slot

*        ----------------

*        If the directed bit is on, the interrupt is sent to the slot specified

*        by this value. If a non-directed interrupt, this field is ignored.

*

*        Source Slot

*        -----------

*        The interrupt request came from the slot specified by this value.

*

*        I/O Bus Interrupt

*        -----------------

*        If a 1, the interrupt came from an i/o bus attached to the S-bus

*        module. The I/O bus slot requesting the interrupt is specified by

*        "I/O Bus Src Slot". If this bit is 0, the interrupt request came from

*        the S-bus module.

*

*        I/O Bus Source Slot

*        -------------------

*        If the I/O Bus Interrupt bit is set, the interrupt came from the I/O

*        bus slot specified by this value. If the I/O bus Interrupt bit is 0,

*        this field is ignored(set to zero).

*

*

*        D bit   Priority Level                Use

*        -----   --------------                ---

*          1      0x07 - 0x04        High priority directed interrupts;

*                                    mapped into 68020 interrupt level 6.

*          1      0x03 - 0x00        Low priority directed interrupts;

*                                    mapped into 68020 interrupt level 5.

*          0      0x0f - 0x0c        Highest priority i/o interrupts;

*                                    mapped into 68020 interrupt level 4.

*          0      0x0b - 0x08        High priority i/o interrupts;

*                                    mapped into 68020 interrupt level 3.

*          0      0x07 - 0x04        Low priority i/o interrupts;

*                                    mapped into 68020 interrupt level 2.

*          0      0x03 - 0x00        Lowest priority i/o interrupts;

*                                    mapped into 68020 interrupt level 1.

*

*

******************************************************************************/

/*

Number of slots  in a css back plane.

*/

#define MAX_CSS_SLOT 16