THE MACH CPU_SERVER:

AN IMPLEMENTATION OF PROCESSOR ALLOCATION

DRAFT of 14 August 1989

David L. Black

Carnegie Mellon University

Introduction

This  document  describes  the  cpu_server,  a  user-mode  server that performs

processor allocation for the Mach operating system, and its library interfaces.

The  server  implements  processor allocation policy; the actual mechanisms are

contained in the Mach kernel (see related document).   The  library  interfaces

hide the details of server interaction from applications with simple scheduling

requirements.  This server is an example implementation of processor allocation

policy; many other policies can be implemented using the mechanisms provided by

the kernel.

The software described herein is  experimental  and  subject  to  change.    On

systems  that  support  it,  this software (MiG interface to server and library

interfaces) is available in  the  cpu  library  (/usr/mach/lib/libcpu.a,  -lcpu

command to ld).

Concepts

The  server  performs processor allocation by assigning processors to processor

sets provided by its clients.  This allows the clients to use  and  manage  the

processors  without  giving clients complete control over them; only the server

has the port capabilities required to reassign processors.  Processor sets  are

entities  exported  by the Mach kernel; threads assigned to a processor set run

exclusively on processors assigned  to  that  set  and  vice  versa  (with  the

tm

exception of some Unix   system calls).

The  server interface is designed around a class of objects called requests.  A

request consists of the following components:

- A run duration,

- A sequence of <processor set, number of processor> pairs.

A request is satisfied by assigning each processor set its corresponding number

of  processors  for  the  run duration specified.  The server enforces internal

limits on the number of processors and the maximum run time.    Current  limits

are 15 minutes and 75% or less of the processors on the system.

Implementation

The  server satisfies requests in a greedy fashion with strict adherence to the

order in which they are received.  For example, if the server has 10 processors

and  receives  requests for 4, 7, and 2 processors, it will satisfy the request

for 4 first, and then the requests for 7 and 2 together.   This  algorithm  was

chosen for its simplicity and lack of starvation; more sophisticated algorithms

that make better use of the processors by  satisfying  requests  out  of  order

could be used.

The  server  implementation  was  based  on  the  cthreads library and the Mach

Interface Generator(MiG).  Two threads are used  internally;  one  manages  the

assignment  of  processors to requests, the other manages all interactions with

clients.    Clients  communicate  with  the  server  via  rpcs;   MiG-generated

interfaces hide the details of the message formats.  In addition the server can

optionally generate notification messages to indicate that processors have been

allocated  to  a  request  and  that  processors are about to be removed from a

request;  these  messages  can  be  used  for   internal   synchronization   of

applications that require multiple processors for proper execution.

Interface

The  server's interface to its clients uses remote procedure calls to implement

the following primitives:

- cpu_request_create(server,   total_processors,   run_time,    *delay,

*request)  --  create  a  request for total_processors processors for

run_time seconds.  The  request  object  and  a  delay  estimate  are

returned.  The initial server port (server) is obtained by looking up

the name "cpu_server" with the local name service.

- cpu_request_add(request, processor_set, processors, *processors_left)

--  Add  the  tuple  <processor_set,  processors>  to  the  specified

request.  Return the number of processor  remaining  in  the  request

(i.e. that can be used in future cpu_request_add calls).

- cpu_request_set_notify(request, notify_port) - Ask the server to send

a notification  message  to  notify_port  after  the  processors  are

allocated  and 1 second before removing the processors.  Applications

that receive these messages can use them to make sure that  execution

only  takes  place while all of the processors are allocated; the end

message can be used to initiate a  barrier  synchronization  to  stop

cleanly, and the start message can be used to exit the barrier.

- cpu_request_activate(request,   options,   total_time,   *delay)   --

Activate the request (i.e. request the server  to  satisfy  it).    A

maximum  delay  until  the  requested processors will be allocated is

returned.  No further cpu_request_add  calls  are  permitted  on  the

request.  The following options are supported:

* Destroy  -  Destroy  the  processor  sets  when  the  request is

completed.    This  is  intended  to  support  naive  users   by

preventing  a  program that overruns its request from going into

suspended animation; destroying its processor  sets  forces  the

program  back  into  the  default  processor  set  where it will

continue to run.

* Repeat - Repeat the request for total_time.  The time  for  each

instance of the request is specified by the run_time argument of

the cpu_request_create operation that created the request.

- cpu_request_activate_task(request, options, total_time, task, *delay)

-- Identical to cpu_request_activate, but informs the server that the

application using the processor sets  is  a  task;  this  allows  the

server  to  optimize  assignment  and  removal of processors by using

task_suspend and task_resume -- processor assignment is  much  faster

if the processor is idle.

- cpu_request_destroy(request) - Destroy the specified request, freeing

any processors that were allocated to it.  If the Notify  end  option

applies,  an  end  notification  will  be sent and the freeing of the

processors will be delayed by 1 second.

- cpu_request_status(request,                     *reserved_processors,

*assigned_processors,   *active,   *options,   *time)   -   Find  out

information about a  request  including  whether  it  has  processors

assigned  to  it,  and how long until the assigned processors will be

removed, or the maximum delay until processors will  be  assigned  to

it.

- cpu_server_info(server,  *max_time, *max_total_time, *max_processors,

*delay) -- Obtain information about the  server.    max_time  is  the

maximum   time   for   a   single   request   in  cpu_request_create.

max_total_time is the maximum total  time  for  a  repeating  request

created   by   the   Repeat   option   to   cpu_request_activate   or

cpu_request_activate_task.  max_processors is the maximum  number  of

processors for any request.  delay is the current maximum delay until

a new request can be satisfied.  The cpuinfo program uses  this  call

to  determine  if  the  cpu_server  is available and what its current

situation is.

Library Interfaces

The above server interface  along  with  the  kernel  interface  will  be  used

directly  by  programs  that  require  explicit  control over which threads are

executing on which processors at  which  time.    For  applications  with  less

stringent  processor  allocation  requirements, simple library interfaces which

hide all of the internal details of interaction with the kernel and server  may

be appropriate.  Four interfaces have been developed, the allocate, task, hook,

and task-hook interfaces.

The allocate interface supports a single allocation of a  pool  of  processors.

It exports the following two calls:

- allocate_processors(num_processors,  time,  interactive)  -  Allocate

num_processors processors for the specified time.   If  the  time  is

larger than the server's maximum slice time, then a repeating request

is automatically submitted.  If interactive is TRUE, errors  generate

printfs,  and the user is asked whether the server's maximum delay is

acceptable; no allocation is performed if the answer is no.

- deallocate_processors()  -   Free   the   processors   allocated   by

allocate_processors.    This  must  be called by a thread in the same

task as the thread that did the allocation.

allocate_processors does not return until  the  allocation  of  processors  has

started;  it  performs  a task_assign internally so that the initial thread and

all threads and tasks subsequently created share the allocated processors.   If

a  program  overruns  its time allocation, it will continue to run, but without

dedicated processors.

The task interface is identical to the allocate interface, but is restricted to

applications  consisting  of  a  single  task  so  that  the server can exploit

efficiencies available in  this  case  (suspending  the  task  before  removing

processors).    The  server  will  perform its own task_suspend and task_resume

calls on the task in this case.  The task interface exports the  following  two

calls:

- task_allocate_processors(num_processors,  num_seconds, interactive) -

Identical to allocate_processors, but also promises the  server  that

all or the threads using the processors will be in the current task.

- task_deallocate_processors() - Deallocate the processors allocated by

task_allocate_processors.

The hook interface supports allocation of  a  pool  of  processors,  with  user

scheduling hooks.  It exports the following two calls:

- allocate_processors_with_hooks(num_processors,           num_seconds,

start_hook, end_hook, interactive) - Allocate the specified number of

processors  for  the  specified  number of seconds.  If the number of

seconds is greater than the server's maximum slice time, a  repeating

request  is  automatically submitted.  start_hook is called each time

after the processors are allocated.  Similarly,  end_hook  is  called

approximately   1   second   before   any   processor   deallocation.

interactive functions identically to the previous interfaces.

- deallocate_processors_with_hooks() - Free the processors allocated by

allocate_processors_with_hooks.  The end_hook will not be called, but

users should be aware of the race between this  deallocate  call  and

the server ending a time slice.

A thread must be dedicated to the allocate_processors_with_hooks call; i.e. the

calling thread does not return until the  allocation  request  is  finished  or

terminated.    This  dedicated  thread  is assigned to the allocated processors

(this can be reversed by performing a thread_assign_default or thread_assign as

part  of  the  first invocation of start_hook.  All threads within its task and

subsequently  created  tasks  are  also  assigned  (c.f.  task_assign).    Both

start_hook and end_hook must return.

Finally,  there is the task-hook interface, which combines the functionality of

the hook interface with the server optimization  of  the  task  interface;  the

calls  in  this  interface  are  the  hook interface calls with task_ prefixed.

Users should note that the  interfaces  are  independent;  processors  must  be

deallocated  with  the deallocate call from the same interface that was used to

allocate them.

Extensions

The cpu_server could be extended in a number of ways:

- Time Sensitivity - Allow more of the machine to be  allocated  during

off peak periods.

- Request Size Sensitivity - Use a declining priority system instead of

absolute ordering to speed throughput.  This is a standard  technique

from   schedulers   for   physical  memory  machines;  other  similar

techniques are also  applicable  (the  current  processor  allocation

problem  for  multiprocessors  is  essentially  similar to the memory

allocation problem for schedulers for physical memory machines).

- User Sensitivity - Reserve some portion of the  machine  for  certain

users  based  on  administrative  decisions  (e.g. they paid for some

portion of the machine, so they should always be able to get at least

that portion).

- Non-Uniform  Topologies  -  For NUMA (Non Uniform Memory Access), all

topology knowledge and policy is located in the server.    Techniques

such  as  first-fit  and  best-fit may be applicable to the processor

allocation server for such a  machine  (e.g.  the  clustered  Gigamax

machine being built by Encore).

In  addition  the  server and the kernel interface (thread assignment) could be

used directly in a language runtime that is sophisticated enough to control and

allocate processors to its threads.