Release Notes                                         Page  1 of 34

CTOS Modem Service s5.3

May 1995

CONTENTS                                                     PAGE

Product Interdependencies.....................................  2

Restrictions and Known Limitations............................  7

Product Improvements..........................................  8

Migration Guidelines..........................................  13

Support.......................................................  14

Product Information Updates...................................  18

Release Notes                                         Page  2 of 34

PRODUCT INTERDEPENDENCIES

The following workstations and shared resource processors can run

Modem Service 5.3:

Workstations (186, 286, 386, 486)

XE520 shared resource processor

(CP, channels 13; TP, channels 14)

XE530 shared resource processors

(GP, channels 18)

Workstations equipped with the B25IDS or B25ID2 Intelligent

Data Communications slice modules

Workstations with B25DCX, or XC002 port expansion modules,

SG400P, SG410P, SG430P or SG450P IOP cards, or an SG400C

expansion card

SuperGen Series 1000 through 7000 workstations

Release Notes                                         Page  3 of 34

PRODUCT INTERDEPENDENCIES (continued)

One of the following operating system levels is required to run

the CTOS Modem Service 5.3:

CTOS II 3.3, or higher, for workstations

CTOS III 1.0, or higher, for workstations

XEBTOS 7.0, or higher, for XE520 shared resource processors

CTOSXE 3.0, or higher, for shared resource processors

IDMSS 5.0, or higher, for Intelligent Data Communications

modules

AndersonJacobsen AJ2412

BIZCOMP 1012 1200300 Intelligent Modem

Convergent DC 2400

Hayes Stack Smartmodem 1200

Release Notes                                         Page  4 of 34

PRODUCT INTERDEPENDENCIES (continued)

CTOS Modem Service 5.3 is qualified for use with the following

modems:

Hayes Smartmodem 2400

Hayes Optima 144

Microcom AX9624c

RacalMilgo RMD 3222

RacalVadic Auto Dial VA212

Sematrans 9696

VenTel MC120031

Release Notes                                         Page  5 of 34

PRODUCT INTERDEPENDENCIES (continued)

The following additional modems offer support for

industrystandard error correction and data compression:

Microcom AX2400c

SupraFax V.32bis

Telebit WorldBlazer

US Robotics Courier HST Dual Standard

Release Notes                                         Page  6 of 34

PRODUCT INTERDEPENDENCIES (continued)

CTOS Modem Service can coexist with the following products:

OFIS Mail 1.0, or higher

BNet II 1.0, or higher

RAF 2.0, or higher

Note: Use of CTOS Modem Service with some hardwaremodem

combinations may require later releases of the products

listed here.

Release Notes                                         Page  7 of 34

RESTRICTIONS AND KNOWN LIMITATIONS

Modem Service can control communications on supported modems

at speeds up to 9600 baud.

Because of limitations in the XC002 service, DTR dialing and

the DCI interface are not supported on the XC002.  These

limitations do not apply to the DCX module.

The time the DTR signal is lowered between making a call and

handing control to a DCI user program has been much reduced.

Subsequently declared lines no longer use the wrong data

rate if one line is declared to use Speed_Adjust.

Release Notes                                         Page  8 of 34

PRODUCT IMPROVEMENTS

Clarification is made regarding possible modification of standard

modem scripts for outgoing calls.  See the Product Information

Update provided in this Release Notes file for page 94 of the

CTOS Modem Service Operations and Programming Guide.

New modem definition samples are included in the additional.

modems file.

Maximum length of dial string has been increased to 50.

Product now installs from disk drives other than F0.

On B39, DTR dial now works correctly when DMA is enabled.

Release Notes                                         Page  9 of 34

PRODUCT IMPROVEMENTS (continued)

DTR control now works correctly for DCI clients using the

82530 chip.

Modem did not always reset after a CLEAR CALL.  Incoming calls

could cause the ClosePort procedure to fail.  The ClosePort

procedure will now automatically clear any existing call before

closing the port, however, only PORT CLOSED is reported to the

client.

Modem Service would deinstall while DCI clients were active,

causing a GP fault when those clients later attempted to access

Modem Service procedures.  The code will now check for attached

clients and, if any exist, will return a Cannot Deinstall with

Active Clients message.

Release Notes                                         Page 10 of 34

PRODUCT IMPROVEMENTS (continued)

For compatibility with multiboard systems, request SRPROUTING

has been changed to rLocal, forcing client services to reside on

same board as Modem Service.

Support is now included for the following modems: Hayes Optima

144, USR Courier HST DUAL Standard, UDS V.3227, Telebit

Worldblazer, and SupraFaxModem V.32bis.  (Refer to page 19 for

details.)

Support is now included for the synchronous and asynchronous

ports on the SuperGen series machines.

Release Notes                                         Page 11 of 34

PRODUCT IMPROVEMENTS (continued)

Internal buffer sizes have been increased to allow for larger

message sizes and higherspeed lines.  The maximum

messagesizelinespeed combination is extremely hardware and

application dependent.  However, message sizes greater than 2048

bytes should be carefully tested before deployment.  Applications

that push the combined limit will detect numerous 2346 errors,

and should be carefully examined for possible redesign.

The Wait for Signal script commands now function properly for

DCI clients (s5.3.01)

Modem Service now provides improved recovery for DCI Clients

when WAITing for incoming calls. (s5.3.01)

The Modemservice.config parameter :DCIWaitTime: is now available

on a per line basis to permit setting of the maximum time Modem

Service will allow for an INCOMING call before it resets the

modem.  This value is given in seconds, and defaults to 600,

after which Modem Service will transparently reset the modem and

return to the Wait_For_Call state.

Release Notes                                         Page 12 of 34

PRODUCT IMPROVEMENTS (continued)

When configuring :DCIWaitTime: you must balance the time it takes

to RESET the modem and enter Data the Transfer state on a normal

call with the time you find acceptable to delay recovery in the

case of a bad call.  Remember that while processing a RESET, the

line will not be available to take incoming calls, so a 30 second

RESET combined with a 30 second :DCIWaitTime: will mean you reject

over 50 of the incoming calls.

Error Code 10706 has been added to indicate an invalid entry for

this  parameter.

Random crashes have been eliminated. (s5.3.03)

AddModemLine with unknown modem name no longer causes Modem Service

to deinstall. (s5.3.03)

DCD is now detected properly after a DTRDial call. (s5.3.03)

DTR no longer remains HIGH after failed outgoing DTRDial Call. (s5.3.03)

The Modem Script commands :SetParity: and :SetDataBits: no longer cause an

invalid bswa error if placed early in the RESET script. (s5.3.04)

Modem Service now works correctly when both comm ports on SG hardware are in

use. (s5.3.06)

RS232 IOP crash on SG hardware has been eliminated. (s5.3.06)

ReleaseActivate line functionality has been corrected to eliminate deadly

embrace requiring to reinstall modem service (s5.3.06)

Release Notes                                         Page 13 of 34

MIGRATION GUIDELINES

To use CTOS Modem Service 5.3 with a configuration file

customized for the previous product (called Modem Server), specify

the name of the configuration file during installation when

prompted to do so.  Details will appear during installation.

Release Notes                                         Page 14 of 34

SUPPORT

Modem Service 5.3 is a warranted Unisys software product.

Unisys warrants that Modem Service 5.3, in its unaltered form,

will substantially conform to Unisys current published functional

specifications when used in Unisys supported configurations.  The

customer is responsible for reporting suspected deviations using

the User Communication Form (UCF) process within ninety (90) days

following delivery.  The product is neither field repairable nor

field modifiable.  Corrections to reported deviations are provided

to the user via updated code files.

This standard warranty is not a substitute for Unisys Service

Agreement offerings.

Release Notes                                        Page 15 of 34

SUPPORT (continued)

Support Category

Modem Service 5.3 is fully supported by Unisys.

Support services for Modem Service 5.3 are offered under the

SURETY (United States) or A la Carte (International) support

programs and are consistent with the support services offered for

the entire CTOS Hardware and Software family of products.

Release Notes                                        Page 16 of 34

SUPPORT (continued)

Unisys SureNet is an electronic bulletin board service, providing

customers in the United States access to technical information

seven days a week, 24 hours a day.  SureNet Technical Information

Services are available at no additional cost to customers with a

current Unisys service agreement or software license and include

access to system alerts, customer technical bulletins and online

UCF entry. Registration is required.  SureNet Support Services are

available on an annual subscription basis.  For more information,

contact Unisys Direct at 18004481424, prompt 5.  You may register

for Technical Information Services andor Support Services either

online by dialing 18008288796 (8 data bits, 1 stop bit, no

parity, asynchronous, VT100 emulation) or by calling Unisys Direct.

Release Notes                                        Page 17 of 34

SUPPORT (continued)

If you need further assistance or information regarding support

services, please contact your Unisys representative or your local

Unisys office.

Support Discontinuance

Unisys supports the current release level and one level back.  Prior

levels are not supported.

Release Notes                                        Page 18 of 34

PRODUCT INFORMATION UPDATES

CTOS Modem Service Operations and Programming Guide 3912 6107000

*** Page 21 ***

Under the last bullet item in the subsection add the following

bullet:

SuperGen Series 1000 through 7000 workstations.

Release Notes                                        Page 19 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page 23 ***

Under Supported Modems add the following:

CTOS Modem Service is qualified for use with these modems.

Anderson Jacobsen AJ2412

BIZCOMP 1012 1200300 Intelligent Modem

Convergent DC 2400

Hayes Stack Smartmodem 1200

Hayes Smartmodem 2400

Microcom AX9624c

RacalMilgo RMD 3222

(continued)

Release Notes                                        Page 20 of 34

PRODUCT INFORMATION UPDATES (continued)

RacalVadic Auto Dial VA212

Sematrans 9696

VenTel MC120031

The following additional modems offer support for industry

standard error correction and data compression schemes.

Microcom AX2400c

SupraFax V.32bis

Telebit WorldBlazer

US Robotics Courier HST Dual Standard

The scripts provided enable these features and will provide the

(continued)

Release Notes                                        Page 21 of 34

PRODUCT INFORMATION UPDATES (continued)

best possible communications to modems with similar capabilities.

However, the nature of these schemes can cause connection

difficulties with  older style modems.  Refer to the comments at

the beginning of each script for warnings and possible

workarounds when installing in a mixed modem environment.

*** Page 25 ***

Under Software replace the BTOS II or CTOSVM entries with

the following:

CTOS II                  3.3 or higher

CTOS III                 1.0 or higher

Release Notes                                        Page 22 of 34

PRODUCT INFORMATION UPDATES (continued)

Under RAM and Disk Usage replace the table with the following:

Disk Size       Disk Size

File                     (Bytes)         (Sectors)

ModemService.run            105708             207

ModemServiceStatus.form       1351               3

ModemServiceStatus.run       46384              91

DeinstallModemService.run    10476              21

Sys.Modems                   75015             147

Additional.Modems            44119              87

ModemMsg.Bin                  3440               7

ModemService.Config           2319               7

RqLablModem.lib (optional)    3584               7

Release Notes                                        Page 23 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page 53 ***

Replace the last bullet item in the subsection

Additional Modems with this paragraph:

A way to ignore DTR for 0.5 seconds if it is used for DCI

interface data transmission, unless it uses DTR dial.  In Hayes

modems, register S25 provides this function.  On some modems,

register S25 has a different resolution depending on whether

the mode is synchronous or asynchronous.

Release Notes                                        Page 24 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page 53 ***

Add the following bullet after the last bullet in the

subsection Additional Modems:

If you are using a modem with builtin data compression or error

correction, using these options may not be advisable with

some protocols.  Modems with data compression or error

correction perform some level of flow control themselves, and

sometimes direct the user program to stop sending.  This can

create problems.  The Data CompressionError Correction

parameters should first be set to FALSE for these modems when

attempting to isolate any problems that occur.

Release Notes                                        Page 25 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page 58 ***

Replace the first bullet item in ModemGen Commands with the

following sentence:

n represents a positive integer.

*** Page 514 ***

Add the following two commands and descriptions after the

WaitForCarrier description:

:WaitforConnectMSGandIgnore:xx..x

Release Notes                                        Page 26 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page 514 (continued)***

This command is used in place of WaitStringReturn or

WaitNoStringReturn for modems which do not raise the carrier

detect as soon as an incoming call is detected.  The string

specified is the string which the modem sends to indicate an

incoming call (for example, CONNECT 2400).

:WaitforConnectMSGandSave:xx..x

This command is similar to the WaitForConnectMSGandIgnore,

except that the connect message received from the modem is

saved.  This string can then be examined using a

ComparedWithSavedString command.

Release Notes                                        Page 27 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page 81  Figure 81 ***

Add the following note to page 81 just under Figure 81:

NOTE: When using a port expansion module the CHANNEL

indicated on the status screen will still list A

through D not 1a as illustrated.  The port expander

number will be shown in the TYPE field as [COMM]1 for

the first expander and [COMM]2 for the second.

Release Notes                                        Page 28 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page 94 ***

Add the following note to page 94:

General Programming Cautions

When creating or modifying client applications, keep in mind

that modems will operate at a much slower rate than the calling

system.  Be sure to include delays in your applications which

will allow the modem(s) sufficient time to respond to calls.

Alternatively, modem scripts may be modified to DELAY before

returning from Make_A_Call to allow remote end time to properly

initialize.

Release Notes                                        Page 29 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page 1016 ***

Replace the description of the bChannel parameter with the

following text:

Value returned is

Channel A for 1

Channel B for 2

Channel C for 3

Channel D for 4

(0 is not used)

Release Notes                                        Page 30 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page 1024 ***

Replace the last sentence in the description of ParamType with

the following:

There are no parameter values for Send Break.

Add the following text to the description for pbParam:

When ParamType is 3, a value of 0FFh will cause all subsequent

ReadModemBytes commands to return currently buffered data or

the first received character, rather than wait for timeout or a

full buffer.  A value of 0 will turn off this function.

Release Notes                                        Page 31 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Section 10 ***

Add the following introductory note to Section 10:

On the Series 1000 through 7000 workstations, the API described

in Section 10 is supported on the asynchronous ports.

*** Section 11 ***

Add the following introductory note to Section 11:

The Data Communications Interface (DCI) described in Section 11

is not supported on the Series 1000 through 7000 workstations

asynchronousonly ports.  However, it is supported on

synchronousasynchronous ports.

Release Notes                                        Page 32 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page 1118 ***

Add the following:

Note:  In releases prior to 5.3, ClosePort would return an erc if

a call existed.  ClosePort will now automatically clear any

existing call.  The only event reported will be Port Closed.

*** Page C1 ***

Add the following sentence to the end of the first paragraph:

A directconnect cable is used when you want to connect

two or more workstations without using a modem.

Release Notes                                        Page 33 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page C1 (continued) ***

In Figure C1, two workstations should be shown instead of a

workstation and a modem.  Crossout the label Modem and write

in the label Workstation.

Add the following sentence after Figure C1:

The physical connection on each end is a standard DB25

connector.

Release Notes                                        Page 34 of 34

PRODUCT INFORMATION UPDATES (continued)

*** Page D2 ***

Add the following notes to the note appearing on page D2:

NOTE: The pin 12 to 16 crossover may vary depending upon your

modem.  Pin 12 on the DTE side must connect to whichever

pin the modem uses to provide RS232 signal CI (V.24 112).

Often this is pin 23 not 16.

The physical connection on each end is usually a standard

DB25 connector.

When using the :WaitForSignal:1 command the cable must also

include connection between pins 21 (RI  Ring Indicator).

* Page 81  Figure 81 ***

Add the following note to page 81 just under Figure 81:

NOTE: When using a port expansion module the CHANNEL

indicated on the status screen will still list A

through D not 1a as illustrated.  The port expander

number will be shown in the TYPE field as [COMM]1 for

the first expander and [COMM]2 for the second.

Release Notes                                        Page 28 of 34

PRODUCT INFORMA