9253 Printer Control Codes

||

| Command |      Function code      |      Description           |

|         |  Decimal    Hexidecimal |                            |

|	LF	10	0A	Moves paper up one line      |

|	CR	13	0D	Returns print head           |

|	FF	12	0C	Feeds paper to the first line|

|				of next format (TOF).        |

|	VT	11	0B	Feeds paper to tab position  |

|	Channel	49 to	31 to	of same channel number as set|

|	No.	60	3C	in VFU.                      |

|	DC1	17	11	Sets the printer in select   |

|				(online) condition.         |

|	DC3	19	13	Releases the printer from    |

|				select condition. and sets it|

|				in deselect (offline) cond. |

|	DC4	20	14	Loads tab position in VFU.   |

|	RS	30	1E	Designates 10 CPI            |

|	GS	29	1D	Designates 16.5 CPI          |

|	US	31	1F	Designates 5 CPI with RS     |

|                                   Designates 8.3 CPI with GS   |

|	ESC,VT	27,11	1B,0B	Directly skips the designated|

|	00	48,48	30,30	number of lines.             |

|	to 99	to 57,57	to 39,39	                             |

|	ESC,F	27,70	1B,46	Designates the number of     |

|	00	48,48	30,30	lines for page length.       |

|	to 99	to 57,57	to 39,39	                             |

|	ESC,5	27,53	1B,35	Sets TOF to Current Line     |

|	ESC,6	27,54	1B,36	Designates 6 LPI             |

|	ESC,8	27,56	1B,38	Designates 8 LPI             |

|	ESC,A	27,65	1B,41	Designates long line         |

|	ESC,B	27,66	1B,42	Designates short line        |

|	SO	14	0E	Shifts out character set in  |

|				case of 7bit code           |

|	SI	15	0F	Shifts in character set in   |

|				case of 7bit code           |

|	CAN	24	18	Clears print buffer          |