Documentation for Utility Programs

Decmation                                              14-July-84

A number of CP/M utility programs are supplied with the

Decmation D100 processor card. These include some utilities which

operate in an identical or nearly identical manner with the

Digital Research utilities. In other cases the function of the

Digital Research Utility has been replaced by an improved

utility. Many of the utility programs supplied are public domain

utilities. When this is the case we identify the author when

known. The standard CP/M version 2.2 utilites and system are

supplied with the CP/M programming package, Decmation order

number D116.

ASM.COM

This is the public domain utility LASM.COM. It operates

essentially the same as the Digital Research program ASM.COM.

To assemble a program use the following command line:

A>ASM FILE

File should have the file type .ASM. The files FILE.PRN

and FILE.HEX will be created on the default disk.

DATE.COM

This program prints the current system date (of the DEC

system) on the terminal. May not be implemented under all DEC

operating systems.

DEL.COM

This is a public domain utility which may be used to

delete files from a CP/M disk. It will accept wild cards. If

a wild card is used, then it will query the user for each

file to see if he wishes to delete it. Examples of use:

A>DEL *.WPB             (Delete files of type .WPB)

A>DEL MYFIL.ABC         (Delete MYFIL.ABC)

See also SWEEP.COM for another way to delete files.

DIR

This is the built-in directory command. XDIR.COM is

generally more useful. Gives a list of files meeting a

certain file specification. The number following the file

name is the number of 256-byte pages occupied by the file.

A>DIR B:*.WP

1

DUMP.COM

This a a public domain utility which may be used to

dump a file to the terminal screen. The file is dumped in

both hex and ASCII formats. The printout may be aborted by a

control-C. Example of use:

A>DUMP MYFILE.COM

ERA

This is the built-in erase file command. Examples of

use:

A>ERA A:*.WPB       (Erase all on A: of type .WPB)

A>ERA B:MYFILE.TYP  (Erase the single file given.)

For your protection the command ERA A:*.* is not accepted.

Also, you must specify the disk name for the command to be

accepted. To erase all files use SWEEP.COM.

EXIT.COM

This routine causes an immediate exit back to the DEC

operating system. May not be implemented under some operating

sytems.

A>EXIT

(DEC operating sytem prompt is typed.)

INITFL.COM

This program initializes a floppy disk in standard

CP/M, 8-inch format. The command line is:

A>INITFL

The program will prompt you to specify which disk you wish to

initialize (A:, B:, C:, etc.). It will reject your choice if

that is not a floppy disk. To initialize pseudo disks use the

DEL.COM function or delete them under your DEC operating

system and create a new one using the OPEN command.

LOAD.COM

This program is supplied only if the CP/M programming

package is ordered. The exact same function may be performed

with the program PATCH.COM. Example of use:

A>LOAD MYFILE

2

MYFILE must be of type .HEX. A file of type .COM is

generated. To accomplish the same task with the PATCH program

use the following syntax:

A>DEL MYFILE.COM        (Delete existing .COM file.)

A>PATCH MYFILE=MYFILE   (Create .HEX file.)

MOVSYS.COM

This utility moves the CP/M system, which is stored on

the system tracks of a bootable CP/M disk, from one disk to

another. Example:

A>MOVSYS A B

The above command copies the system from disk A: to disk B:.

Movsys can also be used to move the CP/M system to or from a

standard location in memory:

A>MOVSYS A M  (Move from A: to memory.)

A>MOVSYS M A  (Move from memory to A:)

If MOVSYS is executed with no command line, then it operates

like the Digital Research routine SYSGEN:

A>MOVSYS

(Asks questions as to source and destination.)

PATCH.COM

This is a general purpose routine to convert files in

.HEX fromat to files in .COM format. The general command line

is as follows:

A>PATCH outfil=infil1[/sw1],infil2[/sw2], etc

The output file if it exists must be of type .COM. The file

type does not have to be explicitly entered. All the input

files must be of type .HEX and the file type does not have to

be given. An optional switch after each input file name can

specify a relocation of the code. The switch can have one of

2 forms:

/49FB     (A hex constant added to load address.)

/FA00>1F80          (Old and relocated address.)

Examples of use:

A>PATCH PROGRAM=PROGRAM

(Create PROGRAM.COM From PROGRAM.HEX)

A>PATCH CPM64=BIOS64/FA00>1F80,JUMP

3

(Modify the file CPM64.COM by loading the code

contained in BIOS64.HEX relocated such at code which would

load at FA00 will load at 1F80. Also add the code in the

module JUMP.HEX with no relocation.)

PATCH may be used to replace DDT.COM for applications

such as the following:

A>DDT MYFILE.COM

-IXYZFILE.HEX

-R2480                  -G0

A>

The sequence:

A>PATCH MYFILE=XYZFILE/2480

performs exactly the same function.

PIP.COM

This PIP works in a nearly identical manner to Digital

Research PIP.COM. To copy a file from one place to another

the syntax is:

A>PIP [output file]=[input file]

For example:

A>PIP A:=B:MYFILE.ASM

A>PIP B:JUNK.XXX=D:KKK.YYY        (Copy with new name.)

A>PIP C:=B:*.*                    (Copy all files.)

See also SWEEP.COM for another way to copy files.

REC.COM

This program is used to "receive" a file from the DEC

operating system. It converts a DEC operating system file

into a CP/M file. The command syntax is as follows:

A>REC CPMFIL.XXX

or for a binary transfer:

A>REC CPMFIL.XXX[B]

The file name typed above is the CP/M file name. The system

will query for the DEC operating system file name with a

message such as the following:

Request for input device REC: enter file: DECFIL.XXX

4

At which point, the user types the DEC file name. For most

DEC systems (not RT-11 or TSX+) DEC file name should be

followed by [B] if the transfer is to be binary.

SAVE

This is the built-in save command. This saves the

contents of memory starting at 0100H. The number of 256-byte

pages to be saved and the file name to save it in must be

specified. For example:

A>SAVE 33 B:MYFILE.COM

The page count is a decimal number.

SEN.COM

This utility is used to "SEND" a file to the DEC

operating system. The syntax is as follows:

A>SEN MYFILE.ABC

or, for a binary transfer:

A>SEN MYFILE.ABC[B]

The file name above is the CP/M file name of the file

to send. After you type the above command line the control

program will come back with a message such as the following:

Request for output device SEN: give file name: DECFIL.ABC

"DECFIL.ABC" is typed by the user and is the name the

file is to have under the DEC operating system. This file

name must meet the rules for file names under the DEC

operating sysytem being used. Once the DEC file name has been

typed the transfer will take place. Under some operating

systems the DEC filename must be followed by "[B]" if the

transfer is to be binary. Under RT-11 or TSX+ it makes no

difference, because there is no difference between ASCII and

binary files under those operating systems.

STAT.COM

This program gives information about CP/M disks and

files and allows attributes to be set. The functions are as

follows:

A>STAT        Give space left on logged disks.

A>STAT B:     Give space on particular disk.

A>STAT D:FILE Give file size and attributes.

A>STAT D:FILE U:        Give users and space.

A>STAT D:FILE $(R/O,R/W,SYS,DIR) Set attributes.

A>STAT B:X S: Give disk attributes.)

A>STAT X V:   Display help message.

5

SUBMIT.COM

This program works in the same manner as the standard

CP/M utility of the same name.

SWEEP.COM

This is a public domain program which can be used for

nearly all file maintainence. It is invoked as follows:

A>SWEEP [file spec]

Examples:

A>SWEEP

A>SWEEP B:    (Look at files on B: only.)

A>SWEEP B:*.ASM         (.ASM files on B: only)

When sweep starts it presents a menu. The user can scan

the files of the selected class and tag or untag them. Then

he can do various things such as delete all the tagged files

or copy all the tagged files to another place.

TIME.COM

This program types the current (DEC) system time. May

not be implemented under some DEC operating systems.

UNLOAD.COM

This program converts a .COM file to a .HEX file. It

can be useful for transferring files over a communication

link which cannot accept 8-bit data. The command syntax is as

follows:

A>UNLOAD FILE

(Creates FILE.HEX from FILE.COM)

XDIR.COM

This program prints a directory on the terminal. It is

sorted in alphabetical order and the space left on the disk

is shown. Examples:

A>XDIR B:     (Directory of B:)

A>XDIR *.ASM  (Directory showing all files of type

.ASM)

6

XRPMORG.COM

This program creates a new copy of the CP/M-compatible,

RP/M operating sytem and leaves the copy in memory starting

at address 0980h. The calling sequence is as follows:

A>XRPMORG @ E4          (Create system with origin

0E400h.)

The next step is always to save the copy of the

operating sysystem with the following command:

A>SAVE 34 RPM64.COM

The hex number given in the command line for XRPMORG is

the start of the CCP (RCP). For a "64K" system this is E4.

For a 63K system this is E0. For 62K, DC, and so forth. Note

that the system can be adjusted in smaller increments too.

For example E2 would give a 63-1/2K system.

The equivalent of MOVCPM * 63 is: XRPMORG @ E0

XSUB.COM

This program is supplied only with the CP/M programming

package.

7

#################