COMMENT*

IMPORTANT NOTE:  Set Z100 to TRUE for Z-100 version and FALSE

to assemble for Z-150/PC

SCRNSAVE.ASM - Version 1.4

SCRNSAVE is a simple 8088 assembly language program which will

blank the screen of an H/Z-100 or PC computer when there has been

no keyboard input for a period of time.  This emulates the

screen saver function found on many professional terminals

and is intended to help preserve the screen phosphors by

not keeping the same display on for an extended period of time.

The default value is set by changing the TIME equate prior to

assembly.  In addition, any time from 0 to 10 minutes (in whole

minutes) can be given as an argument when the program is run.

SCRNSAVE merely traps the system timer interrupts and increments

a counter.  Any keyboard input will zero the timer, causing it

to start over.	When the counter reaches the trigger value

the screen is blanked until a key is struck.  The keystroke that

reactivates the screen is 'eaten' and does not pass through to

any program that may be running at the time.

Invoke the program by entering "SCRNSAVE[/n]" from the MS-DOS prompt.

SCRNSAVE will load itself and exit via the terminate and stay resident

function after modifying the system interrupt vectors in low memory.

The keyboard and timer interrupts are trapped by SCRNSAVE and used to

check for the passing of time and the presence (or absence) of key

activity.  The optional numeric switch [n] can be any number from

0 to 10 inclusive and represents the number of minutes that must elapse

before SCRNSAVE will blank the screen.	The default value is set by

altering the TIME equate in this file.	If the SCRNSAVE routine is

already resident it will set the time-out value to the default or

the supplied argument in the already existing routine, avoiding

making itself resident twice.  At any time SCRNSAVE can be run with

an argument of "/0" which will disable the SCRNSAVE feature.  This

is necessary since there is no practical way to remove a resident

process without rebooting the system.  If the routine is deactivated

in this manner it can be turned back on by simply running it and

supplying a new argument or accepting the default.

SCRNSAVE is an example of the proper method of 'stealing' various

interrupts for your own use.  It is important that any routine that

depends on intercepting interrupts meet two essential criteria:

1.	Minimize the amount of time spent in your routine, it is

essential that any interrupt time processing be kept as short as

possible, since multiple routines may want to steal the same interrupt.

2.	Make absolutely no assumptions about where the interrupt is

going when you are done with it.  You must always first get the old

interrupt vector from low memory, save that address and then jump to

it via a far jump when your routine is finished.  Do not assume that

the BIOS is the next in line, since there may be several other routines

like this one all waiting for the same interrupt.

In all Zenith versions of the MS-DOS IO.SYS starting with BIOS release

1.10, also known as the winchester supplement, there are user interrupts

provided that emulate the basic interrupt structure.  You should always

use these interrupts rather than the actual system interrupts.	They

will save you some processing time, since several system interrupts

that have multiple origins are split up for you into individual

interrupts.  An example of this is the ZINTKD interrupt, which could

be caused by the keyboard, lightpen or CRT vsynch.  Each of these is

available as a separate user interrupt.  In addition, the BIOS

processes multiple interrupts in such a way that it can prevent you

from seeing an actual system interrupt, but it always will generate

an equivalent user interrupt for you to intercept.

The Z-100PC (aka Z-150) and the IBM PC do not provide the same convenien

'hooks' as the Z-100.  Instead of special user vectors, we must directly

trap the basic system interrupts.

AUTHOR: 	Michael J. Cogswell

MicroMagic

10413 Mt. Sunapee Road

Vienna, VA  22180

(703) 759-6176

Compuserve PPN: 70140,363

COPYRIGHT:	This program copyright (c) 1984 by Michael J. Cogswell

All rights reserved.

This program is entered into the public domain for

the benefit of all Heath/Zenith Z-100 and PC owners.

The program may be freely distributed and reproduced

as long as this copyright notice is included and the

distribution is strictly non-commercial.  Local Heath

Users' Groups may include the program in their library,

and may charge a nominal distribution fee to recover

the costs of distribution without violating the intent

of this notice.

HARDWARE:	SCRNSAVE is intended to run on the Heath/Zenith model

100 and 100PC series of MicroComputers.

SOFTWARE:	Requires MS-DOS, version 1.10 or later (the winchester

supplement) to run; MS-DOS, MASM.EXE, LINK.EXE and

EXE2BIN to assemble.

ASSEMBLY:	MASM SCRNSAVE;

LINK SCRNSAVE;

(ignore stack warning)

EXE2BIN SCRNSAVE SCRNSAVE.COM

DEL SCRNSAVE.OBJ

DEL SCRNSAVE.EXE

HISTORY:	Version 1.0 - 22 May 1984

The initial incarnation - Enjoy yourselves!

Version 1.1 - 28 May 1984

Added interrupt information to comments and

modified to run under MS-DOS versions from 1.10 to

2.13

Version 1.2 - 24 Jun 1984

Added CRT interrupt handler so that CRT output

would also zero timer or restore screen.  This is

handy for those times when no keys are struck while

watching a long listing being typed.

Version 1.3 - 15 Sept 1984

Modified to run on Z-100PC (IBM clone) machines.

Version 1.4 - 6 Jan 1985

Corrected bug in Z-100PC code.

Modified by Earl Hall - 19 Aug 1985

[72746,3244]

Added 2 equates, EATBYTE and MONVID.

EATBYTE determines whether the keyboarded character

that turns the screen on will be "eaten".

MONVID determines whether CRT output will be monitored

for zeroing the timer or restoring the screen.

END COMMENTS*

PAGE