ERRMON

By Robert J. Newton

After experiencing a variety of problems with the hard disk on

my AT, I decided that I wanted a more informative report of disk

I/O errors than that provided by DOS.  ERRMON is the result.

ERRMON is a resident program which inserts itself in the INT 13

chain.  It then sits quietly watching for any error condition

returned by the disk driver.  When an error is detected, it

springs to life and prints an error message on the screen.  This

message is in most cases far more informative than that provided

by DOS.  It then gracefully returns to the caller to let it do

what it wishes with the error.

Under the proper conditions ERRMON will let you know when DOS is

making retries.  However, ERRMON does not scroll the screen.

This means that it is possible that a second message will overlay

the first and you would not be aware of the retry unless the

calling program causes a scroll.

A word of caution.  ERRMON has been written to respond to the

error codes returned by the IBM BIOS disk drivers on the PC, XT

and AT as shown in the various BIOS listings.  Results with

other drivers are totally unknown although it might be assumed

that they map their error codes the same in order to achieve

compatibility.  There are no checks for machine or drive type.

In addition, the errors defined for the PC/XT and AT fixed disk

drivers are not the same in all cases.  ERRMON responds to the

codes for both.  It is possible, but not likely, that say the AT

could through some glitch return an error code that is not

defined on the AT, but is on the XT.  Instead of displaying an

"Undefined error" message, ERRMON would display the message

defined for the XT.  This possibility was considered so remote

that it was not trapped.

The only error that ERRMON does not respond to is applicable

only to the AT.  Its floppy disk driver reports an error when a

disk is changed.  This is not a true i/o error and is ignored by

ERRMON.

ERRMON's overhead is essentially nil, only four instructions

when there is no error.  Approximately 1K of memory will be used,

mostly for the message text.

The video attribute for the error messages has been set to 0F,

intense white.  The messages will print in the lower right

corner of the screen.  You may change both of these by using

DEBUG.  The attribute byte is at offset 02EE and the screen line

is at offset 02E2.  Note that the screen line is relative to 0

1

so that the last line is hex 18, decimal 24.  For those with

MASM, you may change the EQUATEs at the beginning.  You may also

change the message text, perhaps to shorten it so that less

memory will be needed.

Installation is simple, just type errmon.  A successful

installation will return ERRORLEVEL 0, unsuccessful will return

255.  It can be installed by an AUTOEXEC.BAT, preferably after

anything else which might place itself in the INT 13 chain.  You

can use >nul to avoid the sign on message.

Error messages

Sense failure (PC/XT only)

Status error (AT only) - The controller status register returned

an error condition, but the error register did not contain an

error code.

Write fault (AT only) - Indicates a hardware problem with the

drive.

Undefined error - An error code was returned for which BIOS has

not defined an error.

Drive not ready (AT only)

No response - What DOS calls Drive not ready.

Seek failure - An attempt to seek to the requested track was

unsuccessful.

Controller failure - Probably indicates that the controller could

not successfully complete the requested command within the

alloted time.

EEC corrected data error - An information report that the

controller's EEC algorithm successfully corrected a soft data

error.  You should copy the file and delete the original if this

happens frequently.

Bad CRC/EEC on read - The sector could not be successfully read.

Retries should be made and if successful the file should be

rewritten or copied which will usually cure the situation.

Otherwise, use RECOVER.

Bad track (PC/XT only) - An operation was attempted on a track

flagged bad by the low level formatter.  Such areas should have

been marked bad in the FAT.

2

Bad sector (AT only) - Similar to Bad track.

Data crosses DMA boundary - This indicates a software problem;

DMA cannot operate with more than 64K of data.

DMA overrun - The controller could not get DMA access; a retry

should succeed.

Drive init failure - Probably indicates an attempt to initialize

an invalid drive number.

Drive reset failure - An attempt to reset the drive system

failed.

Sector not found - Bad news.  The requested sector could not be

found.  Assuming the values were valid, this indicates a flawed

disk.  Absent a very capable disk zapper and the knowledge of its

use, the only cure is a low level format.  It is also possible

that a disk made in one drive cannot be read in another because

one of them is out of alignment.

Diskette write protected - Take the tab off.  If there is no tab

the drive's sensors are probably bad.

Address mark not found - Very bad news.  See sector not found.

Bad controller command - An invalid command was sent to the

controller.

ERRMON is (C) Copyright 1985 by Robert J. Newton, but hereby

released to the public domain for private non-commercial use.  It

may be freely copied and distributed, but no consideration may be

requested other than any customary handling fees charged by

recognized user's groups.  No warranties of any kind are provided

and by using the program the user assumes all risk.

3