DD58(1)             Xinu Programmer's Manual              DD58(1)

NAME

dd58 - copy a file to or from a TU58 connected to an asyn-

chronous VAX line

SYNOPSIS

dd58 [option[=value]] ...

DESCRIPTION

The TU58 tape drive is a block-addressable, random access

cartridge tape storage unit often used with LSI 11 microcom-

puters.  The program _#d_#d_#5_#8 provides a way to write TU58 tapes

from the VAX (e.g., to create a bootable system tape).  It

copies standard input to the TU58, or a given number of

blocks from the TU58 to its standard output.  Because the

TU58 controller is a block-oriented interface that relies on

the driver to supply data quickly, one should never use a

terminal as standard input.  Options are:

_#o_#p_#t_#i_#o_#n         _#v_#a_#l_#u_#e_#s

sam            set special addressing mode

-sam           disable special addressing mode (default)

verify         check data after reading or writing (default)

-verify        do not check data after reading or writing

maint          set maintenance mode

-maint         disable maintenance mode (default)

drive=_#n        use drive number _#n (default 0)

posit=_#n        reading or writing begins with tape block _#n

(default 0)

count=_#n        read or write _#n bytes (default for _#w_#r_#i_#t_#i_#n_#g is

to end of file; this must be specified for

reading)

_#r_#e_#a_#d           read from the TU58 and write to the standard

output

_#w_#r_#i_#t_#e          read from the standard input and write to the

TU58

_#s_#e_#e_#k           position the TU58 drive at the given block

number

For example, to put the file _#x on a tape cartridge at block

31, using special address mode, use

dd58 sam posit=31 write < x

To read its first 512 bytes into the file _#y, use

dd58 sam posit=31 count=512 read > y

DIAGNOSTICS

Error messages are printed on the terminal; they are self-

explanatory.

Version 6b               Printed 1/12/87                        1