Yorick Documentation

for functions, variables, and structures

defined in file std.i

Printed: Thu Mar  5 21:36:10 2009

Contents:

_                   close102_default    is_struct           set_blocksize

__alpha             collect             is_void             set_filesize

__cray              conj                jc                  set_idler

__dec               copyright           jr                  set_path

__i86               cos                 jt                  set_primitives

__ibmpc             cosh                legal               set_vars

__mac               cray_primitives     library             sgi64_primitives

__macl              create              log                 show

__sgi64             createb             log10               sign

__sun               data_align          log1p               sin

__sun3              dbauto              lsdir               sinh

__vax               dbcont              mac_primitives      sizeof

__vaxg              dbdis               macl_primitives     sort

__xdr               dbexit              max                 span

_car                dbinfo              maybe_prompt        spanl

_cat                dbret               median              spawn

_cdr                dbskip              merge               spawn_callback

_cpy                dbup                merge2              sqrt

_init_clog          dec_primitives      mergef              sread

_init_pdb           digitize            mergeg              strcase

_jc                 dimsof              min                 strchar

_jr                 disassemble         mkdir               streplace

_jt                 dump_clog           mkdirp              strfind

_len                edit_times          nallof              strglob

_lst                eq_nocopy           nameof              strgrep

_map                error               noneof              string

_not_cdf            exit                numberof            strlen

_not_pdb            exp                 open                strmatch

_nxt                expm1               open102             strpart

_prt                fflush              openb               strtok

_read               floor               openb_hooks         strtrim

_rev                funcdef             orgsof              struct_align

_set_pdb            funcset             pc_primitives       structof

_write              get_addrs           pi                  strword

abs                 get_argv            plug_dir            sum

acos                get_cwd             plug_in             sun3_primitives

add_member          get_env             poly                sun_primitives

add_next_file       get_home            popen               suspend

add_record          get_member          pr1                 swrite

add_variable        get_ncycs           print               symbol_def

after               get_path            print_format        symbol_set

after_error         get_pkgnames        process_argv        system

allof               get_primitives      ptcen               tan

alpha_primitives    get_times           quit                tanh

am_subroutine       get_vars            random              timer

anyof               grow                random_seed         timer_print

area                has_records         randomize           timestamp

array               help                rdfile              transpose

asin                histinv             rdline              typeof

asinh               histogram           re_part             uncen

at_pdb_close        i86_primitives      read                updateb

at_pdb_open         im_part             read_clog           use_origins

atan                include             read_n              vax_primitives

autoload            include_all         recover_file        vaxg_primitives

avg                 indgen              reform              volume

backup              info                remove              warranty

batch               install_struct      rename              where

bookmark            integ               require             where2

call                interp              reshape             write

catch               is_array            restore             xdr_primitives

cd                  is_func             resume              yorick_stats

ceil                is_list             rmdir               zncen

close               is_range            save

close102            is_stream           sech

FROM _ TO __xdr

_

/* SEE grow     */

__alpha

/* SEE __xdr     */

__cray

/* SEE __xdr     */

__dec

/* SEE __xdr     */

__i86

/* SEE __xdr     */

__ibmpc

/* SEE __xdr     */

__mac

/* SEE __xdr     */

__macl

/* SEE __xdr     */

__sgi64

/* SEE __xdr     */

__sun

/* SEE __xdr     */

__sun3

/* SEE __xdr     */

__vax

/* SEE __xdr     */

__vaxg

/* SEE __xdr     */

__xdr

/* DOCUMENT primitive data types for various machines:

little-endians

__i86      Intel x86 Linux

__ibmpc    IBM PC (2 byte int)

__alpha    Compaq alpha

__dec      DEC workstation (MIPS), Intel x86 Windows

__vax      DEC VAX (H-double)

__vaxg     DEC VAX (G-double)

big-endians

__xdr      External Data Representation

__sun      Sun, HP, SGI, IBM-RS6000, MIPS 32 bit

__sun3     Sun-2 or Sun-3 (old)

__sgi64    SGI, Sun, HP, IBM-RS6000 64 bit

FROM __xdr TO _len

__mac      MacIntosh 68000 (power Mac, Gx are __sun)

__macl     MacIntosh 68000 (12 byte double)

__cray     Cray XMP, YMP

SEE ALSO: set_primitives

*/

_car

/* SEE _lst     */

_cat

/* SEE _lst     */

_cdr

/* SEE _lst     */

_cpy

/* SEE _lst     */

_init_clog

/* DOCUMENT _init_clog, file

initializes a Clog binary file.  Used after creating a new file --

must be called AFTER the primitive data formats have been set.

*/

_init_pdb

/* DOCUMENT _init_pdb, file, at_pdb_close

_set_pdb, file, at_pdb_close

initializes a PDB binary file.  Used after creating a new file --

must be called AFTER the primitive data formats have been set.

The _set_pdb call only sets the CloseHook, on the assumption that

the file header has already been written (as in recover_file).

SEE ALSO: createb, recover_file, at_pdb_close

*/

_jc

/* SEE _jr     */

_jr

/* DOCUMENT _jt, file, time

_jc, file, ncyc

_jr, file

are raw versions of jt and jc provided to simplify redefining

the default jt and jc functions to add additional features.

For example, you could redefine jt to jump to a time, then

plot something.  The new jt can pass its arguments along to

_jt, then call the appropriate plotting functions.

There is a raw version of jr as well.

*/

_jt

/* SEE _jr     */

_len

/* SEE _lst     */

FROM _lst TO _lst

_lst

/* DOCUMENT list= _lst(item1, item2, item3, ...)

list= _cat(item_or_list1, item_or_list2, item_or_list3, ...)

list= _cpy(list)

list= _cpy(list, i)

length= _len(list)

item= _car(list)

item_i= _car(list, i)

_car, list, i, new_item_i

list= _cdr(list)

list= _cdr(list, i)

_cdr, list, i, new_list_i

implement rudimentary Lisp-like list handling in Yorick.

However, in Yorick, a list must have a simple tree structure

- no loops or rings are allowed (loops break Yorick's memory

manager - beware).  You need to be careful not to do this as

the error will not be detected.

Lists are required in Yorick whenever you need to hold an

indeterminate amount of non-array data, such as file handles,

bookmarks, functions, index ranges, etc.  Note that Yorick

pointers cannot point to these objects.  For array data, you have

a choice between a list and a struct or an array of pointers.

Note that a list cannot be written into a file with the save

function, since it may contain unsaveable items.

The _lst (list), _cat (catenate), and _cpy (copy) functions

are the principal means for creating and maintaining lists.

_lst makes a list out of its arguments, so that each argument

becomes one item of the new list.  Unlike Yorick array data

types, a statement like x=list does not make a copy of the

list, it merely makes an additional reference to the list.

You must explicitly use the _cpy function to copy a list.  Note

that _cpy only copies the outermost list itself, not the items

in the list (even if those items are lists).  With the second

argument i, _cpy copies only the first i items in the list.

The _cat function concatentates several lists together,

"promoting" any arguments which are not lists.  This operation

changes the values of list arguments to _cat, except for the

final argument, since after _cat(list, item), the variable list

will point to the new longer list returned by _cat.

Nil, or [], functions as an empty list.  This leads to ambiguity

in the argument list for _cat, since _cat "promotes" non-list

arguments to lists; _cat treats [] as an empty list, not as a

non-list item.  Also, _lst() or _lst([]) returns a single item list,

not [] itself.

The _len function returns the number of items in a list, or 0

for [].

The _car and _cdr functions (the names are taken from Lisp,

where they originally stood for something like "address register"

and "data register" of some long forgotten machine) provide

FROM _lst TO _nxt

access to the items stored in a list.  _car(list,i) returns the

i-th item of the list, and i defaults to 1, so _car(list) is the

first item.  Also, _car,list,i,new_item_i sets the i-th item

of the list.  Finally, _cdr(list,i) returns a list of all the

items beyond the i-th, where i again defaults to 1.  The form

_cdr,list,i,new_list_i can be used to reset all list items

beyond the i-th to new values.  In the _cdr function, i=0 is

allowed.  When used to set values, both _car and _cdr can also

be called as functions, in which case they return the item or

list which has been replaced.  The _cdr(list) function returns

nil if and only if LIST contains only a single item; this is

the usual means of halting a loop over items in a list.

SEE ALSO: array, grow, _prt, _map, _rev, _nxt

*/

_map

/* DOCUMENT _map(f, list)

return a list of the results of applying function F to each

element of the input LIST in turn, as if by

_lst(f(_car(list,1)),f(_car(list,2)),...)

SEE ALSO: _lst

*/

_not_cdf

/* DOCUMENT _not_cdf(file)

is like _not_pdb, but for netCDF files.

*/

_not_pdb

/* DOCUMENT _not_pdb(file, familyOK)

returns 1 if FILE is not a PDB file, otherwise returns 0 after

setting the structure and data tables, and cataloguing any

history records.  Used to open an existing file.  Also detects

a file with an appended Clog description.

Before calling _not_pdb, set the variable yPDBopen to the value

of at_pdb_open you want to be in force.  (For historical reasons

-- in order to allow for the open102 keyword to openb -- _not_pdb

looks at the value of the variable yPDBopen, rather than at_pdb_open

directly.)

*/

_nxt

/* DOCUMENT item= _nxt(list)

return first item in LIST, and set LIST to list of remaining

items.  If you are iterating through a list, this is the way

to do it, since a loop on _car(list,i) with i varying from 1

to _len(list) scales quadratically with the length of the list,

while a loop on _nxt(list) scales linearly.

SEE ALSO: _car, _lst

*/

FROM _prt TO acos

_prt

/* DOCUMENT _prt, list

print every item in a list, recursing if some item is itself a list.

SEE ALSO: _lst

*/

_read

/* DOCUMENT _write, file, address, expression

_read, file, address, variable

or nbytes= _read(file, address, variable);

are low level read and write functions which do not "see" the

symbol table for the binary FILE.  The ADDRESS is the byte address

at which to begin the write or read operation.  The type and number

of objects of the EXPRESSION or VARIABLE determines how much data

to read, and what format conversion operations to apply.  In the

case of type char, no conversion operations are ever applied, and

_read will return the actual number of bytes read, which may be

fewer than the number implied by VARIABLE in this one case.

(In all other cases, _read returns numberof(VARIABLE).)

If the FILE has records, the ADDRESS is understood to be in the

file family member in which the current record resides.

SEE ALSO: openb, createb, updateb, save, restore, sizeof

*/

_rev

/* DOCUMENT _rev(list)

returns the input list in reverse order

SEE ALSO: _lst

*/

_set_pdb

/* SEE _init_pdb     */

_write

/* SEE _read     */

abs

/* DOCUMENT abs(x)

or abs(x, y, z, ...)

returns the absolute value of its argument.

In the multi-argument form, returns sqrt(x^2+y^2+z^2+...).

SEE ALSO: sign, sqrt

*/

acos

/* DOCUMENT acos(x)

returns the inverse cosine of its argument, range [0, pi].

SEE ALSO: sin, cos, tan, asin, acos, atan

*/

FROM add_member TO add_record

add_member

/* DOCUMENT add_member, file, struct_name, offset, name, type, dimlist

adds a member to a data type in the file FILE.  The data type name

(struct name) is STRUCT_NAME, which will be created if it does

not already exist.  The new member will be at OFFSET (in bytes)

from the beginning of an instance of this structure, and will

have the specified NAME, TYPE, and DIMLIST.  Use OFFSET -1 to

have add_member compute the next available offset in the structure.

The TYPE can be either a structure definition, or a string naming

a previously defined data type in FILE.  The optional DIMLIST is

as for the "array" function.

The STRUCT_NAME built from a series of add_member calls cannot be

used until it is installed with install_struct.

This function should be used very sparingly, mostly in code which

is building the structure of a foreign-format binary file.

SEE ALSO: add_variable, install_struct, struct_align

*/

add_next_file

/* DOCUMENT failure= add_next_file(file, filename, create_flag)

adds the next file to the FILE, which must contain history records.

If FILENAME is non-nil, the new file will be called that, otherwise

the next sequential filename is used.  If CREATE_FLAG is present

and non-zero, the new file will be created if it does not already

exist.  If omitted or nil, CREATE_FLAG defaults to 1 if the file has

write permission and 0 if it does not.

Returns 0 on success.

SEE ALSO: openb, updateb, createb, add_record

*/

add_record

/* DOCUMENT add_record, file, time, ncyc

or add_record, file, time, ncyc, address

or add_record, file

adds a new record to FILE corresponding to the specified TIME and

NCYC (respectively a double and a long).  Either or both TIME

and NCYC may be nil or omitted, but the existence of TIME and

NCYC must be the same for every record added to one FILE.

If present, ADDRESS specifies the disk address of the new record,

which is assumed to be in the current file.  Without ADDRESS, or

if ADDRESS<0, the next available address is used; this may create

a new file in the family (see the set_filesize function).

The add_record function leaves the new record current

for subsequent save commands to actually write the data.

The TIME, NCYC, and ADDRESS arguments may be equal length vectors

to add several records at once; in this case, the first of the

newly added records is the current one.  If all three of TIME,

NCYC, and ADDRESS are nil or omitted, no new records are added,

but the file becomes a record file if it was not already, and in

any case, no record will be the current record after such an

add_record call.

After the first add_record call (even if no records were added),

subsequent add_variable commands will create record variables.

FROM add_record TO after_error

After the first record has been added, subsequent save commands

will create any new variables as record variables.

After a second record has been added using add_record, neither

save commands nor add_variable commands may be used to introduce

any new record variables.

SEE ALSO: save, createb, updateb, openb, set_filesize, set_blocksize

add_variable

*/

add_variable

/* DOCUMENT add_variable, file, address, name, type, dimlist

adds a variable NAME to FILE at the specified ADDRESS, with the

specified TYPE and dimensions given by DIMLIST.  The DIMLIST may

be zero or more arguments, as for the "array" function.  If the

ADDRESS is <0, the next available address is used. Note that,

unlike the save command, add_variable does not actually write any

data -- it merely changes Yorick's description of the contents of

FILE.

After the first add_record call, add_variable adds a variable to

the record instead of a non-record variable.  See add_record.

SEE ALSO: save, openb, createb, updateb, add_record,

add_member, install_struct, data_align

*/

after

/* DOCUMENT after, secs, on_elapse

start function ON_ELAPSE at idle time SECS (wall) seconds from now.

SECS may be a floating point number to get fractions of a second.

Uncaught errors will cancel all afters; if you need controlled

cancellation, you must built it into ON_ELAPSE.  With SECS==0,

ON_ELAPSE becomes equivalent to set_idler, except that there is

only a single idler function, while there can be many ON_ELAPSE

functions.  ON_ELAPSE is called with no arguments; it must be an

intepreted function, and it will be invoked via the name it was

defined with (in its func statement).

SEE ALSO: spawn, set_idler, after_error

*/

after_error

/* DOCUMENT after_error = error_handler_func

If the variable AFTER_ERROR is set to an interpreted function

with no parameters, that function will be invoked after an error,

before the next prompt, instead of entering or offering to enter

debug mode.  The error message will be printed, and also will be

stored in the catch_message variable.  A fault during the execution

of the after_error function will not invoke after_error, but

otherwise after_error is persistent (unlike set_idler).  An error

resets any functions scheduled using after or set_idler, so the

after_error function must reschedule these if necessary.

The catch function is a more appropriate way to recover from

some errors.

SEE ALSO: set_idler, catch, after

*/

FROM allof TO array

allof

/* DOCUMENT allof(x)

anyof(x)

nallof(x)

noneof(x)

Respectively:

returns 1 if every element of the array x is non-zero, else 0.

returns 1 if at least one element of the array x is non-zero, else 0.

returns 1 if at least one element of the array x is zero, else 0.

returns 1 if every element of the array x is zero, else 0.

SEE ALSO: allof, anyof, noneof, nallof, where, where2

*/

alpha_primitives

/* DOCUMENT alpha_primitives, file

sets FILE primitive data types to be native to DEC alpha workstations.

*/

am_subroutine

/* DOCUMENT am_subroutine()

returns 1 if the current Yorick function was invoked as a subroutine,

else 0.  If am_subroutine() returns true, the result of the current

function will not be used, and need not be computed (the function

has been called for its side effects only).

*/

anyof

/* SEE allof     */

area

/* DOCUMENT area(y, x)

returns the zonal areas of the 2-D mesh (X, Y).  If Y and X are

imax-by-jmax, the result is (imax-1)-by-(jmax-1).  The area is

positive when, say, X increases with i and Y increases with j.

For example, area([[0,0],[1,1]],[[0,1],[0,1]]) is +1.

SEE ALSO: volume

*/

array

/* DOCUMENT array(value, dimension_list)

or array(type, dimension_list)

returns an object of the same type as VALUE, consisting of copies

of VALUE, with the given DIMENSION_LIST appended to the dimensions

of VALUE.  Hence, array(1.5, 3, 1) is the same as [[1.5, 1.5, 1.5]].

In the second form, the VALUE is taken as scalar zero of the TYPE.

Hence, array(short, 2, 3) is the same as [[0s,0s],[0s,0s],[0s,0s]].

A DIMENSION_LIST is a list of arguments, each of which may be

any of the following:

(1) A positive scalar integer expression,

(2) An index range with no step field (e.g.-  1:10), or

(3) A vector of integers [number of dims, length1, length2, ...]

(that is, the format returned by the dimsof function).

SEE ALSO: reshape, is_array, dimsof, numberof, grow, span, use_origins,

_lst

*/

FROM asin TO at_pdb_open

asin

/* DOCUMENT asin(x)

returns the inverse sine of its argument, range [-pi/2, pi/2].

SEE ALSO: sin, cos, tan, asin, acos, atan

*/

asinh

/* DOCUMENT asinh(x)

acosh(x)

atanh(x)

returns the inverse hyperbolic sine, cosine, or tangent of

its argument.  The range of real acosh is >=0.0.

SEE ALSO: sinh, cosh, tanh, sech, csch

*/

at_pdb_close

/* SEE at_pdb_open     */

at_pdb_open

/* DOCUMENT at_pdb_open

at_pdb_close

bits for optional behavior when a PDB file is opened or closed:

at_pdb_open:

000  Major-Order:  value specified in file is correct

001  Major-Order:102 always

002  Major-Order:  opposite from what file says

003  Major-Order:101 always

004  Strip Basis @... suffices from variable names (when possible)

Danger!  If you do this and open a file for update, the variable

names will be stripped when you close the file!

010  Use Basis @history convention on input

The 001 and 002 bits may be overridden by the open102 keyword.

The default value of at_pdb_open is 010.

at_pdb_close (the value at the time the file is opened or created

is remembered):

001  Write Major-Order 102 PDB file

002  Write PDB style history data

The following are no-ops unless bit 002 is set:

004  Use Basis @history convention on output

010  Do NOT pack all history record variables into

a single structure instance.

The 001 bit may be overridden by the close102 keyword or if

close102_default is non-zero.

The default value of at_pdb_close is 007.

SEE ALSO: close102_default

*/

FROM atan TO autoload

atan

/* DOCUMENT atan(x)

or atan(y, x)

returns the inverse tangent of its argument, range [-pi/2, pi/2].

In the two argument form, returns the angle from (1, 0) to (x, y),

in the range (-pi, pi], with atan(1, 0)==pi/2.  (If x>=0, this is

the same as atan(y/x).)

SEE ALSO: sin, cos, tan, asin, acos, atan

*/

autoload

/* DOCUMENT autoload, ifile, var1, var2, ...

or autoload, ifile

causes IFILE to be included when any of the variables VAR1, VAR2, ...

is referenced as a function or subroutine.  Multiple autoload

calls may refer to a single IFILE; the effect is cumulative.  Note

that any reference to a single one of the VARi causes all of them

to be replaced (when IFILE is included).

The semantics of this process are complicated, but should work

as expected in most cases: After the call to autoload, the VARi

may not be redefined (e.g.- VARi=something or func VARi) without

generating a warning message, and causing all the VARi for the

same IFILE to become undefined.  The semantic subtlety arises

from the yorick variable scoping rules; if any of the VARi has local

scope for any function in the calling chain when the inclusion of

IFILE is actually triggered, only those local values will be

replaced.  (The autoload function is no different than the require

or include functions in this regard.)

The second form, with no VARi, cancels the autoload, without giving

any warning; all the VARi become undefined.

Before IFILE is included, the VARi behave like [] (nil) variables

as far as their response to the is_void function, and the ! and ?

operators.  (You can use is_func to discover whether a variable is

an autoload.)  Only their actual use in a function or subroutine call

will trigger the autoload.  While the IFILE may define the VARi

as any type of object, the autoload feature only works as intended

if the VARi are defined as interpreted or built-in functions.  The

only way it makes sense for a VARi to be a built-in function, is

if the IFILE executes a plug_in command to dynamically load an

associated compiled library.

If IFILE (or a file with the same name) has already been included,

autoload is a silent no-op.  This is exactly analogous to the

behavior of the require function; it does not harm to call either

require or autoload if the IFILE has already been included.  Note

that you may want to place a require at the beginning of a file

you expect to be autoloaded, in preference to providing separate

autoloads for the second file.

SEE ALSO: include, require, plug_in, is_func

*/

FROM avg TO call

avg

/* DOCUMENT avg(x)

returns the scalar average of all elements of its array argument.

SEE ALSO: sum, min, max

*/

backup

/* SEE bookmark     */

batch

/* DOCUMENT batch, 1

batch, 0

batch()

turns on, turns off, or tests for batch mode, respectively.

If yorick is started with the command line:

yorick -batch batch_include.i ...

then batch mode is turned on, the usual custom.i startup file is

skipped, and the file batch_include.i is parsed and executed.  The

-batch and batch_include.i command line arguments are removed from

the list returned by get_argv().  These must be the first two

arguments on the command line.

In batch mode, any error will terminate Yorick (as by the quit

function) rather than entering debug mode.  Also, any attempt to

read from the keyboard is an error.

SEE ALSO: process_argv, get_argv, set_idler, after_error

*/

bookmark

/* DOCUMENT backup, f

or bmark= bookmark(f)

...

backup, f, bmark

back up the text stream F, so that the next call to the read

function returns the same line as the previous call to read

(note that you can only back up one line).  If the optional

second argument BMARK is supplied, restores the state of the

file F to its state at the time the bookmark function was

called.

After a matching failure in read, use the single argument form

of backup to reread the line containing the matching failure.

SEE ALSO: read, rdline, open, close

*/

call

/* DOCUMENT call, subroutine(arg1, arg2, arg3, arg4, arg5

arg6, arg7, arg8);

allows a SUBROUTINE to be called with a very long argument list

as an alternative to:

subroutine, arg1, arg2, arg3, arg4, arg5,

arg6, arg7, arg8;

Note that the statement

FROM call TO catch

subroutine(arg1, arg2, arg3, arg4, arg5,

arg6, arg7, arg8);

will print the return value of subroutine, even if it is nil.

If invoked as a function, call simply returns its argument.

*/

catch

/* DOCUMENT catch(category)

Catch errors of the specified category.  Category may be -1 to

catch all errors, or a bitwise or of the following bits:

0x01 math errors (SIGFPE, math library)

0x02 I/O errors

0x04 keyboard interrupts (e.g.- control C interrupt)

0x08 other compiled errors (YError)

0x10 interpreted errors (error)

Use catch by placing it in a function before the section of code

in which you are trying to catch errors.  When catch is called,

it always returns 0, but it records the virtual machine program

counter where it was called, and longjumps there if an error is

detected.  The most recent matching call to catch will catch the

error.  Returning from the function in which catch was called

pops that call off the list of catches the interpreter checks.

To use catch, place the call near the top of a function:

if (catch(category)) {

...<code to execute if error is caught>...

}

...<code "protected" by the catch>...

If an error with the specified category occurs in the "protected"

code, the program jumps back to the point of the catch and acts

as if the catch function had returned 1 (remember that when catch

is actually called it always returns 0).

In order to lessen the chances of infinite loops, the catch is

popped off the active list if it is actually used, so that a

second error will *not* be caught.  Often, this is only desirable

for the error handling code itself -- if you want to re-execute

the "protected" code, do this, and take care of the possibility

of infinite loops in your interpreted code:

while (catch(category)) {

...<code to execute if error is caught>...

}

...<code "protected" by the catch>...

After an error has been caught, the associated error message

(what would have been printed had it not been caught) is left

in the variable catch_message.

***WARNING***

If the code protected by the catch contains include or require

FROM catch TO close102

calls, or function references which force autoloads, and the

fault occurs while yorick is interpreting an included file,

catch will itself fault, and the error code will not execute.

If a fault occurs after an include has pushed a file onto

the include stack for delayed parsing and you catch that fault,

the include stack will not unwind to its condition at the time

catch was called.  That is, catch is incapable of protecting

you completely during operations involving nested levels of

include files.

In some cases, after_error is a more appropriate way to recover

from errors.

SEE ALSO: error, after_error

*/

cd

/* DOCUMENT cd, directory_name

or cd(directory_name)

change current working directory to DIRECTORY_NAME, returning

the expanded path name (i.e.- with leading environment variables,

., .., or ~ replaced by the actual pathname).  If called as a

function, returns nil to indicate failure, otherwise failure

causes a Yorick error.

SEE ALSO: lsdir, mkdir, rmdir, get_cwd, get_home, get_env, get_argv

*/

ceil

/* DOCUMENT ceil(x)

returns the smallest integer not less than x (no-op on integers).

SEE ALSO: floor

*/

close

/* DOCUMENT close, f

closes the I/O stream F (returned earlier by the open function).

If F is a simple variable reference (as opposed to an expression),

the close function will set F to nil.  If F is the only reference

to the I/O stream, then "close, f" is equivalent to "f= []".

Otherwise, "close, f" will close the file (so that subsequent

I/O operations will fail) and print a warning message about the

outstanding ("stale") references.

SEE ALSO: open, read, write, rdline, bookmark, backup, save, restore,

rename, remove

*/

close102

/* DOCUMENT close102  is a keyword for createb or updateb,

open102   is a keyword for openb or updateb

close102_default   is a global variable (initially 0)

***Do not use close102_default -- use at_pdb_close

-- this is for backward compatibility only***

close102=1  means to close the PDB file "Major-Order:102"

close102=0  means close it "Major-Order:101"

FROM close102 TO collect

if not specified, uses 1 if close102_default non-zero,

otherwise the value specified in at_pdb_close

open102=1   means to ignore what the PDB file says internally,

and open it as if it were "Major-Order:102"

open102=0   (the default) means to assume the PDB file is

correctly writen

open102=2   means to assume that the file is incorrectly

written, whichever way it is marked

open102=3   means to ignore what the PDB file says internally,

and open it as if it were "Major-Order:101"

The PDB file format comes in two styles, "Major-Order:101", and

"Major-Order:102".  Yorick interprets these correctly by default,

but other codes may ignore them, or write them incorrectly.

Unlike Yorick, not all codes are able to correctly read both

styles.  If you are writing a file which needs to be read by

a "102 style" code, create it with the close102=1 keyword.

If you notice that a file you though was a history file isn't, or

that the dimensions of multi-dimensional variables are transposed

from the order you expected, the code which wrote the file probably

blew it.  Try openb("filename", open102=2).  The choices 1 and 3

are for cases in which you know the writing code was supposed to

write the file one way or the other, and you don't want to be

bothered.

The open102 and close102 keywords, if present, override the

defaults in the variables at_pdb_open and at_pdb_close.

SEE ALSO: at_pdb_open, at_pdb_close

*/

close102_default

/* SEE close102     */

collect

/* DOCUMENT result= collect(f, name_string)

scans through all records of the history file F accumulating the

variable NAME_STRING into a single array with one additional

index varying from 1 to the number of records.

NAME_STRING can be either a simple variable name, or a name

followed by up to four simple indices which are either nil, an

integer, or an index range with constant limits.  (Note that

0 or negative indices count from the end of a dimension.)

Examples:

collect(f, "xle")        -- collects the variable f.xle

collect(f, "tr(2,2:)")   -- collects f.tr(2,2:)

collect(f, "akap(2,-1:0,)") -- collects f.akap(2,-1:0,)

FROM collect TO createb

(i.e.- akap in the last two values of its

second index)

SEE ALSO: get_times

*/

conj

/* DOCUMENT conj(z)

returns the complex conjugate of its argument.

*/

copyright

/* DOCUMENT copyright, (no) warranty

Copyright (c) 2005.  The Regents of the University of California.

All rights reserved.

Yorick is provided "as is" without any warranty, either expressed or

implied.  For a complete statement, type:

legal

at the Yorick prompt.

SEE ALSO: legal

*/

cos

/* SEE sin     */

cosh

/* SEE sinh     */

cray_primitives

/* DOCUMENT cray_primitives, file

sets FILE primitive data types to be native to Cray 1, XMP, and YMP.

*/

create

/* DOCUMENT f= create(filename)

is a synonym for       f= open(filename, "w")

Creates a new text file FILENAME, destroying any existing file of

that name.  Use the write function to write into the file F.

SEE ALSO: write, close, open

*/

createb

/* DOCUMENT file= createb(filename)

or file= createb(filename, primitives)

creates FILENAME as a PDB file in "w+b" mode, destroying any

existing file by that name.  If the PRIMITIVES argument is

supplied, it must be the name of a procedure that sets the

primitive data types for the file.  The default is to create

a file with the native primitive types of the machine on which

Yorick is running.  The following PRIMITIVES functions are

FROM createb TO dbexit

predefined:

sun_primitives    -- appropriate for Sun, HP, IBM, and

most other workstations

sun3_primitives   -- appropriate for old Sun-2 or Sun-3

dec_primitives    -- appropriate for DEC (MIPS) workstations, Windows

alpha_primitives  -- appropriate for DEC alpha workstations

sgi64_primitives  -- appropriate for 64 bit SGI workstations

cray_primitives   -- appropriate for Cray 1, XMP, and YMP

mac_primitives    -- appropriate for MacIntosh

macl_primitives   -- appropriate for MacIntosh, 12-byte double

i86_primitives    -- appropriate for Linux i86 machines

pc_primitives     -- appropriate for IBM PC

vax_primitives    -- appropriate for VAXen only (H doubles)

vaxg_primitives   -- appropriate for VAXen only (G doubles)

xdr_primitives    -- appropriate for XDR files

SEE ALSO: openb, updateb, cd

save, add_record, set_filesize, set_blocksize

close102, close102_default, at_pdb_open, at_pdb_close

*/

data_align

/* DOCUMENT data_align, file, alignment

in binary file FILE, align new variables to begin at a byte address

which is a multiple of ALIGNMENT.  (This affects placement of data

declared using save and add_variable.  For add_variable, data_align

has an effect only if the address is not specified.)  If ALIGNMENT

is <=0, new variables will be aligned as they would be if they were

data structure members.  The default value is 0.

SEE ALSO: save, add_variable

*/

dbauto

/* SEE dbexit     */

dbcont

/* SEE dbexit     */

dbdis

/* SEE dbexit     */

dbexit

/* DOCUMENT Debug mode.

Yorick errors fall into two general categories: Syntax errors discovered

during parsing, and runtime errors discovered when a Yorick program is

actually running.  When a runtime error occurs, Yorick offers the

choice of entering "debug mode", which you can do by typing the <RETURN>

key immediately after the error occurs.  Typing a non-blank line exits

debug mode automatically by default.  In debug mode, the Yorick prompt

becomes "dbug>" instead of the usual ">".  When you see this prompt,

Yorick has halted "in the middle of" the function in which the error

occurred, and you can print, plot, modify, or save the local variables

in that function by means of ordinary Yorick commands.  Debug mode is

recursive; that is, you can debug an error which occurred during

debugging to any number of levels.

FROM dbexit TO dbinfo

You can exit from debug mode in several ways:

dbexit            -- exit current debug level, discarding all

active functions and their local variables

dbexit, 0         -- exit all debug levels

dbexit, n         -- exit (at most) N debug levels

dbcont            -- continue execution of the current function

Continuing is useful if you have managed to repair the

problem which caused the error.  The expression in which the

error occurred will be evaluated a second time, so beware of

side effects.

dbret, value      -- continue execution by returning VALUE (which

may be nil or omitted) to the caller of the

function in which the error occurred.

This is useful if the function in which the error occurred is

hopelessly confounded, but you know the value it should return.

Yorick does not allow "single stepping" directly, although you can

execute the statements in a function by copying them, then tell

Yorick to skip those statements you have executed "by hand".  There

are two functions for skipping execution:

dbskip            -- skip the next logical line (This will be only

a portion of a source line if several statements

are stacked on the source line.)

dbskip, n         -- skip next N (positive or negative) logical lines

dbup              -- discard the current function, so that you are

debugging its caller -- there is no way to go

back "down", so be careful

There are two functions which print information (like other print

functions, if called as functions instead of subroutines, their

result is returned as a string array with one line per string):

dbinfo            -- returns current function and source line

dbdis             -- returns disassembled virtual machine code

for the next line (use the disassemble function

to get the entire function)

This allows you to see exactly where in a line the error occurred.

Finally,

dbauto            -- toggles whether debug mode will be entered

automatically when a runtime error occurs

dbauto, 1         -- enter debug mode automatically after an error

dbauto, 0         -- type <RETURN> after error to enter debug mode

*/

dbinfo

/* SEE dbexit     */

FROM dbret TO dump_clog

dbret

/* SEE dbexit     */

dbskip

/* SEE dbexit     */

dbup

/* SEE dbexit     */

dec_primitives

/* DOCUMENT dec_primitives, file

sets FILE primitive data types to be native to DEC (MIPS) workstations.

*/

digitize

/* DOCUMENT digitize(x, bins)

returns an array of longs with dimsof(X), and values i such that

BINS(i-1) <= X < BINS(i) if BINS is monotonically increasing, or

BINS(i-1) > X >= BINS(i) if BINS is monotonically decreasing.

Beyond the bounds of BINS, returns either i=1 or i=numberof(BINS)+1

as appropriate.

SEE ALSO: histogram, interp, integ, sort, where, where2

*/

dimsof

/* DOCUMENT dimsof(object)

or dimsof(object1, object2, ...)

returns a vector of integers describing the dimensions of OBJECT.

The format of the vector is [number of dims, length1, length2, ...].

The orgsof function returns the origin of each dimension (normally 1).

If more than one argument is given, dimsof returns the dimension

list of the result of binary operations between all the objects,

or nil if the objects are not conformable.

SEE ALSO: typeof, structof, numberof, sizeof, orgsof

*/

disassemble

/* DOCUMENT disassemble(function)

or disassemble, function

Disassembles the specified function.  If called as a function, the

result is returned as a vector of strings; if called as a subroutine,

the disassembly is printed at the terminal.  If the function is nil,

the current *main* program is disassembled -- you must include the

call to disassemble in the main program, of course, NOT on its own

line as a separate main program.

*/

dump_clog

/* DOCUMENT dump_clog, file, clog_name

dumps a Contents Log of the binary file FILE into the text file

CLOG_NAME.  Any previous file named CLOG_NAME is overwritten.

SEE ALSO: openb

*/

FROM edit_times TO error

edit_times

/* DOCUMENT edit_times, file

or edit_times, file, keep_list

or edit_times, file, keep_list, new_times, new_ncycs

edits the records for FILE.  The KEEP_LIST is a 0-origin index list

of records to be kept, or nil to keep all records.  The NEW_TIMES

array is the list of new time values for the (kept) records, and

the NEW_NCYCS array is the list of new cycle number values for the

(kept) records.  Either NEW_TIMES, or NEW_NCYCS, or both, may be

nil to leave the corresponding values unchanged.  If non-nil,

NEW_TIMES and NEW_NCYCS must have the same length as KEEP_LIST,

or, if KEEP_LIST is nil, as the original number of records in

the file.  If KEEP_LIST, NEW_TIME, and NEW_NCYCS are all omitted

or nil, then edit_times removes records as necessary to ensure

that the remaining records have monotonically increasing times,

or, if no times are present, monotonically increasing ncycs.

(The latest record at any given time/ncyc is retained, and earlier

records are removed.)

In no case does edit_times change the FILE itself; only Yorick's

in-memory model of the file is altered.

SEE ALSO: get_times, get_ncycs, jt, jc

*/

eq_nocopy

/* DOCUMENT eq_nocopy, y, x

is the same as

y= x

except that if x is an array, it is not copied, even if it is

not a temporary (i.e.- an expression).  Having multiple variables

reference the same data can be confusing, which is why the default

= operation copies the array.  The most important use of eq_nocopy

involves pointers or lists:

y= *py

z= _car(list)

always causes the data pointed to by py to be copied, while

eq_nocopy, y, *py

eq_nocopy, z, _car(list)

does not copy the data - often more nearly what you wanted.

Note that scalar int, long, and double variables are always copied,

so you cannot count on eq_nocopy setting up an "equivalence"

between variables.

*/

error

/* DOCUMENT exit, msg

error, msg

Exits the current interpreted *main* program, printing the MSG.

(MSG can be omitted to print a default.)

In the case of exit, the result is equivalent to an immediate

return from every function in the current calling chain.

In the case of error, the result is the same as if an error had

occurred in a compiled routine.

SEE ALSO: print, write, batch, catch

*/

FROM exit TO funcdef

exit

/* SEE error     */

exp

/* DOCUMENT exp(x)

returns the exponential function of its argument (inverse of log).

SEE ALSO: expm1, log, log10, sinh, cosh, tanh, sech, csch

*/

expm1

/* DOCUMENT expm1(x)

or expm1(x, ex)

return exp(X)-1 accurate to machine precision (even for X<<1)

in the second form, returns exp(x) to EX

SEE ALSO: exp, log1p

*/

fflush

/* DOCUMENT fflush, file

flush the I/O buffers for the text file FILE.  (Binary files are

flushed at the proper times automatically.)  You should only need

this after a write, especially to a pipe.

SEE ALSO: write, popen

*/

floor

/* DOCUMENT floor(x)

returns the largest integer not greater than x (no-op on integers).

SEE ALSO: ceil

*/

funcdef

/* DOCUMENT function = funcdef(command_line)

creates an anonymous interpreted function from the input

COMMAND_LINE, equivalent to

func function {

command_line;

}

The COMMAND_LINE string is restricted to the following

format:

"funcname arg1 arg2 ..."

where each of the arguments is one of

(a) a symbol (that is, a yorick variable name)

(b) a decimal integer

(c) a real number

(d) a quoted string

The quoted string is enclosed in double quotes, and a

backslash can be used to escape a double quote or a

backslash (but the other backslash escape sequences are

not recognized and unnecessary - just insert the ascii code).

Note that funcdef merely creates the function; if you want

to execute it and discard it, use the following statement:

funcdef(command_line);

FROM funcdef TO get_addrs

The huge advantage of funcdef over the full yorick parser

is that it is stateless, which means you can invoke it to

generate actions for event callbacks.  The extreme simplicity

of the permitted COMMAND_LINE is not a limitation for this

application, because you are free to invoke an arbitrarily

complex "funcname", and to provide it with arbitrary inputs.

The intent with funcdef is not to permit you to create an

arbitrary toolkit of interpreted functions, but merely to

allow you to invoke such a toolkit; the toolkit itself is

supposed to be parsed by the ordinary include, require, or

autoload mechanisms.  Generally, you will have to design

an interpreted toolkit somewhat differently if it is to be

invoked by funcdef.  For example, funcdef does not allow

you to set variables as in x=value, but you can use the

funcset (or similarly designed) function to set variables

like this:

funcdef("funcset x value")

Do not attempt to use funcdef to input vast amounts of data.

As a rule of thumb, if your funcdef strings have more than a

couple of dozen tokens, you probably haven't thought hard

enough about what you are doing.

SEE ALSO: include, spawn, funcset

*/

funcset

/* DOCUMENT funcset var1 val1 var2 val2 ...

*

*   Equivalent to

*     var1=val1; var2=val2; ...

*

*   This function it is not useful for yorick programs.  It is intended

*   to be used to create functions with funcdef that set variable values.

*

*   Handles at most 8 var/val pairs.

*   As a special case, if given an odd number of arguments, funcset

*   sets the final var to [], e.g.-

*     funcset var1 12.34 var2

*   is equivalent to

*     var1=12.34; var2=[];

*

* SEE ALSO: funcdef

*/

get_addrs

/* DOCUMENT addr_lists= get_addrs(file)

returns the byte addresses of the non-record and record variables

in the binary file FILE, and lists of the record addresses, file

indices, and filenames for file families with history records.

*addr_lists(1)   absolute addresses of non-record variables

*addr_lists(2)   relative addresses of record variables

(add record address to get absolute address)

The order of these two address lists matches the

FROM get_addrs TO get_path

corresponding lists of names returned by get_vars.

*addr_lists(3)   absolute addresses of records

*addr_lists(4)   list of file indices corresponding to

addr_lists(3); indices are into addr_lists(5)

*addr_lists(5)   list of filenames in the family

SEE ALSO: openb, updateb, restore, jt, jc, has_records, get_vars

*/

get_argv

/* DOCUMENT get_argv()

returns string array containing the argv from the command line.

The -batch and batch_include.i arguments are removed (not returned).

SEE ALSO: process_argv, cd, get_cwd, get_home, get_env, batch

*/

get_cwd

/* DOCUMENT get_cwd()

or get_home()

returns the pathname of the current working directory or of your

home directory.

SEE ALSO: cd, lsdir, get_env, get_argv

*/

get_env

/* DOCUMENT get_env(environment_variable_name)

returns the environment variable (a string) associated with

ENVIRONMENT_VARIABLE_NAME (calls ANSI getenv routine).

SEE ALSO: cd, get_cwd, get_home, get_env, get_argv

*/

get_home

/* SEE get_cwd     */

get_member

/* DOCUMENT get_member(f_or_s, member_name)

returns F_OR_S member MEMBER_NAME, like F_OR_S.MEMBER_NAME syntax,

but MEMBER_NAME can be a computed string.  The F_OR_S may be a

binary file or a structure instance.

SEE ALSO: openb

*/

get_ncycs

/* SEE get_times     */

get_path

/* DOCUMENT get_path()

returns the current include file search path.

SEE ALSO: set_path, get_pkgnames

*/

FROM get_pkgnames TO grow

get_pkgnames

/* DOCUMENT get_pkgnames(all)

returns list of package names, ALL non-zero means to return both

statically and dynamically loaded packages, otherwise just the

initial statically loaded packages.

SEE ALSO: get_path

*/

get_primitives

/* DOCUMENT prims = get_primitives(file)

Return the primitive data types for FILE as an array of 32

integers.  The format is described under set_primitives.

SEE ALSO: set_primitives, __xdr, __i86

*/

get_times

/* DOCUMENT times= get_times(file)

ncycs= get_ncycs(file)

returns the list of time or ncyc values associated with the records

if FILE, or nil if there are none.  The time values are not guaranteed

to be precise (but they should be good to at least 6 digits or so);

the precise time associated with each record may be stored as a record

variable.

SEE ALSO: collect, openb, updateb, restore, jt, jc, edit_times

*/

get_vars

/* DOCUMENT name_lists= get_vars(file)

returns the lists of non-record and record variable names in the

binary FILE.  The return value is an array of two pointers to

arrays of type string; *name_lists(1) is the array of non-record

variable names (or nil if there are none), *name_lists(2) is the

array of record variable names.

The get_addrs function returns corresponding lists of disk

addresses; the get_member function can be used in conjunction

with the dimsof, structof, and typeof functions to determine

the other properties of a variable.

SEE ALSO: openb, updateb, restore, jt, jc, has_records, get_addrs,

set_vars

*/

grow

/* DOCUMENT grow, x, xnext1, xnext2, ...

or grow(x, xnext1, xnext2, ...)

or    _(x, xnext1, xnext2, ...)

lengthens the array X by appending XNEXT1, XNEXT2, etc. to its

final dimension.  If X is nil, X is first redefined to the first

non-nil XNEXT, and the remainder of the XNEXT list is processed

normally.  Each XNEXT is considered to have the same number of

dimensions as X, by appending unit-length dimensions if necessary.

All but this final dimension of each XNEXT must be right-conformable

(that is, conformable in the sense of the right hand side of an

assignment statement) with all but the final dimension of X.

The result has a final dimension which is the sum of the final

dimension of X and all the final dimensions of the XNEXT.  Nil

FROM grow TO help

XNEXT are ignored.  The value of the result is obtained by

concatenating all the XNEXT to X, after any required broadcasts.

If invoked as a function, grow returns the new value of X; in

this case, X may be an expression.  X must be a simple variable

reference for the subroutine form of grow; otherwise there is

nowhere to return the result.  The subroutine form is slightly

more efficient than the function form for the common usage:

x= grow(x, xnext1, xnext2)           is the same as

grow, x, xnext1, xnext2              the preferred form

The _ function is a synonym for grow, for people who want this

operator to look like punctuation in their source code, on analogy

with the array building operator [a, b, c, ...].

The _cat function is sometimes more appropriate than grow.

Usage note:

Never do this:

while (more_data) grow, result, datum;

The time to complete this loop scales as the SQUARE of the number

of passes!  Instead, do this:

for (i=1,result=array(things,n_init) ; more_data ; i++) {

if (i>numberof(result)) grow, result, result;

result(i) = datum;

}

result = result(1:i-1);

The time to complete this loop scales as n*log(n), because the

grow operation doubles the length of the result each time.

SEE ALSO: _cat, array

*/

has_records

/* DOCUMENT has_records(file)

returns 1 if FILE has history records, 0 if it does not.

*/

help

/* DOCUMENT help, topic

or help

Prints DOCUMENT comment from include file in which the variable

TOPIC was defined, followed by the line number and filename.

By opening the file with a text editor, you may be able to find

out more, especially if no DOCUMENT comment was found.

Examples:

help, set_path

prints the documentation for the set_path function.

help

prints the DOCUMENT comment you are reading.

This copy of Yorick was launched from the directory:

**** Y_LAUNCH (computed at runtime) ****

Yorick's "site directory" at this site is:

**** Y_SITE (computed at runtime) ****

FROM help TO histogram

You can find out a great deal more about Yorick by browsing

through these directories.  Begin with the site directory,

and pay careful attention to the subdirectories doc/ (which

contains documentation relating to Yorick), and i/ and

contrib/ (which contain many examples of Yorick programs).

Look for files called README (or something similar) in any

of these directories -- they are intended to assist browsers.

The site directory itself contains std.i and graph.i, which

are worth reading.

Type:

help, dbexit

for help on debug mode.  If your prompt is "dbug>" instead of

">", dbexit will return you to normal mode.

Type:

quit

to quit Yorick.

SEE ALSO: quit, info, print, copyright, warranty, legal

*/

histinv

/* DOCUMENT list = histinv(hist)

returns a list whose histogram is HIST, hist = histogram(list),

that is, hist(1) 1's followed by hist(2) 2's, followed by hist(3)

3's, and so on.  The total number of elements in the returned

list is sum(hist).  All values in HIST must be non-negative;

if sum(hist)==0, histinv returns [].  The input HIST array may

have any number of dimensions; the result will always be either

nil or a 1D array.

SEE ALSO: histogram

*/

histogram

/* DOCUMENT histogram(list)

or histogram(list, weight)

returns an array hist which counts the number of occurrences of each

element of the input index LIST, which must consist of positive

integers (1-origin index values into the result array):

histogram(list)(i) = number of occurrences of i in LIST

A second argument WEIGHT must have the same shape as LIST; the result

will be the sum of WEIGHT:

histogram(list)(i) = sum of all WEIGHT(j) where LIST(j)==i

The result of the single argument call will be of type long; the

result of the two argument call will be of type double (WEIGHT is

promoted to that type).  The input argument(s) may have any number

of dimensions; the result is always 1-D.

KEYWORD: top=max_list_value

By default, the length of the result is max(LIST).  You may

FROM histogram TO include

specify that the result have a larger length by means of the TOP

keyword.  (Elements beyond max(LIST) will be 0, of course.)

SEE ALSO: digitize, sort, histinv

*/

i86_primitives

/* DOCUMENT i86_primitives, file

sets FILE primitive data types to be native to Linux i86 machines.

*/

im_part

/* DOCUMENT im_part(z)

returns the imaginary part of its argument.

Unlike z.im, works if z is not complex (returns zero).

*/

include

/* DOCUMENT #include "yorick_source.i"

require, filename

include, filename

or include, filename, now

#include is a parser directive, not a Yorick statement.  Use it

to read Yorick source code which you have saved in a file; the

file yorick_source.i will be read one line at a time, exactly as

if you had typed those lines at the keyboard.  The following

directories are searched (in this order) to find yorick_source.i:

.               (current working directory)

~/yorick        (your personal directory of Yorick functions)

~/Yorick        (your personal directory of Yorick functions)

Y_SITE/i        (Yorick distribution library)

Y_SITE/contrib  (contributed source at your site)

Y_SITE/i0       (Yorick startup and package include files)

Y_HOME/lib      (Yorick architecture dependent include files)

To find out what is available in the Y_SITE/i directory,

type:

library

You can also type

Y_SITE

to find the name of the site directory at your site, go to the

include or contrib subdirectory, and browse through the *.i files.

This is a good way to learn how to write a Yorick program.  Be

alert for files like README as well.

The require function checks to see whether FILENAME has already

been included (actually whether any file with the same final

path component has been included).  If so, require is a no-op,

otherwise, the action is the same as the include function with

NOW == 1.

The include function causes Yorick to parse and execute FILENAME

immediately.  The effect is similar to the #include parser

FROM include TO info

directive, except the finding, parsing, and execution of FILENAME

occurs at runtime.  The NOW argument has the following meanings:

NOW == -1   filename pushed onto stack, popped and parsed

when all pending input is exhausted

NOW == 0    (or nil, default) parsed just before next input

line would be parsed

NOW == 1    parsed immediately, resuming current interpreted

program when finished (like require)

NOW == 2    like 0, except no error if filename does not exist

NOW == 3    like 1, except no error if filename does not exist

Unless you are writing a startup file, or have some truly bizarre

technical reason for using the include function, use #include

instead.  The functional form of include may involve recursive

parsing, which you will not be able to understand without deep

study.  Stick with #include.

SEE ALSO: set_path, Y_SITE, plug_in, autoload, include_all, funcdef

*/

include_all

/* DOCUMENT include_all, dir1, dir2, ...

include all files in directories DIR1, DIR2, ..., with names

ending in the ".i" extension.  (This is mostly for use to load

the i-start directories when yorick starts; see i0/stdx.i.)

If any of the DIRi do not exist, or are empty, they are

silently skipped.  Filenames beginning with "." are also skipped,

even if they end in ".i".  The files are included in alphabetical

order, DIR1 first, then DIR2, and so on.

SEE ALSO: include, autoload

*/

indgen

/* DOCUMENT indgen(n)

or indgen(start:stop)

or indgen(start:stop:step)

returns "index generator" list -- an array of longs running from

1 to N, inclusive.  In the second and third forms, the index

values specified by the index range are returned.

SEE ALSO: span, spanl, array

*/

info

/* DOCUMENT info, expr

prints the data type and array dimensions of EXPR.

SEE ALSO: help, print

*/

FROM install_struct TO interp

install_struct

/* DOCUMENT install_struct, file, struct_name

or install_struct, file, struct_name, size, align, order

or install_struct, file, struct_name, size, align, order, layout

installs the data type named STRUCT_NAME in the binary FILE.  In

the two argument form, STRUCT_NAME must have been built by one or

more calls to the add_member function.  In the 5 and 6 argument calls,

STRUCT_NAME is a primitive data type -- an integer type for the 5

argument call, and a floating point type for the 6 argument call.

The 5 argument form may also be used to declare opaque data types.

SIZE is the size of an instance in bytes, ALIGN is its alignment

boundary (also in bytes), and ORDER is the byte order.  ORDER is

1 for most significant byte first, -1 for least significant byte

first, and 0 for opaque (unconverted) data.  Other ORDER values

represent more complex byte permutations (2 is the byte order for

VAX floating point numbers).  If ORDER equals SIZE, then the data

type is not only opaque, but also must be read sequentially.

LAYOUT is an array of 7 long values parameterizing the floating

point format, [sign_address, exponent_address, exponent_size,

mantissa_address, mantissa_size, mantissa_normalized, exponent_bias]

(the addresses and sizes are in bits, reduced to MSB first order).

Use, e.g., nameof(float) for STRUCT_NAME to redefine the meaning

of the float data type for FILE.

SEE ALSO: add_variable, add_member

*/

integ

/* DOCUMENT integ(y, x, xp)

or integ(y, x, xp, which)

See the interp function for an explanation of the meanings of the

arguments.  The integ function returns ypi which is the integral

of the piecewise linear curve (X(i), Y(i)) (i=1, ..., numberof(X))

from X(1) to XP.  The curve (X, Y) is regarded as constant outside

the bounds of X.  Note that X must be monotonically increasing or

SEE ALSO: interp, digitize, span

*/

interp

/* DOCUMENT interp(y, x, xp)

or interp(y, x, xp, which)

returns yp such that (XP, yp) lies on the piecewise linear curve

(X(i), Y(i)) (i=1, ..., numberof(X)).  Points beyond X(1) are set

to Y(1); points beyond X(0) are set to Y(0).  The array X must be

one dimensional, have numberof(X)>=2, and be either monotonically

increasing or monotonically decreasing.  The array Y may have more

than one dimension, but dimension WHICH must be the same length as

X.  WHICH defaults to 1, the first dimension of Y.  WHICH may be

non-positive to count dimensions from the end of Y; a WHICH of 0

means the final dimension of Y.  The result yp has dimsof(XP)

in place of the WHICH dimension of Y (if XP is scalar, the WHICH

dimension is not present).  (The dimensions of the result are the

same as if an index list with dimsof(XP) were placed in slot

WHICH of Y.)

SEE ALSO: integ, digitize, span

*/

FROM is_array TO is_void

is_array

/* DOCUMENT is_array(object)

returns 1 if OBJECT is an array data type (as opposed to a function,

structure definition, index range, I/O stream, etc.), else 0.

An array OBJECT can be written to or read from a binary file;

non-array Yorick data types cannot.

SEE ALSO: is_func, is_void, is_range, is_struct, is_stream

*/

is_func

/* DOCUMENT is_func(object)

returns 1 if OBJECT is a Yorick interpreted function, 2 if OBJECT

is a built-in (that is, compiled) function, 3 if OBJECT is an

autoload (will become either 1 or 2 on reference), else 0.

SEE ALSO: is_array, is_void, is_range, is_struct, is_stream, autoload

*/

is_list

/* DOCUMENT is_list(object)

returns 1 if OBJECT is a list or nil, else 0 (see _lst).

SEE ALSO: is_array, is_func, is_void, is_range, is_struct, _lst

*/

is_range

/* DOCUMENT is_range(object)

returns 1 if OBJECT is an index range (e.g.-  3:5 or 11:31:2),

else 0.

SEE ALSO: is_array, is_func, is_void, is_struct, is_stream

*/

is_stream

/* DOCUMENT is_stream(object)

returns 1 if OBJECT is a binary I/O stream (usually a file), else 0.

The _read and _write functions work on object if and only if

is_stream returns non-zero.  Note that is_stream returns 0 for a

text stream -- you need the typeof function to test for those.

SEE ALSO: is_array, is_func, is_void, is_range, is_struct

*/

is_struct

/* DOCUMENT is_struct(object)

returns 1 if OBJECT is the definition of a Yorick struct, else 0.

Thus, is_struct(double) returns 1, but is_struct(1.0) returns 0.

SEE ALSO: is_array, is_func, is_void, is_range, is_stream

*/

is_void

/* DOCUMENT is_void(object)

returns 1 if OBJECT is nil (the one instance of the void data type),

else 0.

SEE ALSO: is_array, is_func, is_range, is_struct, is_stream

*/

FROM jc TO jt

jc

/* DOCUMENT jc, file, ncyc

jump to the record of FILE nearest the specified NCYC.

SEE ALSO: jt, _jc, edit_times, show, jr

*/

jr

/* DOCUMENT jr, file, i

or _jr(file, i)

Jump to a particular record number I (from 1 to n_records) in a

binary file FILE.  The function returns 1 if such a record exists,

0 if there is no such record.  In the latter case, no action is

taken; the program halts with an error only if jr was invoked

as a subroutine.  Record numbering wraps like array indices; use

jr, file, 0  to jump to the last record, -1 to next to last, etc.

SEE ALSO: jt, jc, edit_times, show

*/

jt

/* DOCUMENT jt, time

or jt, file, time

or jt, file

or jt, file, -

jump to the record nearest the specified TIME.  If no FILE is

specified, the current record of all open binary files containing

records is shifted.

If both FILE and TIME are specified and jt is called as a function,

it returns the actual time of the new current record.

N.B.: "jt, file" and "jt, file, -" are obsolete.  Use the jr function to

step through a file one record at a time.

If only the FILE is specified, increment the current record of that

FILE by one.  If the TIME argument is - (the pseudo-index range

function), decrement the current record of FILE by one.

If the current record is the last, "jt, file" unsets the current record

so that record variables will be inaccessible until another jt or jc.

The same thing happens with "jt, file, -" if the current record was the

first.

If only FILE is specified, jt returns 1 if there is a new current

record, 0 if the call resulted in no current record.  Thus "jt(file)"

and "jt(file,-)" may be used as the condition in a while loop to step

through every record in a file:

file= openb("example.pdb");

do {

restore, file, interesting_record_variables;

...calculations...

} while (jt(file));

SEE ALSO: jc, _jt, edit_times, show, jr

*/

FROM legal TO macl_primitives

legal

/* DOCUMENT legal

Prints the legal details of Yorick's copyright, licensing,

and lack of warranty.

SEE ALSO: copyright, warranty

*/

library

/* DOCUMENT library

print the Y_SITE/i/README file at the terminal.

*/

log

/* DOCUMENT log(x)

returns the natural logarithm of its argument (inverse of exp).

SEE ALSO: log1p, log10, exp, asinh, acosh, atanh

*/

log10

/* DOCUMENT log10(x)

returns the base 10 logarithm of its argument (inverse of 10^x).

SEE ALSO: log, exp, asinh, acosh, atanh

*/

log1p

/* DOCUMENT log1p(x)

return log(1+X) accurate to machine precision (even for X<<1)

from Goldberg, ACM Computing Surveys, Vol 23, No 1, March 1991,

apparently originally from HP-15C Advanced Functions Handbook

SEE ALSO: expm1, log1p

*/

lsdir

/* DOCUMENT files = lsdir(directory_name)

or files = lsdir(directory_name, subdirs)

List DIRECTORY_NAME.  The return value FILES is an array of

strings or nil; the order of the filenames is unspecified;

it does not contain "." or ".."; it does not contain the

names of subdirectories.  If SUBDIRS is given and is a simple

variable name, it is set to a list of subdirectory names (or

nil if there are no subdirectories).

If DIRECTORY_NAME does not exist, the return value is the

integer 0 rather than nil.

SEE ALSO: cd, mkdir, rmdir, get_cwd, get_home

*/

mac_primitives

/* DOCUMENT mac_primitives, file

sets FILE primitive data types to be native to MacIntosh, 8 byte double.

*/

macl_primitives

/* DOCUMENT macl_primitives, file

sets FILE primitive data types to be native to MacIntosh, long double.

*/

FROM max TO merge

max

/* DOCUMENT max(x)

or max(x, y, z, ...)

returns the scalar maximum value of its array argument, or, if

more than one argument is supplied, returns an array of the

maximum value for each array element among the several arguments.

In the multi-argument case, the arguments must be conformable.

SEE ALSO: min, sum, avg

*/

maybe_prompt

/* DOCUMENT maybe_prompt

Issue prompt for keyboard input if appropriate.

This command only makes sense (I think) as the final statement

of a function invoked as an idler (via set_idler), when yorick is

in a loop with an idler function that continuously re-installs

itself.  Yorick ordinarily issues a prompt only just before it

stops to wait for keyboard input, it will never prompt in this

situation, even though it would accept keyboard input if it

were typed.

SEE ALSO: set_idler

*/

median

/* DOCUMENT median(x)

or median(x, which)

returns the median of the array X.  The search for the median takes

place along the dimension of X specified by WHICH.  WHICH defaults

to 1, meaning the first index of X.  The median function returns an

array with one fewer dimension than its argument X (the WHICH

dimension of X is missing in the result), in exact analogy with

rank reducing index range functions.  If dimsof(X)(WHICH) is

odd, the result will have the same data type as X; if even, the

result will be a float or a double, since the median is defined

as the arithmetic mean between the two central values in that

case.

SEE ALSO: sort

*/

merge

/* DOCUMENT merge(true_expr, false_expr, condition)

returns the values TRUE_EXPR or FALSE_EXPR where CONDITION is

non-zero or zero, respectively.  The result has the data type of

TRUE_EXPR or FALSE_EXPR, promoted to the higher arithmetic type

if necessary.  The result has the dimensions of CONDITION.

The number of elements in TRUE_EXPR must match the number of

non-zero elements of CONDITION, and the number of elements in

FALSE_EXPR must match the number of zero elements of CONDITION.

(TRUE_EXPR or FALSE_EXPR should be nil if there are no such

elements of CONDITION.  Normally, TRUE_EXPR and FALSE_EXPR should

be 1-D arrays if they are not nil.)

This function is intended for vectorizing a function whose

domain is divided into two or more parts, as in:

func f(x) {

big= (x>=threshhold);

FROM merge TO mergeg

wb= where(big);

ws= where(!big);

if (is_array(wb)) {

xx= x(wb);

fb= <function of xx>

}

if (is_array(ws)) {

xx= x(ws);

fs= <function of xx>

}

return merge(fb, fs, big);

}

SEE ALSO: mergef, merge2, where

*/

merge2

/* DOCUMENT merge2(true_expr, false_expr, condition)

returns the values TRUE_EXPR or FALSE_EXPR where CONDITION is

non-zero or zero, respectively.  The result has the data type of

TRUE_EXPR or FALSE_EXPR, promoted to the higher arithmetic type

if necessary.  Unlike the merge function, TRUE_EXPR and FALSE_EXPR

must be conformable with each other, and with the CONDITION.

SEE ALSO: merge, where, mergef

*/

mergef

/* DOCUMENT y = mergef(x, f1, cond1, f2, cond2, ... felse)

*   Evaluate F1(X(where(COND1))), F2(X(where(COND2))),

*   and so on, until FELSE(X(where(!(COND1 | COND2 | ...))))

*   and merge all the results back into an array Y with the

*   same dimensions as X.  Each of the CONDi must have the

*   same dimensions as X, and they must be mutally exclusive.

*

*   During the evaluation of Fi, note that all of the local

*   variables of the caller of mergef are available.  The

*   Fi are invoked as Fi(X(mergel)) and  the variable mergel

*   = where(CONDi) is available to the Fi, in case they need

*   to extract any additional parameters.  If noneof(CONDi)

*   then Fi will not be called at all, otherwise, the Fi are

*   invoked in order.  The return value of Fi must have the same

*   shape as its argument (which will be a 1D array or scalar).

*

*   Use mergeg to construct secondary results the same shape

*   as X and Y.

*

* SEE ALSO: mergeg, merge

*/

mergeg

/* DOCUMENT z = mergeg(z, value)

*       or z = mergeg(z)

*   If secondary results are to be returned from a mergef, besides

*   its return value, the Fi may construct them using the second

*   form of mergef:

*     z = mergeg(z, value)

FROM mergeg TO nallof

*   where z is a variable in the original caller of mergef,

*   and value is its value where(CONDi).  Note that the variable

*   name of the first parameter must be the same as the variable

*   name it is assigned to in this construction -- that variable

*   is being used to hold the state of z as it is built.  After

*   the outer mergef returns, the caller needs to invoke

*     z = mergeg(z)

*   one final time to complete each secondary return value.

*

*   z = [];

*   y = mergef(x, f1, cond, f2);

*   z = mergeg(z);

*   ...

*   func f1(x) { <exprz(x) computes z(x), expry(x) computes y(x)>

*     z = mergeg(z, exprz(x));

*     return expry(x);

*   }

*   func f2(x) { <exprz(x) computes z(x), expry(x) computes y(x)>

*     z = mergeg(z, exprz(x));

*     return expry(x);

*   }

*

* SEE ALSO: mergef, merge

*/

min

/* DOCUMENT min(x)

or min(x, y, z, ...)

returns the scalar minimum value of its array argument, or, if

more than one argument is supplied, returns an array of the

minimum value for each array element among the several arguments.

In the multi-argument case, the arguments must be conformable.

SEE ALSO: max, sum, avg

*/

mkdir

/* DOCUMENT mkdir, directory_name

rmdir, directory_name

Create DIRECTORY_NAME with mkdir, or remove it with rmdir.

The rmdir function only works if the directory is empty.

SEE ALSO: mkdirp, cd, lsdir, get_cwd, get_home

*/

mkdirp

/* DOCUMENT mkdirp, directory_name

Create DIRECTORY_NAME, creating any missing parent directories

(like UNIX utility mkdir -p).  Unlike mkdir, signals error if

the creation is unsuccessful.  If DIRECTORY_NAME already exists

and is a directory, mkdirp is a no-op.

SEE ALSO: mkdir

*/

nallof

/* SEE allof     */

FROM nameof TO open

nameof

/* DOCUMENT nameof(object)

If OBJECT is a function or a structure definition, returns the

name of the func or struct as it was defined (not necessarily

the name of the variable passed to the nameof function).

SEE ALSO: typeof

*/

noneof

/* SEE allof     */

numberof

/* DOCUMENT numberof(object)

returns the number of elements if object is an array, or 0 if not.

SEE ALSO: sizeof, dimsof, typeof, structof

*/

open

/* DOCUMENT f= open(filename)

or f= open(filename, filemode)

or f= open(filename, filemode, errmode)

opens the file FILENAME according to FILEMODE (both are strings).

If ERRMODE is non-nil and non-zero, fail by returning nil F,

otherwise failure to open or create the file is a runtime error.

To use ERRMODE to check for the existence of a file:

if (open(filename,"r",1)) file_exists;

else file_does_not_exist;

The return value F is an IOStream (or just stream for short).  When

the last reference to this return value is discarded, the file will

be closed.  The file can also be explicitly closed with the close

function.  The FILEMODE determines whether the file is to be

opened in read, write, or update mode, and whether writes are

restricted to the end-of-file (append mode).  FILEMODE also

determines whether the file is opened as a text file or as a

binary file.  FILEMODE can have the following values, which are

the same as for the ANSI standard fopen function:

"r"     - read only

"w"     - write only, random access, existing file overwritten

"a"     - write only, forced to end-of-file,

existing file preserved

"r+"    - read/write, random access, existing file preserved

"w+"    - read/write, random access, existing file overwritten

"a+"    - read/write, reads random access,

writes forced to end-of-file, existing file preserved

"rb"  "wb"  "ab"  "r+b"  "rb+"  "w+b"  "wb+"  "a+b"  "ab+"

without b means text file, with b means binary file

The default FILEMODE is "r" -- open an existing text file for

reading.

The read and write functions perform I/O on text files.

I/O to binary files may be performed explicitly using the save

and restore functions, or implicitly by using the stream variable

FROM open TO orgsof

F as if it were a data structure instance (e.g.- f.x refers to

variable x in the binary file f).

SEE ALSO: create, close, read, write, rdline, bookmark, backup, popen

rename, remove, save, restore

*/

open102

/* SEE close102     */

openb

/* DOCUMENT file= openb(filename)

or file= openb(filename, clogfile)

open the existing file FILENAME for read-only binary I/O.

(Use updateb or createb, respectively, to open an existing file

with read-write access or to create a new file.)

If the CLOGFILE argument is supplied, it represents the structure

of FILENAME in the Clog binary data description language.

After an openb, the file variable may be used to extract variables

from the file as if it were a structure instance.  That is, the

expression "file.var" refers to the variable "var" in file "file".

A complete list of the variable names present in the file may

be obtained using the get_vars function.  If the file contains

history records, the jt and jc functions may be used to set the

current record -- initially, the first record is current.

The restore function may be used to make memory copies of data

in the file; this will be faster than a large number of

references to "file.var".

SEE ALSO: updateb, createb, open, cd

show, jt, jc, restore

get_vars, get_times, get_ncycs, get_member, has_records

set_blocksize, dump_clog, read_clog, recover_file

openb_hooks, open102, close102, get_addrs

*/

openb_hooks

/* DOCUMENT openb_hooks

list of functions to be tried by openb if the file to be

opened is not a PDB file.  By default,

openb_hooks= _lst(_not_pdbf, _not_cdf).

The hook functions will be called with the file as argument

(e.g.- _not_cdf(file)), beginning with _car(openb_hooks), until

one of them returns 0.  Note that a hook should return 0 if it

"recognizes" the file as one that it should be able to open, but

finds that the file is misformatted (alternatively, it could call

error to abort the whole process).

*/

orgsof

/* DOCUMENT orgsof(object)

returns a vector of integers describing the dimensions of OBJECT.

The format of the vector is [number of dims, origin1, origin2, ...].

By default, dimension origins are ignored, but use_origins changes

this.  The dimsof function returns the length of each dimension.

*** NOTE NOTE NOTE ***

Unless use_origins(1) is in effect, orgsof will always return

FROM orgsof TO plug_in

1 for all of the originI in the list.  Thus, whether use_origins(1)

is in effect or not, you are guaranteed that x(orgsof(x)(2)) is the

first element of x.

*** DEPRECATED ***

Do not use index origins.  Your brain will explode sooner or later.

SEE ALSO: dimsof, typeof, structof, numberof, sizeof, use_origins

*/

pc_primitives

/* DOCUMENT pc_primitives, file

sets FILE primitive data types to be native to IBM PC.

*/

pi

/* DOCUMENT pi

roughly 3.14159265358979323846264338327950288

*/

plug_dir

/* DOCUMENT old_dirs = plug_dir(dirname)

or plug_dir

or current_dirs = plug_dir()

causes plug_in to look in DIRNAME for dynamic library files, in

addition to Y_HOME/lib.  DIRNAME may be an array of strings to

search multiple directories.  The return value is the previous

list of directories searched by plug_in.  No checks are made

for repeats, so be careful not to grow the list indiscriminately.

In the second form (or called as a subroutine with DIRNAME []),

empties the plug_in search path; in the third form does not

alter the current search path.  Note that Y_HOME/lib is omitted

from the end of the return value, even though it is searched.

SEE ALSO: plug_in

*/

plug_in

/* DOCUMENT plug_in, "pkgname"

Dynamically link to yorick package "pkgname".  The compiled

functions of the package are in a shared object file; these

files have a naming convention which differs slightly on different

platforms.  On most UNIX systems (including Mac OS X), the

binary file is named pkgname.so.  On MS Windows systems, the

binary file is named pkgname.dll.  On HPUX systems, the name is

pkgname.sl.  The "pkgname" argument to plug_in does not include

this platform-dependent file extension, so that the yorick code

containing the plug_in command will be portable.

After dynamically linking the compiled routines in the pkgname

shared object binary, yorick runs the function (which must be

present) yk_pkgname in order to initialize the package.  At

minimum yk_pkgname returns lists of the new compiled (builtin)

functions defined by the package and the names by which they

may be invoked by interpreted code.

FROM plug_in TO pr1

Additionally, yk_pkgname returns a list of files to be included

containing interpreted wrapper functions for the compiled routines

and DOCUMENT comments for the help system.  Conventionally, these

include files are located in the Y_SITE/i0 or Y_HOME/lib directories,

and the name (of one) of the file(s) is pkgname.i.  If the package

has been statically linked (i.e.- not by plug_in), these .i files

are automatically included when yorick starts.  However, if the

package is loaded dynamically by plug_in, you must arrange to

include one or all of these .i files as you would any interpreted

package (e.g.- by the autoload or require functions, or manually).

The upshot of all this is that the plug_in function is designed

to be placed at the top of the .i files associated with the

package.  You are not supposed to call plug_in manually, rather

when you #include (or autoload) a .i file which needs compiled

functions, that .i file invokes plug_in to perform any required

dynamic linking to compiled code.  Thus, the end user does not

do anything differently for a package that uses dynamically loaded

compiled code, than for a purely interpreted package.

Yorick dynamic library support solves a distribution problem.  For

debugging and creating compiled packages for your own use, you want

to build special versions of yorick with your compiled routines

statically linked.  In order to support platforms on which there

is no dynamic linking, if you call the plug_in function for a

package that is statically linked (e.g.- plug_in,"yor"), the

function will silently become a no-op when it notices that the

"pkgname" package was already loaded at startup.

SEE ALSO: plug_dir, include, require, autoload

*/

poly

/* DOCUMENT poly(x, a0, a1, a2, ..., aN)

returns the polynomial  A0 + A1*x + A2*x^2 + ... + AN*X^N

The data type and dimensions of the result, and conformability rules

for the inputs are identical to those for the expression.

*/

popen

/* DOCUMENT f= popen(command, mode)

opens a pipe to COMMAND, which is executed as with the system

function.  If MODE is 0, the returned file handle is open for

reading, and you are reading the stdout produced by COMMAND.

If MODE is 1, f is opened for writing and you are writing to

the stdin read by COMMAND.

SEE ALSO: open, system

*/

pr1

/* DOCUMENT pr1(x)

returns text representing expression X, equivalent to print(X)(1).

SEE ALSO: print, swrite

*/

FROM print TO process_argv

print

/* DOCUMENT print, object1, object2, object3, ...

or print(object1, object2, object3, ...)

prints an ASCII representation of the OBJECTs, in roughly the format

they could appear in Yorick source code.  When invoked as a subroutine

(in the first form), output is to the terminal.  When invoked as a

function (int the second form), the output is stored as a vector of

strings, one string per line that would have been output.

Printing a structure definition prints the structure definition;

printing a function prints its "func" definition; printing files,

bookmarks, and other objects generally provides some sort of

useful description of the object.

SEE ALSO: pr1, print_format, write, exit, error, nameof, typeof

*/

print_format

/* DOCUMENT print_format, line_length, char=, short=, int=, float=,

double=, complex=, pointer=

sets the format string the print function will use for each of

the basic data types.  Yorick format strings are the same as the

format strings for the printf function defined in the ANSI C standard.

The default strings may be restored individually by setting the

associated format string to ""; all defaults are restored if

print_format is invoked with no arguments.  The default format strings

are:  "0x%02x", "%d", "%d", "%ld", "%g", "%g", and "%g%+gi".

Note that char and short values are converted to int before being

passed to printf, and that float is converted to double.

If present, an integer positional argument is taken as the line

length; <=0 restores the default line length of 80 characters.

SEE ALSO: print, write, nameof, typeof

*/

process_argv

/* DOCUMENT remaining= process_argv()

-or- remaining= process_argv("your startup message")

Performs standard command line processing.  This function is

invoked by the default custom.i file (in $Y_SITE/i); you

can also invoke it from your personal ~/yorick/custom.i file.

The process_argv calls get_argv, removes any arguments of

the form "-ifilename" or "-i filename" (the latter is a pair of

arguments.  It returns any arguments not of this form as its

result, after including any filenames it found in the order

they appeared on the command line.

The optional string argument may be an array of strings to print

a multi-line message.

A Yorick package may define the function get_command_line in

order to feed process_argv something other than get_argv.

SEE ALSO: batch

*/

FROM ptcen TO randomize

ptcen

/* DOCUMENT ptcen(zncen)

or ptcen(zncen, ireg)

returns point centered version of the 2-D zone centered array ZNCEN.

The result is imax-by-jmax if ZNCEN is (imax-1)-by-(jmax-1).

If the region number array IREG is specified, zones with region

number 0 are not included in the point centering operation.

Note that IREG should have dimensions imax-by-jmax; the first

row and column of IREG are ignored.

Without IREG, ptcen(zncen) is equivalent to zncen(pcen,pcen).

SEE ALSO: zncen, uncen

*/

quit

/* DOCUMENT quit

Exit YMainLoop when current task finishes.

Normally this terminates the program.

*/

random

/* DOCUMENT random(dimension_list)

random_seed, seed

returns an array of random double values with the given

DIMENSION_LIST (nil for a scalar result), uniformly distributed

on the interval from 0.0 to 1.0.

The algorithm is from Press and Teukolsky, Computers in Physics,

vol. 6, no. 5, Sep/Oct 1992 (ran2).  They offer a reward of $1000

to anyone who can exhibit a statistical test that this random

number generator fails in a "non-trivial" way.

The random_seed call reinitializes the random number sequence;

SEED should be between 0.0 and 1.0 non-inclusive; if SEED is

omitted, nil, or out of range, the sequence is reinitialized as

when Yorick starts.

The numbers are actually at the centers of 2147483562 equal width

bins on the interval [0,1].  Although only these 2 billion numbers

are possible, the period of the generator is roughly 2.3e18.

SEE ALSO: randomize

*/

random_seed

/* SEE random     */

randomize

/* DOCUMENT randomize

randomize()

set the seed for random "randomly" (based on the timer clock

and the current state of random).  As a function, returns the

value of the seed passed to random_seed.

SEE ALSO: random, random_seed

*/

FROM rdfile TO read

rdfile

/* DOCUMENT rdfile(f)

or rdfile(f, nmax)

reads all remaining lines (or at most NMAX lines) from file F.

If NMAX is omitted, it defaults to 2^20 lines (about a million).

The result is an array of strings, one per line of F.

SEE ALSO: rdline

*/

rdline

/* DOCUMENT rdline(f)

or rdline(f, n, prompt= pstring)

returns next line from stream F (stdin if F nil).  If N is non-nil,

returns a string array containing the next N lines of F.  If

end-of-file occurs, rdline returns nil strings.  If F is nil,

uses the PSTRING to prompt for input (default "read> ").

SEE ALSO: read, open, close, bookmark, backup, read_n, rdfile

*/

re_part

/* DOCUMENT re_part(z)

returns the real part of its argument.  (Same as double(z).)

Unlike z.re, works if z is not complex.

*/

read

/* DOCUMENT n= read(f, format=fstring, obj1, obj2, ...)

or n= read(prompt= pstring, format=fstring, obj1, obj2, ...)

or n= sread(source, format=fstring, obj1, obj2, ...)

reads text from I/O stream F (1st form), or from the keyboard (2nd

form), or from the string or string array SOURCE (3rd form),

interprets it according to the optional FSTRING, and uses that

interpretation to assign values to OBJ1, OBJ2, ...  If the input

is taken from the keyboard, the optional prompt PSTRING (default

"read> ") is printed before each line is read.  The Yorick write

function does not interact with the read function -- writes are

always to end-of-file, and do not affect the sequence of lines

returned by read.  The backup (and bookmark) function is the

only way to change the sequence of lines returned by read.

There must be one non-supressed conversion specifier (see below)

in FSTRING for each OBJ to be read; the type of the conversion

specifier must generally match the type of the OBJ.  That is,

an integer OBJ requires an integer specifier (d, i, o, u, or x)

in FSTRING, a real OBJ requires a real specifier (e, f, or g),

and a string OBJ requires a string specifier (s or []).  An OBJ

may not be complex, a pointer, a structure instance, or any non-

array Yorick object.  If FSTRING is not supplied, or if it has

fewer conversion specifiers than the number of OBJ arguments,

then Yorick supplies default specifiers ("%ld" for integers,

"%lg" for reals, and "%s" for strings).  If FSTRING contains more

specifiers than there are OBJ arguments, the part of FSTRING

beginning with the first specifier with no OBJ is ignored.

The OBJ may be scalar or arrays, but the dimensions of every OBJ

FROM read TO read

must be identical.  If the OBJ are arrays, Yorick behaves as

if the read were called in a loop numberof(OBJ1) times, filling

one array element of each of the OBJ according to FSTRING on

each pass through the loop.  (Note that this behavior includes

the case of reading columns of numbers by a single call to read.)

The return value N is the total number of scalar assignments

which were made as a result of this call.  (If there were 4

OBJ arguments, and each was an array with 17 elements, a return

value of N==35 would mean the following:  The first 8 elements

of OBJ1, OBJ2, OBJ3, and OBJ4 were read, and the 9th element of

OBJ1, OBJ2, and OBJ3 was read.)  The read function sets any

elements of the OBJ which were not read to zero -- hence,

independent of the returned N, the all of the old data in the

OBJ arguments is overwritten.

The read or sread functions continue reading until either:

(1) all elements of all OBJ have been filled, or (2) end-of-file

(or end of SOURCE for sread) is reached ("input failure"), or

(3) part of FSTRING or a conversion specifier supplied by

default fails to match the source text ("matching failure").

The FSTRING is composed of a series of "directives" which are

(1) whitespace -- means to skip any amount of whitespace in the

source text

(2) characters other than whitespace and % -- must match the

characters in the source text exactly, or matching failure

occurs and the read operation stops

(3) conversion specifiers beginning with % and ending with a

character specifying the type of conversion -- optionally

skip whitespace, then convert as many characters as

continue to "look like" the conversion type, possibly

producing a matching failure

The conversion specifier is of the form %*WSC, where:

* is either the character '*' or not present

A specifier beginning with %* does not correspond to any of

the OBJ; the converted value will be discarded.

W is either a positive decimal integer specifying the maximum

field width (not including any skipped leading whitespace),

or not present if any number of characters up to end-of-line

is acceptable.

S is either one of the characters 'h', 'l', or 'L', or not

present.  Yorick allows this for compatibility with the C

library functions, but ignores it.

C is a character specifying the type of conversion:

d   - decimal integer

i   - decimal, octal (leading 0), or hex (leading 0x) integer

o   - octal integer

u   - unsigned decimal integer (same as d for Yorick)

x, X            - hex integer

e, f, g, E, G   - floating point real

s   - string of non-whitespace characters

[xxx]   - (xxx is any sequence of characters) longest string

of characters matching those in the list

[^xxx]  - longest string of characters NOT matching those in

FROM read TO remove

the list (this is how you can extend %s to be

delimited by something other than whitespace)

%   - the ordinary % character; complete conversion

specification must be "%%"

The read function is modeled on the ANSI standard C library

fscanf and sscanf functions, but differs in several respects:

(1) Yorick's read cannot handle the %c, %p, or %n conversion

specifiers in FSTRING.

(2) Yorick's read never results in a portion of a line

being read -- any unused part of a line is simply discarded

(end FSTRING with "%[^\n]" if you want to save the trailing

part of an input line).

(3) As a side effect of (2), there are some differences between

fscanf and Yorick's read in how whitespace extending across

newlines is handled.

SEE ALSO: rdline, write, open, close, bookmark, backup, save, restore,

read_n

*/

read_clog

/* DOCUMENT file= read_clog(file, clog_name)

raw routine to set the binary data structure of FILE according

to the text description in the Contents Log file CLOG_NAME.

*/

read_n

/* DOCUMENT read_n, f, n0, n1, n2, ...

grabs the next numbers N0, N1, N2, ... from file F, skipping over

any whitespace, comma, semicolon, or colon delimited tokens which

are not numbers.  (Actually, only the first and last characters of

the token have to look like a number -- 4xxx3 would be read as 4.)

***WARNING*** at most ten Ns are allowed

The Ns can be arrays, provided all have the same dimensions.

SEE ALSO: read, rdline

*/

recover_file

/* DOCUMENT recover_file, filename

or recover_file, filename, clogfile

writes the descriptive information at the end of a corrupted

binary file FILENAME from its Contents Log file CLOGFILE, which

is FILENAME+"L" by default.

*/

reform

/* DOCUMENT reform(x, dimlist)

*    returns array X reshaped according to dimension list DIMLIST.

* SEE ALSO: array, dimsof

*/

remove

/* SEE rename     */

FROM rename TO save

rename

/* DOCUMENT rename, old_filename, new_filename

remove filename

rename or remove a file.

SEE ALSO: open, close, openb

*/

require

/* SEE include     */

reshape

/* DOCUMENT reshape, reference, address, type, dimension_list

or reshape, reference, type, dimension_list

or reshape, reference

The REFERENCE must be an unadorned variable, not an expression;

reshape sets this variable to an LValue at the specified ADDRESS

with the specified TYPE and DIMENSION_LIST.  (See the array

function documentation for acceptable DIMENSION_LIST formats.)

If ADDRESS is an integer (e.g.- a long), the programmer is

responsible for assuring that the data at ADDRESS is valid.

If ADDRESS is a (Yorick) pointer, Yorick will assure that the

data pointed to will not be discarded, and the reshape will

fail if TYPE and DIMENSION_LIST extend beyond the pointee

bounds.  In the second form, ADDRESS is taken to be &REFERENCE;

that is, the TYPE and DIMENSION_LIST of the variable are changed

without doing any type conversion.  In the third form, REFERENCE

is set to nil ([]).  (Simple redefinition will not work on a

variable defined using reshape.)

WARNING: There are almost no situations for which reshape is

the correct operation.  Use reform instead.

SEE ALSO: reform, array, dimsof, numberof, is_array, eq_nocopy

*/

restore

/* SEE save     */

resume

/* SEE suspend     */

rmdir

/* SEE mkdir     */

save

/* DOCUMENT save, file, var1, var2, ...

restore, file, var1, var2, ...

saves the variables VAR1, VAR2, etc. in the binary file FILE,

or restores them from that file.

The VARi may be either non-record or record data in the case that

FILE contains records.

If one of the VARi does not already exist in FILE, it is created

by the save command; after add_record, save adds or stores VARi to

the current record.  See add_record for more.  The VARi may be

structure definitions (for the save command) to declare data

structures for the file.  This is necessary only in the case that

FROM save TO set_idler

a record variable is a pointer -- all of the potential data types

of pointees must be known.  No data structures may be declared

using the save command after the first record has been added.

If no VARi are present, save saves all array variables, and

restore restores every non-record variable in the file if there

is no current record, and every variable in the current record if

there is one.

SEE ALSO: openb, createb, updateb, get_vars, add_record, get_addrs

jt, jc, _read, _write, data_align

*/

sech

/* DOCUMENT sech(x)

csch(x)

returns the hyperbolic secant (1/cosh) or cosecant (1/sinh) of

its argument, without overflowing for large x.

SEE ALSO: sinh, cosh, tanh, asinh, acosh, atanh

*/

set_blocksize

/* DOCUMENT set_blocksize, file, blocksize

sets smallest cache block size for FILE to BLOCKSIZE.  BLOCKSIZE

is rounded to the next larger number of the form 4096*2^n if

necessary; cache blocks for this file will be multiples of

BLOCKSIZE bytes long.  The default BLOCKSIZE is 0x4000 (16 KB).

SEE ALSO: openb, updateb, createb, save, restore, _read, _write

*/

set_filesize

/* DOCUMENT set_filesize, file, filesize

sets the new family member threshhold for FILE to FILESIZE.

Whenever a new record is added (see add_record), if the current file

in the FILE family has at least one record and the new record would

cause the current file to exceed FILESIZE bytes, a new family

member will be created to hold the new record.

Note that set_filesize must be called after the first call to

add_record.

The default FILESIZE is 0x800000 (8 MB).

SEE ALSO: openb, updateb, createb, add_record

*/

set_idler

/* DOCUMENT set_idler, idler_function

sets the idler function to IDLER_FUNCTION.  Instead of waiting

for keyboard input when all its tasks are finished, the interpreter

will invoke IDLER_FUNCTION with no arguments.  The idler function

is normally invoked only once, so input from the keyboard resumes

after one call to the idler.  Of course, an idler is free to call

set_idler again before it returns, which will have the effect of

calling that function in a loop.

SEE ALSO: batch, maybe_prompt, after, after_error

*/

FROM set_path TO set_vars

set_path

/* DOCUMENT set_path, "dir1:dir2:dir3:..."

or set_path

sets the include file search path to the specified list of

directories.  The specified directories are searched left to

right for include files specified as relative file names in

#include directives, or to the include or require functions.

If the argument is omitted, restores the default search path,

".:~/yorick:~/Yorick:Y_SITE/i:Y_SITE/contrib:Y_SITE/i0:Y_HOME/lib",

where y_site is the main Yorick directory for this site.

The Y_LAUNCH directory is the directory which contains the

executable; this directory is omitted if it is the same as

Y_SITE.

Only the "end user" should ever call set_path, and then only in

his or her custom.i file, for the purpose of placing a more

elaborate set of personal directories containing Yorick procedures.

For example, if someone else maintains Yorick code you use, you

might put their ~/yorick on your include path.

SEE ALSO: Y_LAUNCH, Y_SITE, include, require, get_path

*/

set_primitives

/* DOCUMENT set_primitives, file, prims

Return the primitive data types for FILE as an array of 32

integers.  Versions for particular machines are defined in

prmtyp.i, and can be accessed using functions like

sun_primitives or i86_primitives.  See __xdr for a complete

list.  The format is:

[size, align, order] repeated 6 times for char, short, int,

long, float, and double, except that char align is always 1,

so result(2) is the structure alignment (see struct_align).

[sign_address,  exponent_address, exponent_bits,

mantissa_address, mantissa_bits,

mantissa_normalization, exponent_bias] repeated twice for

float and double.  See the comment at the top of prmtyp.i

for an explanation of these fields.

the total number of items is thus 3*6+7*2=32.

SEE ALSO: get_primitives, createb, __xdr, __i86

*/

set_vars

/* DOCUMENT set_vars, file, names

or set_vars, file, nonrec_names, rec_names

Change the names of the variables in FILE to NAMES.  If the

file has record variables, you can use the second form to change

the record variable names.  Either of the two lists may be nil

to leave those names unchanged, but if either is not nil, it must

be a 1D array of strings whose length exactly matches the number

of that type of variable actually present in the file.

SEE ALSO: openb, updateb, has_records, get_vars

*/

FROM sgi64_primitives TO sizeof

sgi64_primitives

/* DOCUMENT sgi64_primitives, file

sets FILE primitive data types to be native to 64-bit SGI workstations.

*/

show

/* DOCUMENT show, f

or show, f, pat

or show, f, 1

prints a summary of the variables contained in binary file F.

If there are too many variables, use the second form to select

only those variables whose first few characters match PAT.

In the third form, continues the previous show command where it

left off -- this may be necessary for files with large numbers of

variables.

The variables are printed in alphabetical order down the columns.

The print function can be used to obtain other information about F.

SEE ALSO: openb, jt, jc

*/

sign

/* DOCUMENT sign(x)

returns algebraic sign of it argument, or closest point on the

unit circle for complex x.  Guaranteed that x==sign(x)*abs(x).

sign(0)==+1.

SEE ALSO: abs

*/

sin

/* DOCUMENT sin(x)

cos(x)

tan(x)

returns the sine, cosine, or tangent of its argument,

which is in radians.

SEE ALSO: asin, acos, atan

*/

sinh

/* DOCUMENT sinh(x)

cosh(x)

tanh(x)

returns the hyperbolic sine, cosine, or tangent of its argument.

SEE ALSO: sech, csch, asinh, acosh, atanh

*/

sizeof

/* DOCUMENT sizeof(object)

returns the size of the object in bytes, or 0 for non-array objects.

sizeof(structure_definition) returns the number of bytes per instance.

sizeof(binary_file) returns the file size in bytes.

SEE ALSO: dimsof, typeof, structof, numberof

*/

FROM sort TO spanl

sort

/* DOCUMENT sort(x)

or sort(x, which)

returns an array of longs with dimsof(X) containing index values

such that X(sort(X)) is a monotonically increasing array.  X can

contain integer, real, or string values.  If X has more than one

dimension, WHICH determines the dimension to be sorted.  The

default WHICH is 1, corresponding to the first dimension of X.

WHICH can be non-positive to count dimensions from the end of X;

in particular a WHICH of 0 will sort the final dimension of X.

WARNING: The sort function is non-deterministic if some of the

values of X are equal, because the Quick Sort algorithm

involves a random selection of a partition element.

For information on sorting with multiple keys (and on making

sort deterministic), type the following:

#include "msort.i"

help, msort

SEE ALSO: median, digitize, interp, integ, histogram

*/

span

/* DOCUMENT span(start, stop, n)

or span(start, stop, n, which)

returns array of N doubles equally spaced from START to STOP.

The START and STOP arguments may themselves be arrays, as long as

they are conformable.  In this case, the result will have one

dimension of length N in addition to dimsof(START, STOP).

By default, the result will be N-by-dimsof(START, STOP), but

if WHICH is specified, the new one of length N will be the

WHICHth.  WHICH may be non-positive to position the new

dimension relative to the end of dimsof(START, STOP); in

particular WHICH of 0 produces a result with dimensions

dimsof(START, STOP)-by-N.

SEE ALSO: spanl, indgen, array

*/

spanl

/* DOCUMENT spanl(start, stop, n)

or spanl(start, stop, n, which)

similar to the span function, but the result array have N points

spaced at equal ratios from START to STOP (that is, equally

spaced logarithmically).  See span for discussion of WHICH argument.

START and STOP must have the same algebraic sign for this to make

any sense.

SEE ALSO: span, indgen, array

*/

FROM spawn TO spawn

spawn

/* DOCUMENT process = spawn(argv, on_stdout)

or process = spawn(argv, on_stdout, on_stderr)

starts the process named in ARGV(1) with additional arguments

in any subsequent elements of ARGV (which is a scalar or 1D

array of strings).  The ON_STDOUT and optional ON_STDERR

are interpreted functions declared like this:

func ON_STDOUT(msg)

{

commands to process msg on stdout from process

}

Yorick will invoke ON_STDOUT asynchronously if process

emits text to its stdout.  Yorick includes the process in

the list of event sources, which it polls whenever it waits

for input.  If the optional ON_STDERR is provided, it is

called asynchronously whenever process emits a line to stderr;

with no ON_STDERR, the process will share yorick's stderr,

which generally means the process stderr prints at the terminal.

(Note that you can make the third argument the same as the second

if you want to use the same function to handle stdout and stderr.)

When the process terminates, ON_STDOUT is invoked with

string(0) and the process object becomes inactive.  Note that

ON_STDOUT and ON_STDERR are invoked via the name they were

originally defined with (in the func or extern statement for

interpreted and compiled functions, respectively).

The object returned by spawn, process, can be used to send input

or signals to the process:

process, msg;

where msg is a string, sends msg to the process's stdin.

process, signum;

sends process the specified signal (e.g.- signum=2 sends SIGINT,

like hitting control-C, while signum=9 kills the process), if

signum is an integer (as opposed to a string).  (Normally you

should not send signals to a process.)  If you redefine the

final reference to process, for example by

process = [];

yorick will disconnect from the process, closing its end of

the stdin, stdout, and, optionally, stderr pipes.  For many

programs, this will stop the program, but if the program can

continue running without stdin and stdout, it will continue

running.  (If yorick were a shell, the process would be running

in the background; if the process would live beyond the shell

which created it, it will also survive its process variable

being freed.)

Note: funcdef may be extremely useful for writing ON_STDOUT.

SEE ALSO: popen, system, suspend, funcdef, after, spawn_callback

*/

FROM spawn_callback TO strchar

spawn_callback

/* DOCUMENT spawn_callback -->

func on_stdout(msg) {

extern fragment; <or otherwise manage fragment>

lines = spawn_callback(fragment, msg);

for (i=1 ; i<=numberof(lines) ; i++) {

line = lines(i);

if (!line) {

<handle process has died>

} else {

<handle complete line emitted by process>

}

}

}

Here is a template for a callback function to be passed to spawn.

The spawn_callback function buffers any fragmentary lines,

delivering only complete lines as output.  Note that FRAGMENT

must somehow be managed between calls to on_stdout; it should

be intialized to [] before calling spawn.

SEE ALSO: spawn

*/

sqrt

/* DOCUMENT sqrt(x)

returns the square root of its argument.

SEE ALSO: abs

*/

sread

/* SEE read     */

strcase

/* DOCUMENT strcase(upper, string_array)

or strcase, upper, string_array

returns STRING_ARRAY with all strings converted to upper case

if UPPER is non-zero.  If UPPER is zero, result is lower case.

(For characters >=0x80, the case conversion assumes the ISO8859-1

character set.)

Called as a subroutine, strcase converts STRING_ARRAY in place.

SEE ALSO: string, strlen, strpart, strglob, strfind, strgrep, strword

*/

strchar

/* DOCUMENT strchar(string_array)

or strchar(char_array)

converts STRING_ARRAY to an array of characters, or CHAR_ARRAY

to an array of strings.  The return value is always a 1D array,

except in the second form if CHAR_ARRAY contains only a single

string, the result will be a scalar string.  Each string is

stored in sequence including its trailing '\0' character, with

any string(0) elements treated as if they were "".  Going in

the opposite direction, a '\0' before any non-'\0' characters

produces string(0), so that "" can never be an element of

the result, and if the final char (of the leading dimension)

FROM strchar TO streplace

is not '\0', an implicit '\0' is assumed beyond the end of the

input char array.  For example,

strchar(["a","b","c"]) --> ['a','\0','b','\0','c','\0']

strchar([['a','\0','b'],['c','\0','\0']]) --> ["a","b","c",string(0)]

The string and pointer data types themselves also convert between

string and char data, avoiding the quirks of strchar.

SEE ALSO: string, strpart, strword, strfind

*/

streplace

/* DOCUMENT streplace(string_array, start_end, to_string)

replaces the part(s) START_END of STRING_ARRAY by TO_STRING.

The leading dimension of START_END must be a multiple of 2,

while any trailing dimensions must be conformable with the

dimensions of STRING_ARRAY.  The TO_STRING must be conformable

with STRING_ARRAY if the leading dimension of START_END is 2.

An element of START_END may represent "no match" (for example,

when end<start), in which case streplace silently skips the

replacement.  Note that when start==end, streplace functions

as text insertion.

However, if the leading dimension of START_END = 2*n > 2, then

TO_STRING must have a leading dimension conformable with n

(that is, of length either 1 or n).  In this case, streplace

performs multiple replacements within each string.  In order

for multiple replacements to be meaningful, the START_END

must be disjoint and sorted, as returned by strfind or

strgrep with a repeat count, or by strword.  In other words,

the first dimension of START_END should be non-decreasing,

except where end<start indicates no replacement.

Invoked as a subroutine, streplace operates on STRING_ARRAY

in place.

Examples:

s = "Hello, world!"

streplace(s,[0,5], "Goodbye")

-->  "Goodbye, world!"

streplace(s,[0,5,7,7], ["Goodbye","cruel "])

-->  "Goodbye, cruel world!"

streplace(s,[0,5,7,7,12,13], ["Goodbye","cruel ","?"])

-->  "Goodbye, cruel world?"

streplace(s,[0,5,0,-1,12,13], ["Goodbye","cruel ","?"])

-->  "Goodbye, world?"

streplace([s,s],[0,5], ["Goodbye", "Good bye"])

-->  ["Goodbye, world!", "Good bye, world!"]

streplace([s,s],[0,5,7,7], [["Goodbye","cruel "], ["Good bye",""]])

-->  ["Goodbye, cruel world!", "Good bye, world!"]

SEE ALSO: string, strfind, strgrep, strword, strpart

*/

FROM strfind TO strglob

strfind

/* DOCUMENT strfind(pat, string_array)

or strfind(pat, string_array, off)

finds pattern PAT in STRING_ARRAY.  Optional OFF is an integer

array conformable with STRING_ARRAY or 0-origin offset(s) within

the string(s) at which to begin the search(es).  The return value

is a [start,end] offset pair specifying the beginning and end

of the first match, or [len,-1] if none, with trailing dimensions

the same as the dimensions of STRING_ARRAY.  This return value

is suitable as an input to the strpart or streplace functions.

The strfind function is the simpler string pattern matcher:

strfind - just finds a literal pattern (possibly case insensitive)

strgrep - matches a pattern containing complex regular expressions

Additionally, the strglob function does filename wildcard matching.

Keywords:

n=  (default 1) returns list of first n matches, so leading

dimension of result will be 2*n

case=  (default 1) zero for case-insensitive search

back=  (default 0) non-zero for backwards search

If back!=0 and n>1, the last match is listed as the

last start-end pair, so the output pairs still appear

in increasing order, and the first few may be 0,-1

to indicate no match.

Examples:

s = ["one two three", "four five six"]

strfind("o",s)  -->  [[0,1], [1,2]]

strfind(" t",s)  -->   [[3,5], [13,-1]]

strfind(" t",s,n=2)  -->   [[3,5,7,9], [13,-1,13,-1]]

strfind("e",s,n=2,back=1)  -->   [[11,12,12,13], [0,-1,8,9]]

SEE ALSO: string, strglob, strgrep, strword, strpart, streplace

*/

strglob

/* DOCUMENT strglob(pat, string_array)

or strglob(pat, string_array, off)

test if pattern PAT matches STRING_ARRAY.  Optional OFF is an integer

array conformable with STRING_ARRAY or 0-origin offset(s) within

the string(s) at which to begin the search(es).  The return value

is an int with the same dimensions as STRING_ARRAY, 1 for a match,

and 0 for no match.

PAT can contain UNIX shell wildcard or "globbing" characters:

*   matches any number of characters

?   matches any single character

[abcd]  matches any single character in the list, which may

contain ranges such as [a-z0-9A-Z]

\c  matches the character c (useful for c= a special character)

(note that this is "\\c" in a yorick string)

The strglob function is mostly intended for matching lists of

file names.  Note, in particular, that unlike strfind or strgrep,

FROM strglob TO strgrep

the entire string must match PAT.

Keywords:

case=  (default 1) zero for case-insensitive search

path=  (default 0) 1 bit set means / must be matched by /

2 bit set means leading . must be matched by .

esc=   (default 1) zero means \ is not treated as an escape

The underlying compiled routine is based on the BSD fnmatch

function, contributed by Guido van Rossum.

Examples:

return all files in current directory with .pdb extension:

d=lsdir("."); d(where(strglob("*.pdb", d)));

return all subdirectories of the form "hackNN", case insensitive:

d=lsdir(".",1);

d(where(strglob("hack[0-9][0-9]", d, case=0)));

SEE ALSO: string, strfind, strgrep, strword, strpart, streplace

*/

strgrep

/* DOCUMENT strgrep(pat, string_array)

or strgrep(pat, string_array, off)

finds pattern PAT in STRING_ARRAY.  Optional OFF is an integer

array conformable with STRING_ARRAY or 0-origin offset(s) within

the string(s) at which to begin the search(es).  The return value

is a [start,end] offset pair specifying the beginning and end

of the first match, or [len,-1] if none, with trailing dimensions

the same as the dimensions of STRING_ARRAY.  This return value

is suitable as an input to the strpart or streplace functions.

The underlying compiled routine is based on the regexp package

written by Henry Spencer (copyright University of Toronto 1986),

slightly modified for yorick.

PAT is a regular expression, simliar to the UNIX grep utility.

Every "regular expression" syntax is slightly different; here is

the syntax supported by strgrep:

The following characters in PAT have special meanings:

'[' followed by any sequence of characters followed by ']' is a

"range", which matches any single one of those characters

'^' first means to match any character NOT one in the sequence

'-' in such a sequence indicates a range of characters

(e.g.- "[A-Za-z0-9_]" matches any alphanumeric character

or underscore, while "[^A-Za-z0-9_]" matches anything else)

to include ']' in the sequence, place it first,

to include '-' in the sequence, place it first or last

(or first after a leading '^' in either case)

Note that the following special characters lose their special

meanings inside a range.

'.' matches any single character

'^' matches the beginning of the string (but no characters)

FROM strgrep TO strgrep

'$' matches the end of the string (but no characters)

(that is, ^ and $ serve to anchor a search so that it will

only find a match at the beginning or end of the string)

'\' (that is, a single backslash, which can only be entered

into a yorick string by a double backslash "\\")

followed by any single character eliminates any special

meaning for that character, for example "\\." matches

period, rather than any single character (its special meaning)

'(' followed by a regular expression followed by ')' matches the

regular expression, creating a sub-pattern, which is a type

of atom (see below)

'|' means "or"; it separates branches in a regular expression

'*' after an atom matches 0 or more matches of the atom

'+' after an atom matches 1 or more matches of the atom

'?' after an atom matches 0 or 1 matches of the atom

The definitions of "atom", "branch", and "regular expression" are:

A "regular expression" (which is what PAT is) consists of zero

or more "branches" separated by '|'; it matches anything that

matches one of the branches.

A "branch" consists of zero or more "pieces", concatenated; it

matches a match for the first followed by a match for the second,

etc.

A "piece" is an "atom", optionally followed by '*', '+', or '?';

it matches the atom, or zero or more repetitions of the atom, as

specified by the optional suffix.

Finally, an "atom" is an ordinary single character, or a

'\'-escaped single character (matching that character), or

one of the special characters '.', '^', or '$', or a

[]-delimited range (matching any single character in the range),

or a sub-pattern enclosed in () (matching the sub-pattern).

A maximum of nine sub-patterns is allowed in PAT; these are

numbered 1 through 9, in order of their opening '(' in PAT.

This recursive definition of regular expressions often leads to

ambiguities, both subtle and glaring.  Here is Henry Spencer's

synopsis of how his routines behave:

-------------------------------------------------------------------

If a regular expression could match two different parts of the

input string, it will match the one which begins earliest.  If both

begin in the same place but match different lengths, or match the

same length in different ways, life gets messier, as follows.

In general, the possibilities in a list of branches are considered

in left-to-right order, the possibilities for `*', `+', and `?' are

considered longest-first, nested constructs are considered from the

outermost in, and concatenated constructs are considered leftmost-

first.  The match that will be chosen is the one that uses the

earliest possibility in the first choice that has to be made.  If

FROM strgrep TO strgrep

there is more than one choice, the next will be made in the same

manner (earliest possibility) subject to the decision on the first

choice.  And so forth.

For example, `(ab|a)b*c' could match `abc' in one of two ways. The

first choice is between `ab' and `a'; since `ab' is earlier, and

does lead to a successful overall match, it is chosen. Since the

`b' is already spoken for, the `b*' must match its last possibility

-the empty string- since it must respect the earlier choice.

In the particular case where no `|'s are present and there is only

one `*', `+', or `?', the net effect is that the longest possible

match will be chosen.  So `ab*', presented with `xabbbby', will

match `abbbb'.  Note that if `ab*' is tried against `xabyabbbz', it

will match `ab' just after `x', due to the begins-earliest rule.

(In effect, the decision on where to start the match is the first

choice to be made, hence subsequent choices must respect it even if

this leads them to less-preferred alternatives.)

-------------------------------------------------------------------

When PAT contains parenthesized sub-patterns, strgrep returns

the [start,end] of the entire match by default, but you can

also get the [start,end] of any or all of the sub-patterns

using the sub= keyword (see below).

If PAT does not contain any regular expression constructs, you

should use the strfind function instead of strgrep.  The strglob

function, if appropriate, will also be faster than strgrep.

Keywords:

n=  (default 1) returns list of first n matches, so leading

dimension of result will be 2*n

sub=[n1,n2,...] is a list of the sub-pattern [start,end] pairs

to be returned.  Thus 0 is the whole PAT, 1 is the first

parenthesized sub-pattern, and so on.  The leading

dimension of the result will be 2*numberof(sub)*n.  The

sequence n1,n2,... must strictly increase: n1<n2<...

The default is sub=0, which returns only the whole match.

Examples:

s = "Hello, world!"

strgrep("(Hello|Goodbye), *([a-zA-Z]+)!", s)

--> [0,13]

strgrep("(Hello|Goodbye), *([a-z]*|[A-Z]*)!", s, sub=[1,2])

--> [0,5,7,12]

strgrep("(Hello|Goodbye), *([a-z]*|[A-Z]*)!", s, sub=[0,2])

--> [0,13,7,12]

strgrep("(Hello|Goodbye), *(([A-Z]*)|([a-z]*))!", s, sub=[0,2,3,4])

--> [0,13,7,12,13,-1,7,12]

SEE ALSO: string, strglob, strfind, strword, strpart, streplace

*/

FROM string TO string

string

/* DOCUMENT string

*

* The yorick string datatype is a character string, e.g.- "Hello, world!".

* Internally, strings are stored as 0-terminated sequences of characters,

* which are 8-bit bytes, the same as the char datatype..

*

* Like numeric datatypes, string behaves as a function to convert objects

* to the string datatype.  There are only two interesting conversions:

*   string(0) is the nil string, like a 0 pointer

*     This is the only string which is "false" in an if test.

*   string(pc) where pc is an array of type pointer where each pointer

*     is either 0 or points to an array of type char, copies the chars

*     into an array of strings, adding a trailing '\0' if necessary

*   pointer(sa) where sa is an array of stringa is the inverse

*     conversion, copying each string to an array of char (including the

*     terminal '\0') and returning an array of pointers to them

* The strchar() function may be a more convenient way to convert from

* string to char and back.

*

* Yorick provides the following means of manipulating string variables:

*

* s+t         when s and t are strings, + means concatentation

*             (this is not perfect nomenclature, since t+s != s+t)

* s(,sum,..)  the sum index range concatentates along a dimension of

*             an array of strings

* sum(s)      concatenates all the strings in an array (in storage order)

*

* strlen(s)          returns length(s) of string(s) s

* strcase(upper, s)  converts s to upper or lower case

* strchar(s_or_c)    converts between string and char arrays

*                    (quick and dirty alternative to string<->pointer)

* strpart(s, m:n)

* strpart(s, sel)    extracts substrings (sel is a [start,end] list)

*   string search functions:

* strglob(pat, s)    shell-like wildcard pattern match, returns 0 or 1

* strword(s, delim)  parses s into word(s), returns a sel

* strfind(pat, s)    simple pattern match, returns a sel

* strgrep(pat, s)    regular expression pattern match, returns a sel

* streplace(s, sel, t)  replaces sel in s by t

*

* strtrim trims leading and/or trailing blanks (based on strword)

* strmatch is a wrapper for strfind that simply returns whether there

*   was a match or not rather than its exact offset

* strtok is a variant of strword that calls strpart in order to

*   return the substrings rather than an sel index list

*

* The strword, strfind, and strgrep functions produce a sel, that is,

* a list of [start,end] offsets into an array of strings.

* These sel indicate portions of a string to be operated on for the

* strpart and streplace functions.

FROM string TO strpart

*

* The sread, swrite, and print functions operate on or produce strings.

* The rdline, rdfile, read, and write functions perform I/O on strings

* to text files.

*/

strlen

/* DOCUMENT strlen(string_array)

returns an long array with dimsof(STRING_ARRAY) containing the

lengths of the strings.  Both string(0) and "" have length 0.

SEE ALSO: string, strchar, strcase, strpart, strfind, strword

*/

strmatch

/* DOCUMENT strmatch(string_array, pattern)

or strmatch(string_array, pattern, case_fold)

or strmatch(string_array, pattern, case_fold)

returns an int array with dimsof(STRING_ARRAY) with 0 where

PATTERN was not found in STRING_ARRAY and 1 where it was found.

If CASE_FOLD is specified and non-0, the pattern match is

insensitive to case, that is, an upper case letter will match

the same lower case letter and vice-versa.

(Consider using strfind directly.)

SEE ALSO: string, strfind, strpart, strlen

*/

strpart

/* DOCUMENT strpart(string_array, m:n)

or strpart(string_array, start_end)

or strpart, string_array, start_end

returns another string array with the same dimensions as

STRING_ARRAY which consists of characters M through N of

the original strings.  M and N are 1-origin indices; if

M is omitted, the default is 1; if N is omitted, the default

is the end of the string.  If M or N is non-positive, it is

interpreted as an index relative to the end of the string,

with 0 being the last character, -1 next to last, etc.

Finally, the returned string will be shorter than N-M+1

characters if the original doesn't have an Mth or Nth

character, with "" (note that this is otherwise impossible)

if neither an Mth nor an Nth character exists.  A 0

is returned for any string which was 0 on input.

In the second form, START_END is an array of [start,end] indices.

A single pair [start,end] is equivalent to the range start+1:end,

that is, start is the index of the character immediately before

the substring (which is to say start is the number of characters

skipped at the beginning of the string).  If end<start, or if

either start or end are <0 or >length, or if the original string

is string(0), strpart returns string(0); otherwise, if end==start,

strpart returns "".

However, the START_END array may have any additional dimensions

(beyond the leading dimension of length 2) which are conformable

with the dimensions of the STRING_ARRAY.  The result will be a

FROM strpart TO strtok

string array with dimensions dimsof(STRING_ARRAY,START_END(1,..)).

Furthermore, the leading dimension of START_END may have any

even length, say 2*n, in which case the leading dimension of

the result will be n.  For example,

strpart(a, [s1,e1,s2,e2,s3,e3,s4,e4])

is equivalent to (or shorthand for)

strpart(a(-,..), [[s1,e1],[s2,e2],[s3,e3],[s4,e4]])(1,..)

In the third form, called a subroutine, strpart operates on

STRING_ARRAY in place.  In this case START_END must have

leading dimension of length 2, although it may have trailing

dimensions as usual.

Examples:

strpart("Hello, world!", 4:6) --> "lo,"

strpart("Hello, world!", [3,6]) --> "lo,"

-it may help to think of [start,end] as the 0-origin offset

of a "cursor" between the characters of the string

strpart("Hello, world!", [3,3]) --> ""

strpart("Hello, world!", [3,2]) --> string(0)

strpart("Hello, world!", [3,20]) --> string(0)

strpart("Hello, world!", [3,6,7,9]) --> ["lo,","wo"]

strpart(["one","two"], [[1,2],[0,1]]) --> ["n","t"]

strpart(["one","two"], [1,2,0,1]) --> [["n","o"],["w","t"]]

SEE ALSO: string, strcase, strlen, strfind, strword

*/

strtok

/* DOCUMENT strtok(string_array, delim)

or strtok(string_array)

or strtok(string_array, delim, n)

strips the first token off of each string in STRING_ARRAY.

A token is delimited by any of the characters in the string

DELIM.  If DELIM is blank, nil, or not given, the

default DELIM is " \t\n" (blanks, tabs, or newlines).

The result is a string array ts with dimensions

2-by-dimsof(STRING_ARRAY); ts(1,) is the first token, and

ts(2,) is the remainder of the string (the character which

terminated the first token will be in neither of these parts).

The ts(2,) part will be 0 (i.e.- the null string) if no more

characters remain after ts(1,); the ts(1,) part will be 0 if

no token was present.  A STRING_ARRAY element may be 0, in

which case (0, 0) is returned for that element.

With yorick-1.6, strtok has been extended to accept multiple

delimiter sets DELIM for successive words, and a repeat count

N for the final DELIM set.  The operation is the same as for

strword, except that the N<=0 special cases are illegal, and

if DELIM consists of only a single set, N=2 is the default

rather than N=1.  The dimensions of the return value are thus

min(2,numberof(DELIM)+N-1)-by-dimsof(STRING_ARRAY).

SEE ALSO: string, strword, strmatch, strpart, strlen

*/

FROM strtrim TO strword

strtrim

/* DOCUMENT strtrim(string_array)

or strtrim(string_array, which)

or strtrim, string_array, which

returns STRING without leading and/or trailing blanks.  WHICH=1

means to trim leading blanks only, WHICH=2 trims trailing blanks

only, while WHICH=3 (the default) trims both leading and trailing

blanks.  Called as a subroutine, strtrim performs this operation

in place.

The blank= keyword, if present, is a list of characters to be

considered "blanks".  Use blank=[lead_delim,trail_delim] to get

different leading and trailing "blanks" definitions.  By default,

blank=" \t\n".  (See strword for more about delim syntax.)

SEE ALSO: string, strpart, strword

*/

struct_align

/* DOCUMENT struct_align, file, alignment

in binary file FILE, align new struct members which are themselves

struct instances to begin at a byte address which is a multiple of

ALIGNMENT.  (This affects members declared explicitly by add_member,

as well as implicitly by save or add_variable.)  If ALIGNMENT is <=0,

returns to the default for this machine.  The struct alignment is in

addition to the alignment implied by the most restrictively aligned

member of the struct.  Most machines want ALIGNMENT of 1.

SEE ALSO: add_member

*/

structof

/* DOCUMENT structof(object)

returns the data type of OBJECT, or nil for non-array OBJECTs.

Use typeof(object) to get the ASCII name of a the data type.

SEE ALSO: typeof, dimsof, numberof, sizeof, nameof

*/

strword

/* DOCUMENT strword(string_array)

or strword(string_array, delim)

or strword(string_array, delim, n)

or strword(string_array, off, delim, n)

scans to the first character in STRING_ARRAY which is not in

the DELIM list.  DELIM defaults to " \t\n", that is, whitespace.

The return value is a [start,end] offset pair, with trailing

dimensions matching the dimensions of the given STRING_ARRAY.

Note that this return value is suitable for use in the strpart

or streplace functions.

If the first character of DELIM is "^", the sense is reversed;

strword scans to the first character in DELIM.  (Except that

if DELIM is the single character "^", it has its usual meaning.)

Also, a "-" which is not the first (or second after "^") or last

character of DELIM indicates a range of characters.  Finally,

if DELIM is "" or string(0), the scan stops immediately, since

the first character (no matter what it is) is not in DELIM.

FROM strword TO strword

Furthermore, DELIM can be a list of delimiter sets, where each

element of the list delimits a new word, so the return value will

be [start1,end1, ..., startN,endN], where N=numberof(DELIM),

and start1 is the offset of the first character not in DELIM(1),

characters with offset between end1 and start2 are in DELIM(2),

characters with offset between end2 and start3 are in DELIM(3),

and so on.  If endM is the length of the string for some M<N,

then all subsequent start,end pairs will be endM,-1.  Note that

endN is always the length of the string.

If N is specified, the final DELIM will be repeated N times, so

the leading dimension of the result is 2*(numberof(DELIM)+N-1).

Thus, instead of DELIM = [d1,d2,d3,d3,d3,d3,d3], you may specify

DELIM = [d1,d2,d3] and N = 5.  (By default, N=1.)

If OFF is supplied (it is distinguished from DELIM by the

fact that DELIM is a string or [], while OFF is an integer),

it represents the offset (0-origin character index) in

STRING_ARRAY at which the word search is to begin.  (DELIM

or N may be omitted to get their default values.)  The OFF

may be a scalar or an array conformable with STRING_ARRAY.

The returned offsets include OFF; you do not need to add OFF

to the return value to get the offsets into STRING_ARRAY.

The action of strword can be described as follows: If d1 is

the first delimiter, d2 is the second delimiter, and so on,

then the input element of STRING_ARRAY looks like this:

d1 word1 d2 word2 d3 word3 d4 word4 ...

The offset array returned by strword will select word1, word2,

word3, etc., if used in strpart.  One defect of this system is

that the final word has no trailing delimiter - it always

extends to the end of the string.

To remedy this defect, N<0 is interpreted as follows: It is

the same as abs(N), except that the final [start,end] pair is

supressed; that is, if you have specified N delimiters, only

N-1 words will be returned.

As a second special case, N=0 forces the final delimeter to be

at the end of the string, rather than as soon after the previous

endM as possible.  Like N<0, this reduces the number of start,end

pairs by one.  (N=0 is useful, for example, in order to trim

trailing blanks.)

Examples:

strword("  Hello, world!") --> [2,15]

strword("Hello, world!") --> [0,13]

strword("Hello, world!", , 2) --> [0,6,7,13]

strword("Hello, world!", , -2) --> [0,6]

strword("Hello, world!", ".!, \t\n", -2) --> [0,5]

strword("Hello, world!", [string(0), ".!, \t\n"], 0) --> [0,12]

strword("Hello, world!", "A-Za-z", 2) --> [5,7,12,13]

strword("Hello, world!", "^A-Za-z", 2) --> [0,5,7,13]

FROM strword TO symbol_def

strword("Hello, world!", "^A-Za-z", 3) --> [0,5,7,12,13,-1]

strword("  Hello, world!", [" \t\n",".!, \t\n"]) --> [2,7,9,15]

strword("  Hello, world!", [" \t\n",".!, \t\n"], 2) --> [2,7,9,14,15,-1]

SEE ALSO: string, strlen, strpart, strfind, strtok, strtrim

*/

sum

/* DOCUMENT sum(x)

returns the scalar sum of all elements of its array argument.

If X is a string, concatenates all elements.

SEE ALSO: avg, min, max

*/

sun3_primitives

/* DOCUMENT sun3_primitives, file

sets FILE primitive data types to be native to Sun-2 or Sun-3.

*/

sun_primitives

/* DOCUMENT sun_primitives, file

sets FILE primitive data types to be native to Sun, HP, IBM, etc.

*/

suspend

/* DOCUMENT suspend

resume

Stop execution of the current interpreted program with suspend.

It resumes at the instruction following suspend when yorick

becomes idle after another interpreted task has called resume.

Note that the task which calls resume must be triggered by an

input stream other than stdin, such as the on_stdout or on_stderr

function of a spawned process or the on_elapse of an after.

Use control-c to escape from a hung suspend state.

SEE ALSO: spawn, funcdef, after

*/

swrite

/* SEE write     */

symbol_def

/* DOCUMENT symbol_def(func_name)(arglist)

or symbol_def(var_name)

invokes the function FUNC_NAME with the specified ARGLIST,

returning the return value.  ARGLIST may be zero or more arguments.

In fact, symbol_def("fname")(arg1, arg2, arg3) is equivalent to

fname(arg1, arg2, arg3), so that "fname" can be the name of any

variable for which the latter syntax is meaningful -- interpreted

function, built-in function, or array.

Without an argument list, symbol_def("varname") is equivalent to

varname, which allows you to get the value of a variable whose name

you must compute.

DO NOT OVERUSE THIS FUNCTION.  It works around a specific deficiency

FROM symbol_def TO timer

of the Yorick language -- the lack of pointers to functions -- and

should be used for such purposes as hook lists (see openb).

SEE ALSO: symbol_set

*/

symbol_set

/* DOCUMENT symbol_set, var_name, value

is equivalent to the redefinition

varname= value

except that var_name="varname" is a string which must be computed.

DO NOT OVERUSE THIS FUNCTION.  It works around a specific deficiency

of the Yorick language -- the lack of pointers to functions, streams,

bookmarks, and other special non-array data types.

SEE ALSO: symbol_def

*/

system

/* DOCUMENT system, "shell command line"

Passes the command line string to a shell for execution.

If the string is constant, you may use the special syntax:

$shell command line

(A long command line may be continued by ending the line with \

as usual.)  The system function syntax allows Yorick to compute

parts of the command line string, while the simple $ escape

syntax does not.  In either case, the only way to get output

back from such a command is to redirect it to a file, then

read the file.  Note that Yorick does not regain control

until the subordinate shell finishes.  (Yorick will get control

back if the command line backgrounds the job.)

WARNING: If Yorick has grown to a large size, this may crash

your operating system, since the underlying POSIX fork function

first copies all of the running Yorick process before the exec

function can start the shell.  See Y_SITE/sysafe.i for a fix.

SEE ALSO: popen

*/

tan

/* SEE sin     */

tanh

/* SEE sinh     */

timer

/* DOCUMENT timer, elapsed

or timer, elapsed, split

updates the ELAPSED and optionally SPLIT timing arrays.  These

arrays must each be of type array(double,3); the layout is

[cpu, system, wall], with all three times measured in seconds.

ELAPSED is updated to the total times elapsed since this copy

of Yorick started.  SPLIT is incremented by the difference between

the new values of ELAPSED and the values of ELAPSED on entry.

This feature allows for primitive code profiling by keeping

FROM timer TO transpose

separate accounting of time usage in several categories, e.g.--

elapsed= total= cat1= cat2= cat3= array(double, 3);

timer, elapsed0;

elasped= elapsed0;

... category 1 code ...

timer, elapsed, cat1;

... category 2 code ...

timer, elapsed, cat2;

... category 3 code ...

timer, elapsed, cat3;

... more category 2 code ...

timer, elapsed, cat2;

timer, elapsed0, total;

The wall time is not absolutely reliable, owning to possible

rollover at midnight.

SEE ALSO: timestamp, timer_print

*/

timer_print

/* DOCUMENT timer_print, label1, split1, label2, split2, ...

or timer_print

or timer_print, label_total

prints out a timing summary for splits accumulated by timer.

timer_print, "category 1", cat1, "category 2", cat2,

"category 3", cat3, "total", total;

SEE ALSO: timer

*/

timestamp

/* DOCUMENT timestamp()

or timestamp(utime)

or timestamp, utime

returns string of the form "Sun Jan  3 15:14:13 1988" -- always

has 24 characters.  If a simple variable reference UTIME is supplied,

it will be set to the number of seconds since 1970 Jan 1 0000 UT.

SEE ALSO: timer

*/

transpose

/* DOCUMENT transpose(x)

or transpose(x, permutation1, permutation2, ...)

transpose the first and last dimensions of array X.  In the second

form, each PERMUTATION specifies a simple permutation of the

dimensions of X.  These permutations are compounded left to right

to determine the final permutation to be applied to the dimensions

of X.  Each PERMUTATION is either an integer or a 1D array of

integers.  A 1D array specifies a cyclic permutation of the

dimensions as follows: [3, 5, 2] moves the 3rd dimension to the

5th dimension, the 5th dimension to the 2nd dimension, and the 2nd

dimension to the 3rd dimension.  Non-positive numbers count from the

end of the dimension list of X, so that 0 is the final dimension,

-1 in the next to last, etc.  A scalar PERMUTATION is a shorthand

for a cyclic permutation of all of the dimensions of X.  The value

of the scalar is the dimension to which the 1st dimension will move.

FROM transpose TO updateb

Examples:  Let x have dimsof(x) equal [6, 1,2,3,4,5,6] in order

to be able to easily identify a dimension by its length. Then:

dimsof(x)                          == [6, 1,2,3,4,5,6]

dimsof(transpose(x))               == [6, 6,2,3,4,5,1]

dimsof(transpose(x,[1,2]))         == [6, 2,1,3,4,5,6]

dimsof(transpose(x,[1,0]))         == [6, 6,2,3,4,5,1]

dimsof(transpose(x,2))             == [6, 6,1,2,3,4,5]

dimsof(transpose(x,0))             == [6, 2,3,4,5,6,1]

dimsof(transpose(x,3))             == [6, 5,6,1,2,3,4]

dimsof(transpose(x,[4,6,3],[2,5])) == [6, 1,5,6,3,2,4]

*/

typeof

/* DOCUMENT typeof(object)

returns a string describing the type of object.  For the basic

data types, these are "char", "short", "int", "long", "float",

"double", "complex", "string", "pointer", "struct_instance",

"void", "range", "struct_definition", "function", "builtin",

"stream" (for a binary stream), and "text_stream".

SEE ALSO: structof, dimsof, sizeof, numberof, nameof

*/

uncen

/* DOCUMENT uncen(ptcen)

or uncen(ptcen, ireg)

returns zone centered version of the 2-D zone centered array PTCEN.

The result is (imax-1)-by-(jmax-1) if PTCEN is imax-by-jmax.

If the region number array IREG is specified, zones with region

number 0 are not included in the point centering operation.

Note that IREG should have dimensions imax-by-jmax, like

the input PTCEN array; the first row and column of IREG are ignored.

Without IREG, uncen(ptcen) is equivalent to ptcen(uncp,uncp).

Do not use uncen to zone center data which is naturally point

centered -- use the zncen function for that purpose.  The uncen

function is the (nearly) exact inverse of the ptcen function,

so that uncen(ptcen(zncen, ireg), ireg) will return the original

zncen array.  The uncen reconstruction is as exact as possible,

given the finite precision of floating point operations.

SEE ALSO: ptcen, zncen

*/

updateb

/* DOCUMENT file= updateb(filename)

or file= updateb(filename, primitives)

open a binary data file FILENAME for update (mode "r+b").

The optional PRIMITIVES argument is as for the createb function.

If the file exists, it is opened as if by openb(filename),

otherwise a new PDB file is created as if by createb(filename).

SEE ALSO: openb, createb, cd, save, restore, get_vars, get_addrs

close102, close102_default, open102, at_pdb_open, at_pdb_close

*/

FROM use_origins TO where

use_origins

/* DOCUMENT dummy= use_origins(dont_force)

Yorick array dimensions have an origin as well as a length.

By default, this origin is 1 (like FORTRAN arrays, unlike C

arrays).  However, the array function and the pseudo-index (-)

can be used to produce arrays with other origins.

Initially, the origin of an array index is ignored by Yorick; the

first element of any array has index 1.  You can change this

default behavior by calling use_origins with non-zero DONT_FORCE,

and restore the default behavior by calling use_origins(0).

When the returned object DUMMY is destroyed, either by return from

the function in which it is a local variable, or by explicit

redefintion of the last reference to it, the treatment of array

index origins reverts to the behavior prior to the call to

use_origins.  Thus, you can call use_origins at the top of a

function and not worry about restoring the external behavior

before every possible return (including errors).

*** DEPRECATED ***

Do not use index origins.  Your brain will explode sooner or later.

SEE ALSO: array, dimsof, orgsof

*/

vax_primitives

/* DOCUMENT vax_primitives, file

sets FILE primitive data types to be native to VAXen, H-double, only.

*/

vaxg_primitives

/* DOCUMENT vaxg_primitives, file

sets FILE primitive data types to be native to VAXen, G-double, only.

*/

volume

/* DOCUMENT volume(r, z)

returns the zonal volumes of the 2-D cylindrical mesh (R, Z).

If R and Z are imax-by-jmax, the result is (imax-1)-by-(jmax-1).

The volume is positive when, say, Z increases with i and R increases

with j.  For example, volume([[0,0],[1,1]],[[0,1],[0,1]]) is +pi.

SEE ALSO: area

*/

warranty

/* SEE copyright     */

where

/* DOCUMENT where(x)

returns the vector of longs which is the index list of non-zero

values in the array x.  Thus, where([[0,1,3],[2,0,4]]) would

return [2,3,4,6].  If noneof(x), where(x) is a special range

function which will return a nil value if used to index an array;

hence, if noneof(x), then x(where(x)) is nil.

FROM where TO write

If x is a non-zero scalar, then where(x) returns a scalar value.

The rather recondite behavior for scalars and noneof(x) provides

maximum performance when the merge function to be used with the

where function.

SEE ALSO: where2, merge, merge2 allof, anyof, noneof, nallof, sort

*/

where2

/* DOCUMENT where2(x)

like where(x), but the returned list is decomposed into indices

according to the dimensions of x.  The returned list is always

2 dimensional, with the second dimension the same as the dimension

of where(x).  The first dimension has length corresponding to the

number of dimensions of x.  Thus, where2([[0,1,3],[2,0,4]]) would

return [[2,1],[3,1],[1,2],[3,2]].

If noneof(x), where2 returns [] (i.e.- nil).

SEE ALSO: where, merge, merge2, allof, anyof, noneof, nallof, sort

*/

write

/* DOCUMENT n= write(f, format=fstring, linesize=l, obj1, obj2, ...)

n= write(format=fstring, linesize=l, obj1, obj2, ...)

or strings= swrite(format=fstring, linesize=l, obj1, obj2, ...)

writes text to I/O stream F (1st form), or to the terminal (2nd

form), or to the STRINGS string array (3rd form), representing

arrays OBJ1, OBJ2, ..., according to the optional FSTRING.  The

optional linesize L defaults to 80 characters, and helps restrict

line lengths when FSTRING is not given, or does not contain

newline directives.  The write function always appends to the

end of a text file; the position for a sequence of reads is

not affected by intervening writes.

There must be one conversion specifier (see below) in FSTRING for

each OBJ to be written; the type of the conversion specifier must

generally match the type of the OBJ.  That is, an integer OBJ

requires an integer specifier (d, i, o, u, x, or c) in FSTRING,

a real OBJ requires a real specifier (e, f, or g), a string OBJ

requires the string specifier (s), and a pointer OBJ requires a

the pointer specifier (p).  An OBJ may not be complex, a structure

instance, or any non-array Yorick object.  If FSTRING is not

supplied, or if it has fewer conversion specifiers than the

number of OBJ arguments, then Yorick supplies default specifiers

(" %8ld" for integers, " %14.6lg" for reals, " %s" for strings, and

" %8p" for pointers).  If FSTRING contains more specifiers than

there are OBJ arguments, the part of FSTRING beginning with the

first specifier with no OBJ is ignored.

The OBJ may be scalar or arrays, but the dimensions of the OBJ

must be conformable.  If the OBJ are arrays, Yorick behaves as

if he write were called in a loop dimsof(OBJ1, OBJ2, ...) times,

writing one array element of each of the OBJ according to FSTRING

on each pass through the loop.  The swrite function returns a

string array with dimensions dimsof(OBJ1, OBJ2, ...).  The write

function inserts a newline between passes through the array if

the line produced by the previous pass did not end with a

FROM write TO write

newline, and if the total number of characters output since the

previous inserted newline, plus the number of characters about

to be written on the current pass, would exceed L characters

(L defaults to 80).  The write function returns the total

number of characters output.

The FSTRING is composed of a series of "directives" which are

(1) characters other than % -- copied directly to output

(2) conversion specifiers beginning with % and ending with a

character specifying the type of conversion -- specify

how to convert an OBJ into characters for output

The conversion specifier is of the form %FW.PSC, where:

F is zero or more optional flags:

- left justify in field width

+ signed conversion will begin with either + or -

(space) signed conversion  will begin with either space or -

# alternate form (see description of each type below)

0 pad field width with leading 0s instead of leading spaces

W is either a decimal integer specifying the minimum field width

(padded as specified by flags), or not present to use the

minimum number of characters required.

.P is either a decimal integer specifying the precision of the

result, or not present to get the default.  For integers, this

is the number of digits to be printed (possibly forcing leading

zeroes), and defaults to 1.  For reals, this is the number of

digits after the decimal point, and defaults to 6.  For strings,

this is the maximum number of characters to print, and defaults

to infinity.

S is either one of the characters 'h', 'l', or 'L', or not

present.  Yorick allows this for compatibility with the C

library functions, but ignores it.

C is a character specifying the type of conversion:

d, i  - decimal integer

o     - octal integer (# forces leading 0)

u     - unsigned decimal integer (same as d for Yorick)

x, X            - hex integer (# forces leading 0x)

f     - floating point real in fixed point notation

(# forces decimal)

e, E  - floating point real in scientific notation

g, G  - floating point real in fixed or scientific notation

depending on the value converted (# forces decimal)

s   - string of ASCII characters

c   - integer printed as corresponding ASCII character

p   - pointer

%   - the ordinary % character; complete conversion

specification must be "%%"

The write function is modeled on the ANSI standard C library

fprintf and sprintf functions, but differs in several respects:

FROM write TO zncen

(1) Yorick's write cannot handle the %n conversion specifier

in FSTRING.

(2) Yorick's write may insert additional newlines if the OBJ

are arrays, to avoid extremely long output lines.

SEE ALSO: print, exit, error, read, rdline, open, close, save, restore

*/

xdr_primitives

/* DOCUMENT xdr_primitives, file

sets FILE primitive data types to be XDR (external data representation).

*/

yorick_stats

/* DOCUMENT yorick_stats

returns an array of longs describing Yorick memory usage.

For debugging.  See ydata.c source code.

*/

zncen

/* DOCUMENT zncen(ptcen)

or zncen(ptcen, ireg)

returns zone centered version of the 2-D point centered array PTCEN.

The result is (imax-1)-by-(jmax-1) if PTCEN is imax-by-jmax.

If the region number array IREG is specified, zones with region

number 0 are not included in the point centering operation.

Note that IREG should have dimensions imax-by-jmax, like

the input PTCEN array; the first row and column of IREG are ignored.

Without IREG, zncen(ptcen) is equivalent to ptcen(zcen,zcen).

SEE ALSO: ptcen, uncen

*/