AudioDriveÒ   Development    Kit

Version 3.03

-ESS CONFIDENTIAL INFORMATION -

ESS Technology, Inc.

March, 1995

Contents

1.   Overview 	  3

2.   Windows Driver Customization 	  4

3.   Windows Setup Customization 	  8

4.   GPO Pin Programming	 11

5.   Windows Libraries for AudioDrive Application Software Developers 	 13

6.   AudioDrive DOS Utilities 	 25

7.   Localization Kit 	 26

8.   Programming ESS AudioDrive 	 27

9.   Software Test Plan for Audio Applications 	 32

1.   Overview

AudioDrive Development Kit 3.03 contains information intended for AudioDrive OEM developers as well as Windows application developers.

Section 2 "Windows Driver Customization" provides information for OEMs who need to customize Windows Drivers included in the Audio Applications 3.03.

Section 3 "Windows Setup Customization" explains how to customize the Windows installation program in the Audio Applications 3.03.

Section 4 "Windows Libraries for AudioDrive Application Software Developer" is prepared for Windows application developers who want to integrate the AudioDrive-based products into their applications.

Section 5 "AudioDrive DOS Utilities"  describes how to use a few DOS utility programs.

Section 6 "Localization Kit" presents information needed to localize Audio Applications for foreign language support.

Section 7 "Programming ESS AudioDrive" has a source listing which illustrates how to program the ESS AudioDrive family of devices.

Section 8 "Software Test Plan for Audio Applications 3.03" presents a check list in testing the Audio Applications 3.03 on AudioDrive OEM hardware.

2.   Windows Driver Customization

This section is intended for the AudioDrive OEM developers whose product designs are different from the ESS reference designs (i.e. Audio Magician, Audio Magician Plus, Stereo Magician, and Stereo Magician Plus).

The OEM customers can edit the CONFIG.INI file to customize driver and Setup without any modification on the source code.  The customizable features through the CONFIG.INI file are as following:

·	Choices for I/O Port Address

·	Choices for IRQ

·	Choices for DMA Channel

·	Factory Default for I/O Port Address

·	Factory Default for IRQ

·	Factory Default for DMA Channel

·	Choices for MPU401 I/O Port Address  [ES1688 only]

·	Choices for MPU401 DMA Channel  [ES1688 only]

·	Choices for MPU401 IRQ  [ES1688 only]

1.  The CONFIG.INI file must be located in the directory "\ESSSET" of the first diskette.

2.  CONFIG.INI is a text file and can be edited using Windows Notepad or any text editor.

3.  Edit entries in the [es488] , [es688], or [es1688] sections. The following examples illustrate how to change this section:

Example 1.     	ES488-based design.

Port Choices		220 and 240 (factory default: 220)

IRQ Choices		2, 3, 5 and 7 (factory default: 7)

DMA Choices		1 and 3 (factory default: 1)

Edit the [es488] section as following:

[es488]

portchoices=220,240

irqchoices=2,3,5,7

dmachoices=1,3

defaultport=220

defaultirq=7

defaultdma=1

Example 2.      ES688-based design.

Port Choices		220, 230, 240 and 250 (factory default: 220)

IRQ Choices		2, 5, 7 and 10 (factory default: 7)

DMA Choices		0, 1 and 3 (factory default: 1)

Edit the [es688] section as following:

[es688]

portchoices=220,230,240,250

irqchoices=2,5,7,10

dmachoices=0,1,3

defaultport=220

defaultirq=7

defaultdma=1

Example 3.      ES688-based design.

Same as in Example 2, but IRQ2 is skipped.

Edit the [es688] section as following:

[es688]

portchoices=220,230,240,250

irqchoices=,5,7,10

dmachoices=0,1,3

defaultport=220

defaultirq=7

defaultdma=1

4.  If the design allow only one setting for port address, IRQ and DMA channel, add the [custom] section as following:

[custom]

entryCount=3

entry1=AutoConfig

value1=no

entry2=VerifyInt

value2=no

entry3=VerifyDMA

value3=no

AutoConfig={yes, no}

Specifies if the Windows driver should attempt to configure the settings of IRQ and DMA channel automatically when Windows boots.

VerifyInt={yes, no}

Specifies if the Windows driver should check the current IRQ setting when Windows boots.

VerifyDMA={yes, no}

Specifies if the Windows driver should check the current DMA channel setting when Windows boots.

5.	Driver File Description

All driver-related files are located in the \ESSSET directory of Disk 1.  The destination is the Windows System directory.

| File Name in Source | File Name in Destination | Description |
| --- | --- | --- |
| es1688wn.drv | es1688wn.drv | Windows standard driver (waveaudio, synthesizer, mixer, aux) for ES1688 |
| es1688wn.38_ | es1688wn.386 | Windows virtual driver for ES1688 |
| es688win.drv | es688win.drv | Windows standard driver (waveaudio, synthesizer, mixer, aux) for ES688 |
| es688win.38_ | es688win.386 | Windows virtual driver for ES688 |
| es1488wn.drv | es1488wn.drv | Windows standard driver for ES1488 |
| es1488wn.38_ | es1488wn.386 | Windows virtual driver for ES1488 |
| es488win.drv | es488win.drv | Windows standard driver for ES488 |
| es488win.38_ | es488win.386 | Windows virtual driver for ES488 |
| audmpio.dr_ | audmpio.drv | External MIDI Port driver |
| auddrive.hl_ | auddrive.hlp | on-line help file for the Help button in the standard driver dialog box |
| midimap.cf_ | midimap.cfg | MIDI Mapper configuration file for OPL2 |
| midimap3.cf_ | midimap.cfg | MIDI Mapper configuration file for OPL3 |
| essset.hi_ | essset.him | Hardware Installation Module as defined in Windows Sound System 2.0 DDK |
| config.ini |  | Configuration information file |

3.   Windows Setup Customization

The purpose of this section is to provide information for OEMs who design the own Setup program rather than using the ESS Setup program included in the Audio Applications 3.03.  This information is valid for Windows 3.1, Windows for Workgroup 3.1, and Windows for Workgroup 3.11.  It may NOT be valid for Windows NT 3.1 or any future version of Windows.

The ESS Setup for Audio Applications 3.03 has been developed in C using Microsoft Visual C++.  Setup for Audio Applications consists of two parts: the driver installation and the application software installation.

3.1   Driver Installation

Examples in this section are for ES688. If your hardware design is based on ES488 instead of ES688, "es688win.drv" should be replaced by "es488win.drv" and "es688win.386" should be replaced by "es488win.386". If your hardware design is based on ES1488 instead of ES688, "es688win.drv" should be replaced by "es1488wn.drv" and "es688win.386" should be replaced by "es1488wn.386".  If your hardware design is based on ES1688 instead of ES688, "es688win.drv" should be replaced by "es1688wn.drv" and "es688win.386" should be replaced by "es1688wn.386".

a.  Add the 4 entries below to the [drivers] section in the SYSTEM.INI file:

[drivers]

WAVE=es688win.drv

AUX=es688win.drv

MIXER=es688win,drv

MIDI=es688win.drv

MIDI1=audmpio.drv

If your hardware design does not include any music synthesizer, you may omit the entry "MIDI=es688win.drv".

b. Add the entry "device=es688win.386" to the [386Enh] section in the SYSTEM.INI file.  If there are more than  one entries for "device=es688win.386" in the same section, Windows will fail to boot properly.  Do NOT use the WritePrivateProfileString function to add this entry, or else, the first entry starting with "device=" will be overwritten and one virtual driver will be omitted at the boot time.

c.  If the [auddrive.drv] section exists in the SYSTEM.INI file, delete all entries and add the following 8 entries.  If the section does not exist, create one.

[auddrive.drv]

AudioDrive=ES688

Synth=OPL3

portchoices=220,230,240,250

irqchoices=2,5,7,10

dmachoices=0,1,3

port=220

int=7

dmachannel=1

·	The above entries are for Port Address 220, IRQ 7, and DMA 1.

·	If ES488 is used in your design, the first entry should be "AudioDrive=ES488".

·	If OPL2 is used in your design, the second entry should be "Synth=OPL2".

·	If no music synthesizer is used in your design, the second entry should be "Synth=None".

·	If your design support a different set of I/O Ports, IRQs, and DMA channels,  you should modify the third entry through the fifth entry accordingly.  They must be in the ascending order.

d.  Add "msmixmgr.dll" to the entry "drivers=mmsystem.dll" in the [boot] section of the SYSTEM,INI file.  (e.g. drivers=mmsystem.dll msmixmgr.dll)

e.  Copy the following files to the Windows System directory:

es688win.drv

es688win.386

audmpio.drv

midimap.cfg

msmixmgr.dll

·	Do not overwrite if any file of newer version already exists in the directory.

·	If OPL3 is used in your design, expand midimap3.cf_ to midimap.cfg.  If OPL2 is used in your design, expand midimap.cf_ to midimap.cfg.

3.2   Application Software Installation

·	Refer to the "esssoft.ini" file in Disk1 for the information regarding the file installation.

The subdirectory "C:\PCAudio\essset" is used by ESS Setup and you do not need to create it if you are designing your own Setup.  In such case, exclude the following files from the directory "C:\PCAudio" because these belong to ESS Setup:

setup.exe

esssetup.exe

esssoft.ini

·	The list of files which need to be copied to the Windows System directory are as follows:

volapi.dll

recapi.dll

esstb.dll

essctl.dll

ctl3d.dll

NOTE:	For the files listed above, a special consideration needs to be in place. When the Setup finds a file with the same name in the Windows System directory and the file is newer than the file from the source, it should offer user an option to overwrite it or not.

4.   GPO Pin Programming

GPO0 and GPO1 pins in ES688 are general-purpose output pins which any OEM designs can use to control external circuits.  Within Windows, no software other than the ES688 driver should access the ES688 registers directly.  Threrefore, the ES688 driver has a custom message through the auxOutMessage() function call to get or set the status of these pins.  The auxOutMessage() function is part of  Windows Multimedia standard API. The following example illustrates how to use this custom message:

#define AUXDM_GPO_STATE_INFO    0x4687

#define GPO_STATE_INFO_READ     -1

#define GPO_STATE_INFO_WRITE    0

DWORD FAR PASCAL GPO_API_demo()

{

UINT  wDeviceID ;

UINT  msg ;

DWORD dw1 ;

DWORD dw2 ;

DWORD dwRet ;

//

// read current GPO settings...

//

wDeviceID = 0 ;                // _assume_ first AUX device

msg = AUXDM_GPO_STATE_INFO ;   // our custom message

dw1 = GPO_STATE_INFO_READ ;    // read GPO settings

dwRet = auxOutMessage( wDeviceID, msg, dw1, (DWORD FAR *)&dw2 );

if (dwRet == MMSYSERR_NOERROR)

{

// bit 0 of dw2 contains current GPO0 state

// bit 1 of dw2 contains current GPO1 state

}

else

{

return dwRet ;

}

//

// write back current GPO settings...

//

wDeviceID = 0 ;                // _assume_ first AUX device

msg = AUXDM_GPO_STATE_INFO ;   // our custom message

dw1 = GPO_STATE_INFO_WRITE ;   // write GPO settings

dwRet = auxOutMessage( wDeviceID, msg, dw1, dw2 ) ;

if (dwRet == MMSYSERR_NOERROR)

{

// GPO information has been successfully written...

}

else

{

// do error handling...

}

return dwRet ;

}

5.   Windows Libraries for AudioDrive Application Software Developers

One of the objectives of AudioDrive Development Kit (ADK) is to help Windows application software developers integrate audio features into their applications in a relatively short time-frame and with ease of programming.  ADK includes two DLLs (Dynamic Link Library), VOLAPI.DLL and RECAPI.DLL.

Windows Mixer API (Application Programming Interface) is a new extension to the existing Windows 3.1 Multimedia API, contributed by Microsoft Windows Sound System 2.0, and will be included in the Windows 4.0 Multimedia API.  It is complicated due to the abstraction for scalability, and it takes quite significant  amount of time  for the experienced Windows programmers to learn.  The use of these libraries will shorten the application design time and improve the code maintainability by simplifying of application coding.

The following subsections describe the programming reference to these libraries.

5.1   Volume Control DLL

Version 3.03

File Name	VOLAPI.DLL

Description

This DLL includes a simple set of functions to get the number of volume controls, get volume level, set volume level, get mute status, set mute, etc.  These functions can be called from C or C++ or Visual Basic applications.

Data structure and exported functions defined in this DLL are as follows:

VOLUMEINFO

vcGetSourceCount

vcGetVolumeInfo

vcSetVolume

vcSetMute

vcGetWaveStrPeakMeter

vcGetWavePeakMeter

VOLUMEINFO

The VOLUMEINFO structure describes the information associated with an individual volume control.  Typically there is one master volume control at the output of a mixer and one volume control at each of the inputs of the mixer.

Syntax

typedef struct _VOLUMEINFO

{

WORD  wChannels;

WORD  wVolume;

WORD  wBalance;

WORD  wMute;

char  szName[16];

WORD  wNameLength;

} VOLUMEINFO;

typedef VOLUMEINFO FAR *LPVOLUMEINFO;

Members

wChannels

Specifies the number of channels.  The value is 1 if mono, and 2 if stereo.  This value is set by the device driver.

wVolume

Specifies the volume level, ranging from 0 to 255 (Max: 255).  Initially this value is set by the device driver, and then, can be changed by the calling program.

wBalance

Specifies the balance between the left and right volumes, ranging from 0 to 255 (Left Maximum: 0, Right Maximum: 255).  Initially this value is set by the device driver, and then, can be changed by the calling program.

wMute

Specifies the state of mute.  The value is TRUE if muted, FALSE if not muted.  Initially this value is set by the device driver, and then, can be changed by the calling program.

szName

Specifies the name of the volume control as a NULL-terminated string (Maximum 15 characters).  This string is set by the device driver and can be used as a label in GUI.

wNameLength

Specifies the number of characters in szName. This string is set by the device driver. This field may be useful if the calling program is written in Visual Basic.  This field can be ignored if the calling program is written in C.

vcGetSourceCount

The vcGetSourceCount function returns the number of volume controls at the input of the mixer.

short vcGetSourceCount()

Parameters

None

Return Value

Returns the number of volume controls at the input of the mixer, if successful.  This number does not include the master volume control at the output of the mixer.  The return value is zero if unsuccessful.

vcGetVolumeInfo

The vcGetVolumeInfo function provides information associated with the specified volume control.

short vcGetVolumeInfo(short wVolumeID, LPVOLUMEINFO lpVolumeInfo)

Parameters

wVolumeID

Specifies the ID of the volume control.  The value is zero for the master volume control, one for the first volume control at the input of the mixer.  The ID of the last volume control at the input of the mixer is equal to the return value of the vcGetSourceCount function. (e.g. for ES688, 0=Vol, 1=Line, 2=Wave, 3=Mic, 4=CD, 5=Synth)

lpVolumeInfo

Points to a VOLUMEINFO structure.  The contents of the VOLUMEINFO structure is filled when the vcGetVolumeInfo function returns.

Return Value

Returns TRUE if the function is successful.  Otherwise, it returns FALSE.

vcSetVolume

The vcSetVolume function sets the volume level and balance of the specified volume control.

short vcSetVolume(short wVolumeID, short nVolume, short nBalance)

Parameters

wVolumeID

Specifies the ID of the volume control.  The value is zero for the master volume control, one for the first volume control at the input of the mixer.  The ID of the last volume control at the input of the mixer is equal to the return value of the vcGetSourceCount function.

nVolume

Specifies the volume level, ranging from 0 to 255 (Maximum: 255).

nBalance

Specifies the balance, ranging from 0 to 255 (Left Maximum: 0,  Right Maximum: 255).

Return Value

Returns zero if the function is successful.  Otherwise, it returns a non-zero number.

vcSetMute

The vcSetMute function sets the state of the mute control.

short vcSetMue(short wVolumeID, short nMute)

Parameters

wVolumeID

Specifies the ID of the volume control.  The value is zero for the master volume control, one for the first volume control at the input of the mixer.  The ID of the last volume control at the input of the mixer is equal to the return value of the vcGetSourceCount function.

nMute

Specifies the state of the mute control.  The value is TRUE if muted. Otherwise, it is FALSE.

Return Value

Returns zero if the function is successful.  Otherwise, it returns a non-zero number.

vcGetWaveStrPeakMeter

The vcGetWaveStrPeakMeter function provides the current values of the WaveOut peak meter.

short vcGetWaveStrPeakMeter(short FAR* lpPeak)

Parameters

lpPeak

Specifies a pointer to an array of two short integers.  Upon return, vcGetWavePeakMeter loads the value of  the left channel peak meter on the first short integer, and the value of the right channel peak meter on the second short integer, ranging from 0 to 256.  the maximum peak value is 256. If the WaveOut device is monaural, the two short integers will have same value.

Return Value

Returns TRUE if the function is successful. Otherwise it returns FALSE.

vcGetWavePeakMeter

The vcGetWavePeakMeter function provides the current values of the WaveOut peak meter.

short vcGetWavePeakMeter(void)

Parameters

None.

Return Value

Returns the average of the left and right channel peak meter, ranging from 0 to 256.

5.2   Recording Control DLL

Version 3.0

File Name 	RECAPI.DLL

Description

This DLL includes a simple set of functions to initialize recording control device, get recording sources, set recording source, get record level, set record level. get peak meter level, get record monitor status, set record monitor, etc. These functions can be called from C or C++ or Visual Basic applications.

If the audio hardware is based on ES688, the recording control is a mixer type.  In other words, you can select more than one recording sources, and record the mixed audio.  The recording control for ES688 has a master record level and balance control as well as individual record level and balance controls for recording sources.

If the audio hardware is based on ES488, the recording control is a multiplexer type.  There are two recording sources in ES488, i.e. microphone and line-in.  You can select one or other, not both.

Data structure and exported functions defined in this DLL are as follows:

RECORDINFO

rcInit

rcetRecordInfo

rcSetVolume

rcSetSource

rcGetPeakMeter

rcGetStrPeakMeter

rcGetRecordMonitor

rcSetRecordMonitor

RECORDINFO

The RECORDINFO structure describes the information associated with an individual recording  control.

Syntax

typedef struct _RECORDINFO

{

WORD  wChannels;

WORD  wVolume;

WORD  wBalance;

WORD  wEnabled;

char  szName[16];

WORD  wNameLength;

} RECORDINFO;

typedef RECORDINFO FAR *LPRECORDINFO;

Members

wChannels

Specifies the number of channels.  The value is 1 if mono, and 2 if stereo.  This value is set by the device driver.

wVolume

Specifies the record level, ranging from 0 to 255 (Max: 255).  Initially this value is set by the device driver, and then, can be changed by the calling program.

wBalance

Specifies the balance between the left and right record levels, ranging from 0 to 255 (Left Maximum: 0, Right Maximum: 255).  Initially this value is set by the device driver, and then, can be changed by the calling program.

wEnabled

Specifies if the specified recording source is enabled.  The value is TRUE if enabled, FALSE if disabled.  Initially this value is set by the device driver, and then, can be changed by the calling program.

szName

Specifies the name of the individual recording control as a NULL-terminated string (Maximum 15 characters).  This string is set by the device driver and can be used as a label in GUI.

wNameLength

Specifies the number of characters in szName. This string is set by the device driver. This field may be useful if the calling program is written in Visual Basic.  This field can be ignored if the calling program is written in C.

rcInit

The rcInit function initializes the Recording Control DLL and returns the number of recording sources.  This function must be called first before any other function is to be called.

short rcInit(UINT uDeviceID, BOOL bLowPri)

Parameters

uDeviceID

Specifies the ID of the Mixer Device.  The value is 0 if there is only one mixer device in the system.

bLowPri

Should be set to FALSE.  It is reserved for future use.

Return Value

Returns the number of recording sources, if successful.  Otherwise, the return value is zero.

rcGetRecordInfo

The rcGetRecordInfo function provides information associated with the specified record control.

short rcGetRecordInfo(short wVolumeID, LPRECORDINFO lpRecordInfo)

Parameters

wVolumeID

Specifies the ID of the record control.  The value is zero for the master record control, and one for the first recording source.  The ID of the record control for the last recording source is equal to the return value of the rcInit function. (e.g. for ES688, 0=Master, 1=Line, 2=Mic, 3=CD, 4=Synth)

lpRecordInfo

Points to a RECORDINFO structure.  The contents of the RECORDINFO structure is filled when the rcGetRecordInfo function returns.

Return Value

Returns TRUE if the function is successful.  Otherwise, it returns FALSE.

rcSetVolume

The rcSetVolume function sets the record level and balance of the specified record control.

short rcSetVolume(short wVolumeID, short nVolume, short nBalance)

Parameters

wVolumeID

Specifies the ID of the record control.  The value is zero for the master record control, and one for the first recording source.  The ID of the record control for the last recording source is equal to the return value of the rcInit function.

nVolume

Specifies the record level, ranging from 0 to 255 (Maximum: 255).

nBalance

Specifies the balance, ranging from 0 to 255 (Left Maximum: 0,  Right Maximum: 255).

Return Value

Returns TRUE if the function is successful.  Otherwise, it returns FALSE.

rcSetSource

The rcSetSource function enables or disables the specified recording source.

short rcSetSource(short wVolumeID, short wEnabled)

Parameters

wVolumeID

Specifies the ID of the recording source.  The value is one for the first recording source.  The ID of the record control for the last recording source is equal to the return value of the rcInit function.

nEnabled

Specifies if the source is to be enabled.  The value is TRUE to enable. Otherwise, it is FALSE.

Return Value

Returns TRUE if the function is successful.  Otherwise, it returns FALSE.

rcGetStrPeakMeter

The rcGetStrPeakMeter function provides the peak meter values of recording when this function is called.

short rcGetStrPeakMeter(short FAR* lpPeak)

Parameters

lpPeak

Specifies a pointer to an array of two short integers.  Upon return, rcGetPeakMeter loads the value of  the left channel peak meter on the first short integer, and the value of the right channel peak meter on the second short integer, ranging from 0 to 256.  the maximum peak value is 256. If the WaveIn device is monaural, two short integers will have same value.

Return Value

Returns TRUE if the function is successful. Otherwise it returns FALSE.

rcGetPeakMeter

The rcGetPeakMeter function provides the peak meter value of recording when this function is called.

short rcGetPeakMeter(void)

Parameters

None.

Return Value

Returns the average of the left and right channel peak meter, ranging from 0 to 256.

rcGetRecordMonitor

The rcGetRecordMonitor function gets the state of the Record Monitor control.  If the Record Monitor control is enabled, you hear what you are recording.

short rcGetRecordMonitor(LPMIXERCONTROLDETAILS_BOOLEAN lpRecordMonitor)

Parameters

lpRecordMonitor

Points to the MIXERCONTROLDETAILS_BOOLEAN structure.  The MIXERCONTROLDETAILS_BOOLEAN structure is defined in msmixmgr.h as follows:

typedef struct {

LONG fValue;

} MIXERCONTROLDETAILS_BOOLEAN;

Return Value

Returns TRUE if the function is successful.  Otherwise, it returns FALSE.

rcSetRecordMonitor

The rcSetRecordMonitor function sets the state of the Record Monitor control.  If the Record Monitor control is enabled, you hear what you are recording.

short rcSetRecordMonitor(short RecordMonitor)

Parameters

RecordMonitor

Specifies if the Record Monitor is to be enabled.  The value is TRUE if enabled.  Otherwise, the value is FALSE.

Return Value

Returns TRUE if the function is successful.  Otherwise, it returns FALSE.

6.    AudioDrive DOS Utilities

The AudioDrive DOS Utilities included in ADK are ESSCFG.EXE and ESSVOL.EXE.

ESSCFG.EXE

Sets values for IRQ and DMA channel.  For the ES1688, also sets the MPU401 IRQ, MPU401 I/O base address, and enables/disables the joystick.  Type "ESSCFG/?" at the DOS prompt to get the command format.  CONFIG.INI and ESSCFG.EXE files must be copied into the same directory.  ESSCFG.EXE supports ES688 and ES1688 only.  Source files are located in \dosutil\esscfg\src directory.

ESSVOL.EXE

Sets volume levels for Master, Wave, Line, Microphone, Synthesizer, CD, and AuxB [ES1688 only.]  Type "ESSVOL/?" at DOS prompt to get the command line format.  ESSVOL.EXE supports ES688, ES1688, and ES1488.  Source files are located in \dosutil\essvol\src directory.

7.    Localization Kit

Localization Kit is prepared for AudioDrive developers who want to implement foreign-language supports in Audio Applications.

Localization Kit is located in the LOCALIZE directory of the ADK diskette.  The zip files must be unzipped using the -d option to restore the directory tree.  For example, to unzip the audiorec.zip file, type "pkunzip -d audiorec.zip"

Description of files:

File            	Description

audclips.zip    	Audio Clip Library

audiorec.zip    	Audio Recorder

audsetup.zip    	Audio Clip Library Setup

chime.zip       	Chime

clock.zip       	Talking Clock

csetup.zip      	ESS Setup (esssetup.exe)

esspm.zip       	ESS Power Management

mixer.zip       	Mixer

reccntrl.zip    	Recording Control

reminder.zip    	Reminder

setup.zip		Setup Loader (setup.exe)

stpwatch.zip    	Stopwatch

tcalc.zip       	Talking Calculator

timer.zip       	Timer

volcntrl.zip    	Volume Control

xrecord.zip     	Extended Recorder

essctl.zip      	essctl.dll

esshim.zip      	essset.him

essdrv.zip      	audmpio.drv   (audmpio subdirectory)

audmpu.drv   (audmpu\mpu subdirectory)

es488win.drv  (auddrive\ES488 subdirectory)

es1488wn.drv  (auddrive\ES1488 subdirectory)

es688win.drv  (auddrive\ES688 subdirectory)

es1688wn.drv  (auddrive\ES1688 subdirectory)

es488win.386  (vaudrv\ES488 subdirectory)

es1488wn.386  (vaudrv\ES1488 subdirectory)

es688win.386  (vaudrv\ES688 subdirectory)

es1688wn.386  (vaudrv\ES1688 subdirectory)

NOTE: drivers can be built using the "go.bat" in each subdir.

Some applications must be linked to .LIB files in the LIB directory when you build them.  If you modify any DLL which corresponds to the .LIB file in the LIB directory, you might have to copy the new .LIB file into the LIB directory before you link it to applications.

All modules have been compiled in Microsoft Visual C++ 1.5.  Before you build drivers, you must install Windows 3.1 DDK.

8.   Programming ESS AudioDrive

Sample source code listed below illustrates how to reset, issue commands to, and read data from the ESS AudioDrive family of devices.  The iolib.asm file is located in \samples\iolib directory.

;---------------------------------------------------------------------------

;	Copyright (C) 1994, ESS Technology, Inc.

;---------------------------------------------------------------------------

;	FILENAME: iolib.asm

;---------------------------------------------------------------------------

;	DESCRIPTION:

;	This file contains example low level library routines to reset,

;	issue commands, and read data from the AudioDrive family of devices.

;---------------------------------------------------------------------------

.8086

;--------------------------------------------------------------------

;	I/O port equates

;--------------------------------------------------------------------

AUD_PORT_BASE	EQU	00220h			; base port

AUD_PORT_RESET	EQU	AUD_PORT_BASE+6	; reset port

AUD_MASK_RESET	EQU	001h			; reset mask

AUD_DATA_RESET	EQU	0aah			; reset status byte

AUD_PORT_RDRDY	EQU	AUD_PORT_BASE+14	; data read ready port

AUD_MASK_RDRDY	EQU	080h			; data read ready mask

AUD_PORT_RDDAT	EQU	AUD_PORT_BASE+10	; data read port

AUD_PORT_WRRDY	EQU	AUD_PORT_BASE+12	; data write ready port

AUD_MASK_WRRDY	EQU	080h			; data write ready mask

AUD_PORT_WRDAT	EQU	AUD_PORT_BASE+12	; data write port

;--------------------------------------------------------------------

;	code segment

;--------------------------------------------------------------------

cod	SEGMENT

ASSUME	cs:cod

;--------------------------------------------------------------------

;	Reset the AudioDrive device

;

;	input:	none

;	output:	CF set if failed, clear otherwise

;	modify:	flags

;--------------------------------------------------------------------

io_reset proc near

push	ax

push	cx

push	dx

;

; do the reset

;

mov	al, AUD_MASK_RESET		; reset flag

mov	dx, AUD_PORT_RESET		; reset port

out	dx, al				; do the reset

call	delay_1ms			; 3 us needed, but 1 ms safer

xor	al, al				; clear reset flag

out	dx, al				; clear the reset

;

; read the reset status byte

;

mov	cx, 10				; 1 ms needed, but 10 ms safer

irt_loop:

call	io_read				; read reset status byte

cmp	al, AUD_DATA_RESET		; Q: reset successful?

je	irt_exit				;   Y: jump (CF cleared)

call	delay_1ms			; delay 1 ms

loop	irt_loop

stc					; failed

irt_exit:

pop	dx

pop	cx

pop	ax

ret

io_reset endp

;--------------------------------------------------------------------

;	Write data byte to the AudioDrive device

;

;	input:	al = data byte to write

;	output:	CF set if failed, clear otherwise

;	modify: flags

;--------------------------------------------------------------------

io_write proc near

push	ax

push	cx

push	dx

mov	ah, al				; save data to write

;

; wait for device to be ready to accept data

;

mov	cx, 50				; 5 ms needed, but 50 ms safer

mov	dx, AUD_PORT_WRRDY	; write ready port

iw_loop:

in	al, dx				; read write ready status

test	al, AUD_MASK_WRRDY	; Q: ready for data?

jz	iw_ready			;   Y: jump (CF cleared)

call	delay_1ms			; delay 1 ms

loop	iw_loop

stc					; failed

jmp	short iw_exit

iw_ready:

;

; device is ready, write data to it

;

.errnz	AUD_PORT_WRDAT - AUD_PORT_WRRDY

mov	al, ah

out	dx, al				; write data (CF cleared above)

iw_exit:

pop	dx

pop	cx

pop	ax

ret

io_write endp

;--------------------------------------------------------------------

;	Read data byte from the AudioDrive device

;

;	input:	none

;	output:	CF set if failed, clear otherwise

;		al = data byte read (only if CF clear)

;	modify: flags, al

;--------------------------------------------------------------------

io_read proc near

push	cx

push	dx

;

; wait for data to be available

;

mov	cx, 50				; 5 ms needed, but 50 ms safer

mov	dx, AUD_PORT_RDRDY		; data available port

ir_loop:

in	al, dx				; read data available bit

test	al, AUD_MASK_RDRDY		; Q: data available?

jnz	ir_ready				;   Y: jump (CF cleared)

call	delay_1ms			; delay 1 ms

loop	ir_loop

stc

jmp	short ir_exit

ir_ready:

;

; data is ready, read it from data port

;

mov	dx, AUD_PORT_RDDAT		; data port

in	al, dx				; read data (CF cleared above)

ir_exit:

pop	dx

pop	cx

ret

io_read endp

;--------------------------------------------------------------------

;	Delay 1 millisecond

;

;	input:	none

;	output:	none

;	modify:	flags

;--------------------------------------------------------------------

delay_1ms proc near

;

; insert code here that delays 1 millisecond

;

ret

delay_1ms endp

;--------------------------------------------------------------------

cod	ENDS

end

9.   Software Test Plan for Audio Applications 3.03

This section presents a check list in testing the Audio Applications 3.03 on AudioDrive OEM hardware.

General Information

Tester Name:	  ___________________________		Date: _______________

Audio Applications Version:  ___________

PC Configuration: __________________________________________

Windows:	  _____  Windows 3.1

_____  Windows 3.11

_____  Windows for Workgroup 3.1

_____  Windows for Workgroup 3.11

Windows Mode:     _____  Enhanced Mode

Display:   _____  Standard VGA

_____  Super VGA	Resolution:  _____ x _____

____  with large font

AudioDrive-based Board:	_____  ES488

_____  ES488 + OPL2

_____  ES1488

_____  ES688

_____  ES688 + OPL3

_____  ES1688

_____  Other  _____________________

Settings:	I/O Port 	_____

IRQ 	_____

DMA 	_____

ESS Setup

_____  device driver installation

_____  device driver un-installation

_____  software installation (complete)

_____  software installation (custom)

Audio Recorder

_____  About box

_____  playback wave files (8/16 bits and ESPCM, 8/11/22/44 kHz, Stereo/Mono)

_____  record and save wave files (8/16 bits and ESPCM, 8/11/22/44 kHz, Stereo/Mono)

_____  save and open ADPCM and ESPCM (Low/Medium/High) files (*.aud).

_____  Copy to Clipboard

_____  Cut

_____  Paste

_____  Paste Mix

_____  Delete

_____  Mute

_____  Zoom In

_____  Zoom Out

_____  Insert File

_____  Mix with File

_____  Effects

_____  OLE server

Volume Control

_____  About box

_____  playback volume level

_____  left/right balance control

_____  mute control

_____  Always Stay On Top

_____  run Mixer from Volume Control

Mixer

_____  About box

_____  volume level controls for Wave, Synth, Microphone, Line-In,  and CD

_____  left/right balance controls for Wave, Synth, Microphone, Line-In,  and CD

_____  mute controls for Wave, Synth, Microphone, Line-In,  and CD

_____  Always Stay On Top

_____  run Volume Control from Mixer

Recording Control

_____  About box

_____  record level control for Microphone, Line-In, CD, and Synth

_____  left/right balance control for Microphone, Line-In, CD, and Synth

_____  record from Microphone/Line-In/CD/Synth

_____  record from more than one source

_____  record monitor

_____  Always Stay On Top

Audio Accessories

_____  Chime

_____  Timer

_____  Stop Watch

_____  Audio Reminder

_____  Talking Clock

_____  Talking Calculator

_____  Audio Clip Library

Extended Recorder

_____  About box

_____  playback wave files (8/16 bits, 8/11/22/44 kHz, Stereo/Mono)

_____  record for more than a few minutes and save wave files (8/16 bits, 8/11/22/44 kHz, 	Stereo/Mono)

_____  record and playback using ESPCM files using on-chip compression (*.aud).

_____  record to fill up the free disk space.

Other Tests

_____  play *.mid and *.rmi files in Media Player.

_____  play *.wav files, which have been saved by Audio Recorder,  in Media Player and Sound 		Recorder.

_____  Windows Sound System 2.0

_____  Video for Windows

_____  Microsoft Multimedia Pack