setpalette: palette set/get program

===================================

Another quick hack.

This allows you to set or get the current color palettes used in all

RAMDAC's. That should allow you to change all 16 colors normally used for

text mode display. A real necessity if you have a grayscale monitor.

The program allows you to change all 256 palette entries, although text mode

only uses 16. Just in case someone has a use for it.

There are other similar utilities around (like the ones in svgalib), so this

will be of little real use. But is was lying around somewhere picking up

dust...

The commands are used as follows:

../getpalette: version 0.9. (c) 1995 Koen Gadeyne.

Usage: ../getpalette [options] [color_index]

Options: -n  Don't program VGA hardware

-d  print debugging information

-h  print usage information

color_index: index number in the palette look-up table (0..255).

or 'all' to show all 256 entries

../setpalette: version 0.9. (c) 1995 Koen Gadeyne.

Usage: ../setpalette [options] color_index R G B

Options: -n  Don't program VGA hardware

-d  print debugging information

-h  print usage information

color_index: index number in the palette look-up table (0..255).

or '-' to use standard input

(Input format = '<index>: <R-value> <G-value> <B-value>')

R, G, B : color intensity for Red, Green or Blue (0..63).

This should be clear enough.

SPECIAL FEATURE:

----------------

A special feature (if you can call that a feature) is the possibility to use

the output of getpalette DIRECTLY as input of setpalette (with '-' as color

index, using standard input).

This opens new possibilities for power-users who want to use a script to

dynamically and selectively change text colors.

It also allows you to redirect the output from getpalette to a file, and

then manually edit it, using it as new input for setpalette, e.g. for

setting custom colors when booting Linux.

Examples:

a simple "no-op" would be:

> getpalette all | setpalette -

This should change absolutely nothing (note the "should" ;-)

You could insert any sort of fancy processing into that pipeline, if you

like.