This file contains descriptions of all the functions of the library.

See the source for all the dirty deatils if you like.

Interrupt driven routines

=========================

uart_int_define_port

--------------------

Syntax: int uart_int_define_port (unsigned baseaddr, unsigned intlevel,

unsigned inbuf_size, unsigned outbuf_size,

unsigned char handshake)

What it is for:

This function registers a port with the library for further use.

It returns a handle that is then used with other functions.

Return values:

UE_ERROR (-1)		no UART found at this base address

UE_NO_HANDLES (-3)	no more free handles left

UE_NO_FREE_MEMORY (-4)	not enough free memory for the buffers

UE_BAD_INBUF_SIZE (-5)	illegal inbuf size (see header file)

UE_BAD_OUTBUF_SIZE (-6)	illegal outbuf size (dito.)

UE_BAD_HANDSHAKING (-7)	improper handshaking mode (see header file)

UE_BAD_INTLEVEL (-8)	only interrupt levels 3-7, 9-12, 14-15 allowed

0-31			handle

Explanation:

Call this function prior to any other uart_int_... function.

It sets up the buffers and the interrupt handlers.

uart_int_undef_port

-------------------

Syntax: int uart_int_undef_port (int handle)

What it is for: This is the counterpart of the function above.

Return values:

UE_OK (0)		if everything has gone well

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

Explanation:

Use this function to revoke calls to uart_int_define_port whenever

you want. You don't need to call this function prior to exiting to

DOS; this is done automagically.

uart_int_enable

---------------

Syntax: void uart_int_enable (void)

What it is for: Start servicing interrupts by unmasking them.

Return values: none

Explanation:

This function clears the appropriate bits in the IMR of the ICU,

so that serial interupts are being serviced.

Note that you can't call uart_detect_intlevel after a call to this

function!

uart_int_disable

----------------

Syntax: void uart_int_disable (void)

What it is for: Counterpart of the above.

Return values: none

Explanation:

This function sets the appropriate bits in the IMR of the ICU,

so that no serial interrupts can be serviced.

uart_int_set_bps_rate

---------------------

Syntax: int uart_int_set_bps_rate (int handle, unsigned bpsrate)

What it is for: Specify the bps rate at which you want the port to

operate.

Return values:

UE_OK (0)		if everything has gone well

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

Explanation:

This function needs the correct divisor value, not something like

9600 or 19200. Use the constants in the header file or find your own

values by dividing 115200 by the rate you wish.

uart_int_set_line_params

------------------------

Syntax: int uart_int_set_line_params (int handle, int lineparams)

What it is for: Tell the UART which protocol you want (eg. 8N1)

Return values:

UE_OK (0)		if everything has gone well

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

Explanation:

Use the constants in the header file, eg.

LP_8BITS | LP_1STOP | LP_EVEN_PARITY    for 8E1

uart_int_read_line_params

-------------------------

Syntax: int uart_int_read_line_params (int handle)

What it is for: Read the line parameters previously set with the

function above.

Return values:

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

non-negative value		the line params

Explanation:

I don't know if there's much use for a function like that, but

I thought it should be there. It might stop you from fiddling with

the bits in the LCR yourself.

uart_int_receive_byte

---------------------

Syntax: int uart_int_receive_byte (int handle)

What it is for: Guess. :)

Return values:

UE_ERROR (-1)		if no byte is available

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

0-255			the next byte in the input stream

Explanation:

This function reads the next byte from the input buffer. It does not

wait for bytes to come in.

uart_int_send_byte

------------------

Syntax: int uart_int_send_byte (int handle, int byte)

What it is for: Output, obviously.

Return values:

UE_OK (0)		if the byte has been written to the output

buffer. This does not imply that it has

already been sent!

UE_ERROR (-1)		if the output buffer couldn't take your byte

(so try again later)

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

Explanation:

This function writes a byte to the output buffer, where it is taken

over by the interrupt when it is its turn to be sent.

uart_int_send_string

--------------------

Syntax: int uart_int_send_string (int handle, char *string)

What it is for: Sends a null-terminated string.

Return values:

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

non-negative		# of bytes sent

Explanation:

This function sends a null-terminated string, leaving out the \0

at the end. It returns the number of bytes sent; this value can be

different from the string length when a timeout situation appeared

(that is, XOFF received or CTS dropped for more than 2 seconds).

Thus the return value provides you with an index to your string from

where to start over again.

Limitations:

No segment wraps, not even in huge models, so there is a maximum

size of 65535 characters.

Example:

strcpy(string,"foobarbaz");

x=0; while (string[x]!=0) x+=uart_int_send_string(handle,&string[x]);

uart_int_send_block

-------------------

Syntax: int uart_int_send_block (int handle, void *block, unsigned count)

What it is for: Sends a bunch of data.

Return values:

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

non-negative		# of bytes sent

Explanation:

Same as above, except that it does not stop when it encounters a \0

but when 'count' bytes have been sent or when a timeout occured.

Limitations:

Same as above.

uart_int_clear_receive_buffer

-----------------------------

Syntax: int uart_int_clear_receive_buffer (int handle)

What it is for: clears the receive buffer.

Return values:

UE_OK (0)		if everything has gone well

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

Explanation:

This functions throws away all bytes that have been received but

not yet read from the buffer. It also clears the internal rx fifo of

the 16550A.

uart_int_clear_send_buffer

--------------------------

Syntax: int uart_int_clear_send_buffer (int handle)

What it is for: clears the output buffer.

Return values:

UE_OK (0)		if everything has gone well

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

Explanation:

This functions throws away all bytes that have been put into the

output buffer but not yet taken over by the interrupt. It stops

transmission immediately.

uart_int_set_break

uart_int_clear_break

--------------------

Syntax: int uart_int_set_break (int handle)

int uart_int_clear_break (int handle)

What they are for: set/clear the break bit

Return values:

UE_OK (0)		if everything has gone well

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

Explanation:

The UART has a condition called break (see The Serial Port); these

functions allow you to control that.

uart_int_set_local_loopback

uart_int_clear_local_loopback

-----------------------------

Syntax: int uart_int_set_local_loopback (int handle)

int uart_int_clear_local_loopback (int handle)

What they are for: switch UART to local loopback or normal mode

Return values:

UE_OK (0)		if everything has gone well

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

Explanation:

In local loopback mode, your software receives every byte sent; see

The Serial Port for details.

uart_int_read_scr

uart_int_write_scr

------------------

Syntax: int uart_int_write_scr (int handle, int value)

int uart_int_read_scr (int handle)

What they are for: read/write the scratch register

Return values:

UE_OK (0)		if everything has gone well (write)

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

non-negative		contents of the scratch register (read)

Explanation:

The scratch register is the register at base+7 of the UART. With

some cards, this register has special meaning, that's why these

functions have been provided.

uart_int_set_dtr

uart_int_clear_dtr

uart_int_set_rts

uart_int_clear_rts

------------------

Syntax: int uart_int_set_dtr (int handle)

int uart_int_clear_dtr (int handle)

int uart_int_set_rts (int handle)

int uart_int_clear_rts (int handle)

What they are for: modem control

Return values:

UE_OK (0)		if everything has gone well (write)

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

Explanation:

These functions allow you to set/clear the modem control outputs

of the port. See ... you guess it.

uart_int_set_handshake

----------------------

Syntax: int uart_int_set_handshake (int handle, unsigned handshake)

What it is for: change the handshake settings for a port

Return values:

UE_OK (0)		if everything has gone well (write)

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

UE_BAD_HANDSHAKING (-7)	if this mode does not exist (use the constants

from the header file)

Explanation:

This functions allows you to change handshaking whenever you want.

uart_int_set_msi_handler

uart_int_set_lsi_handler

uart_int_default_handler

------------------------

Syntax: int uart_int_set_msi_handler (int handle,

void (*msihandler)(int handle, unsigned baseaddr, int msr));

int uart_int_set_lsi_handler(int handle,

void (*lsihandler)(int handle, unsigned baseaddr, int lsr));

What they are for: Specify your own handlers for error conditions.

uart_int_default_handler is a function that does just nothing but

matches the criteria for handler functions, so use this one to get rid

of your own... It is automagically assigned with uart_int_define_port.

Return values:

UE_OK (0)		if everything has gone well

UE_BAD_HANDLE (-2)	if this handle is not associated with a port

UE_BAD_HANDLER (-11)	null pointer encountered...

Explanation:

Whenever there is an error condition or the modem changes state of the

control lines (say, the phone rings), the UART generates an interrupt.

With these two functions you have a means of intercepting such inter-

rupts. You get the handle, the base address, and the condition that

triggered the interrupt, so you can take care of whatever you want.

But _don't_ read LSR or MSR, that's why you get this value from the

caller!

Another thing: don't use DOS functions in your handlers. DOS is not

re-entrant, so you will likely crash the system if you use disk or

screen I/O.

uart_int_get_baseaddr

---------------------

Syntax: unsigned uart_int_get_baseaddr (int handle)

What it is for: read the base address from the port table

Return values:

UE_BAD_HANDLE (65534)	if this handle has not been assigned to a port

else			handle

Explanation:

This function allows you to read back the base address information

you have provided previously with uart_int_define_port.

Polled mode routines

====================

uart_set_bps_rate

-----------------

Syntax: int uart_set_bps_rate (unsigned baseaddr, unsigned divisor)

What it is for: Set the bps rate of a UART

Return values:

UE_ERROR (-1)		if no UART found

UE_OK (0)		if OK

Explanation:

Use this function to write the divisor value to the divisor latch

register of a UART.

divisor = 115200 / desired rate

uart_set_line_params

--------------------

Syntax: int uart_set_line_params (unsigned baseaddr, unsigned lineparams)

What it is for: Define line parameters like parity and # of bits

Return values:

UE_ERROR (-1)		if no UART found

UE_OK (0)		if OK

Explanation:

Use the constants from uart.h; join them with ||

#define LP_5BITS	0x00

#define LP_6BITS	0x01

#define LP_7BITS	0x02

#define LP_8BITS	0x03

#define LP_1STOP	0x00

#define LP_2STOP	0x04

#define LP_NO_PARITY	0x00

#define LP_ODD_PARITY	0x08

#define LP_EVEN_PARITY	0x18

#define LP_MARK_PARITY	0x28

#define LP_SPACE_PARITY	0x38

uart_set_break

uart_clear_break

uart_set_dtr

uart_clear_dtr

uart_set_rts

uart_clear_rts

uart_set_local_loopback

uart_clear_local_loopback

-------------------------

Syntax:

void uart_set_break (unsigned baseaddr)

void uart_clear_break (unsigned baseaddr)

void uart_set_dtr (unsigned baseaddr)

void uart_clear_dtr (unsigned baseaddr)

void uart_set_rts (unsigned baseaddr)

void uart_clear_rts (unsigned baseaddr)

void uart_set_local_loopback (unsigned baseaddr)

void uart_clear_local_loopback (unsigned baseaddr)

What they are for: set or clear several control flags of the UART (see

The Serial Port for details)

Return values: none

Explanation:

See The Serial Port.

uart_enable_fifo

uart_disable_fifo

-----------------

Syntax:

int uart_enable_fifo (unsigned baseaddr, unsigned triggerlevel)

void uart_disable_fifo (unsigned baseaddr)

What they are for: control of the 16550 fifos

Return values (uart_enable_fifo only):

UE_OK (0)		if OK

UE_ERROR (-1)		if no FIFO available (8250 or 16450)

UE_BAD_FIFO (-10)	if it's a 16550 (no A), FIFO not enabled

Explanation:

Use these functions to enable/disable the built-in FIFOs of some

UARTs (see The Serial Port). Both functions clear the FIFOs! Use

these constants for RX triggerlevel (the value is only important if

you are using interrupt-driven functions with this port as well):

FIFO_1		0x00

FIFO_4		0x40

FIFO_8		0x80

FIFO_14		0xC0

uart_poll_send_byte

uart_poll_send_string

uart_poll_send_block

--------------------

Syntax:

int uart_poll_send_byte (unsigned baseaddr, int character)

int uart_poll_send_string (unsigned baseaddr, char *string)

int uart_poll_send_block (unsigned baseaddr, void *block, unsigned blocksize)

What they are for: send data in polled mode

Return values:

UE_OK (0)		if OK

UE_ERROR (-1)

uart_poll_receive_byte

----------------------

Syntax: int uart_poll_receive_byte (unsigned baseaddr)

What it is for: Receive characters

Return values:

UE_ERROR (-1)		if no character available

non-negative		character

Explanation:

This routine fetches a byte from the RBR of the UART (see

The Serial Port for the dirty details) if there is one available.

uart_get_line_status

--------------------

Syntax: int uart_get_line_status (unsigned baseaddr)

What it is for: reading the LSR

Return values:

LS_DR		0x01		data ready

LS_OE		0x02		overrun error

LS_PE		0x04		parity error

LS_FE		0x08		framing error

LS_BI		0x10		break indicator

LS_THRE		0x20		transmitter holding register empty

LS_TEMT		0x40		transmitter empty

LS_PENDING	0x80		error pending in FIFO (16550+ only)

Each bit of the return value si to be regarded as a flag indicating

a certain situation if set.

Explanation:

Use this function to see if everything's gone well with the trans-

mission. A good receiver function uses this function to determine

if there's a byte pending (LS_DR) and if the byte has been received

without error prior to retrieving the byte with

uart_poll_receive_byte.

uart_get_modem_status

---------------------

Syntax: int uart_get_modem_status (unsigned baseaddr)

What it is for: reading the modem status register

Return values:

MS_DELTA_CTS	0x01	CTS has changed state

MS_DELTA_DSR	0x02	DSR has changed state

MS_TERI	    	0x04	trailing edge of RI

MS_DELTA_DCD	0x08	DCD has changed state

MS_CTS	    	0x10	CTS

MS_DSR	    	0x20	DSR

MS_RI	    	0x40	RI

MS_DCD	    	0x80	DCD

Explanation:

This function provides you with information on the modem state.

uart_read_scratch_register

uart_write_scratch_register

---------------------------

Syntax:

int uart_read_scratch_register (unsigned baseaddr)

void uart_write_scratch_register (unsigned baseaddr, int value)

What they are for: reading and writing the SCR

Return values (uart_read_scratch_register only):

Contents of the SCR

Explanation:

The scratch register has no function with most serial cards.

However, there are boards (especially RS-485) that use bits

of this register to enable or disable receiver and transmitter

drivers or to select an interrupt level. This is why these

functions are provided.

Detection functions

===================

uart_detect_type

----------------

Syntax: int uart_detect_type (unsigned baseaddr)

What it is for: Detects UART type

Return values:

UE_ERROR (-1)		if no UART found at this base address

0			8250

1			16450

2			16550 w/o scratch register

3			16550

4			16550A w/o scratch register

5			16550A

Explanation:

This functions does its best to determine the UART type used in your

PC. However, there is no safe method to distinguish between 8250 and

16450, so you might get false info here (if you get 0 or 1, see if

it's an XT -> 8250 or an AT or higher -> 16450).

uart_detect_intlevel

--------------------

Syntax: int uart_detect_intlevel (unsigned baseaddr)

What it is for: Detection of interrupt level associated with this UART

Return values:

UE_ERROR (-1)		if no intlevel found

0-15			the intlevel of this UART

Explanation:

This function can be used to find out about the interrupt level your

UART is connected to. Please note that this function cannot detect

interrupt levels currently in use! So if you have two UARTs sharing

one intlevel, call this function for both of them _before_ you enable

any of their interrupts. With this library this means: don't call

this function after uart_int_enable().

Constants

=========

char *uart_version, *uart_date, *uart_author, *uart_internet, *uart_address;