Additional notes for CRT terminals.

The menu of CRT terminals displayed in the customization

program contains two undefined entries: "Customer 2" and

"Customer 1". These are actually preconfigured as alternatives

for some Hazeltine and SOROC terminals.  They are described

below:

CUSTOMER 2     This is preconfigured to support Hazeltine and

Craig Volker terminals which use the "ESC" (code

1B hex) character as the lead in, instead of the

"~" (code 7E hex).

CUSTOMER 1     This is preconfigured to support the "Insert

Line" and "Delete Line" functions on the SOROC

140 terminal, which are not available on the

SOROC 120.

On Hazeltine terminals  you must  set the  "Auto LF" option

switch to the OFF position.  This will not affect the terminal's

operation with CP/M based systems, but is necessary for VEDIT to

work properly.

Following your selection of  a terminal from  the menu, the

customization  program  asks   for  up   to  two   "Escape  Mode

Characters", in  case you  are using  escape sequences  from the

keyboard.  Your terminal may also send escape sequences when you

type a function  key.  For example,  the Heath/Zenith  H19 sends

escape sequences beginning with the ESC character. The Televideo

920C sends escape sequences beginning with "CTRL-A".

The next two questions  are "Enter Common  2nd Character #1

in Escape Sequence" and "Enter Common 2nd Character #2 in Escape

Sequence". Simply answer with  a <RETURN>  if you  are not using

escape sequences or are  typing them in  by hand.  However, some

terminal's special function keys send  3 letter escape sequences

where the  second  letter  is  always  the  same  and  should be

ignored.  In this case, answer with this second character.

For example, on the H19 using Heath Mode, the PF1 key sends

an "ESC ? p", the PF2 key sends an "ESC ? r" and so on.  In this

case, the  "?"  is  the  "Common  2nd  Character #1...".  Answer

RETURN to "Common 2nd Character #2...".

On the VT-100,  VT-180 and  other ANSI  standard terminals,

the function keys send sequences  such as "ESC O  A", "ESC O B",

"ESC [ H", "ESC [ I", etc, where both the "O" and "[" are common

2nd characters which  are to be  ignored.  Therefore "Common 2nd

Character #1..." should be  answered with a  "O" (capital letter

O), and "Common 2nd  Character #2..." should  be answered with a

"[".

Some terminals, like  the IBM  3101,  also send  a Carriage

Return  at   the   end   of   escape   sequences.  The  keyboard

customization detects this  automatically  and  you  need not be

concerned with it.

While most  terminals  have  24  lines,  some  have  a 25th

"Status Line".  On some of  these, it  is possible  for VEDIT to

place its  status line  on the  25th line.  These  terminals are

marked with a "*" following the terminal's name in the menu.  To

use the 25th line  answer the  question "Enter  Number of Screen

Lines" with a "25".  Note  that the Intertube  must be specified

as having 25  lines. VEDIT  cannot use  the status  lines on the

TELEVIDEO or IBM 3101 terminals.

In the  VEDIT CRT  version 1.37,  the right-most  column of

the screen is  not  used.  This  is  necessary  because many CRT

terminals perform a scroll  when the  last column  in the bottom

row is written.  If  loosing this  last column  is a  problem to

you, we can supply  a patch which  will permit VEDIT  to use the

last column, but at  the expense of  having scrolling not appear

as "clean".  (The status line will appear to jump up and down as

the screen scrolls.)  If  you are  sure that  your CRT  does not

scroll when writing the last  screen position, you may customize

VEDIT with the screen  length  value  one  larger  than the true

length.  Specifically, the H19 may be customized with a "81" for

the screen length parameter (customization Task 5.6).

For  your  information   the  file  "VEDITZC.LST"  includes

several pages  of  source  code  listing  of  the  CRT dependent

routines and tables in  VEDIT. The file  "CRT.ASM" is the source

code for the file "CRT.TBL" which contains the control codes for

each of the CRT  terminals displayed in  the menu. These control

codes and escape  sequences  perform  the  cursor addressing and

other CRT functions.  99% of  users will never  need to refer to

these listings, since  the customization  requires no "patches".

If you have a  terminal which is  not mentioned in  the menu, we

will gladly modify  the tables  for you.  However,  we also know

that some users run  custom hardware or  CRT emulation software,

and they may wish to make changes to these tables themselves (we

will gladly be of assistance here too).

While we  have  made  every  effort  to  support  each  CRT

terminal correctly, it is  not feasible  for anyone  to test all

models of every manufacturer. Should you notice any aberrations,

such as  scrambled  lines  or  lost  characters,  it  is  almost

certainly due to  an incorrect  setting on  your terminal  or an

error in the support for your terminal.  Please be sure that any

switches pertaining to cursor  addressing, editing functions and

correct "Escape Sequence Lead In"  are enabled.  Should you have

any suggestions or  difficulties,  no  matter  how minor, please

contact us. Should you write, please include you office and home

phone numbers.

###################################