GENERAL NOTES ON CDOS' UNDOCUMENTED FEATURES

When  CDOSGEN asks you whether the drive is large [L] or  small

[S] try answering 'X'.  You will then get a menu for  Shugarts,

etc.

You  can  use  the  'SYS.DIR'  FCB  create call of CDOS 1.07 or

higher to access  the  directory  regardless  of  its  size  or

position  on the disk.  Although the FCB created with this call

is write protected, you can reset that  attribute  bit and  can

then write to the directory as well as read it.

Disk Labels.

A directory label is written to the disk by STAT and is used to

ascertain  the  storage capacity of the disk and the number  of

directory entries (64 to 512).

The last 8 bytes of the first boot loader sector (usually  side

0  track 0 sector 1) are always recorded in single density  and

contain eight bytes indicating the type of disk to the BIOS, eg

LGSSSD

for Large (8"), Single Sided, Single Density

or   LGDSDD

for 8" Double Sided, Double Density diskettes (1.2 MBytes)

STAT  2.15  was written for a WD1797 FDC chip (it  records  the

side  numbers  into the address fields) although a  WD1793  was

eventually used.  CDOS 2.36 does not support the 1797, however,

and this chip will not work instead of the 1793 on the 16FDC.

Double Density Recording Format:

16 sectors of 512 bytes are used per track.(MFM)

A 12 interleave is used (1,C,7,2,D,8,3,E,9,4,F,A,5,10,B,6)

Although  a 4 interleave can be read as COM files in my 4MHz no

wait state system a 6 interleave speeds throughput by a  factor

of  two.  (use 1,2,3,4,5,6,C,D,E,F,10,7,8,9,A,B).  INIT can  be

modified  to  do  this..  if  interested write me  and  I  will

disclose all....

Finally,  if  someone has deciphered how to call the 2.36  BIOS

directly without getting error returns I am all ears...

Trevor Marshall,

26 Mirrelia Way, Ferndale, Western Australia 6155

phone International (619)4576049   or national (09)457 6059

14 December 1980

Notes added by:	Chuck Weingart		February 1, 1981

2152 W. Iowa

Chicago, Ill 60622

On the "X" feature of CDOSGEN, described above, you must respond

to  several  questions,  such  as  "Fast  or  Slow",  "Single or

Double".  All Shugart type drives are "slow".   "Double"  drives

are  those like the Persci, that have one seek mechanism for two

different disks, or when you are  using  a  Shugart  851  double

sided  drive as two "drives" (one "drive" on each side, requires

rewiring the cable).

In CDOSGEN 2.36, when you respond "S" to  the  drive  type,  you

again  get  a "Fast or Slow?"  question.  MPI drives are "fast",

and Shugart 400 and Wangco drives are "slow" in my experience.

It is easy to attach one of the new Shugart  801/851  drives  to

the 4FDC, just use the "TS" data separator (install the jumper).

If  you  are  trying  to use an old 800-1 or 801 drive, the data

separator adjustments probably have to be changed to  work  with

the 4FDC.

To attach the MPI model 52 double-sided 5" drive to a 4FDC, just

add  a jumper on the 4FDC from J4 pin 21 to J2 pin 32, if one is

not already there.  Then gen a 5"  double-sided  drive  at  that

position  with 2.17 or greater, initialize some disks with INIT,

and write a double-sided label with STAT, and you are  ready  to

go. I converted to double-sided in one hour, and it's great!

To  attach  a  Shugart 851 to the 4FDC, install the TS jumper as

noted above.  Disconnect the wires 2, 12, 18, and  32  from  the

cable (at the drive end is easiest).  Connect the Shugart pin 12

to  the 4FDC pin 2 (for side select), Shugart pin 30 to 4FDC pin

4 (for Drive select 3), and Shugart pin 32 to 4FDC pin  18  (for

Drive select 4).  Connect a jumper from J4 pin 21 to J3 pin 2 if

one  is  not  already  there.   Use  the "X" feature and 2.17 or

greater, initialize disks with INIT, and label  the  disks  with

STAT for double-sided operation.

CDOS  1.07  can  be  used  unchanged  on the California Computer

Systems 2422 disk controller if there is a  Cromemco  compatible

console  port  at  0,  such  as a 3P + S, Cromemco TUART, or the

console on the Cromemco SCC.  The latter will have to  have  two

modifications  made:  the  prom (U25) must be altered to put the

parallel port at address 04 somewhere else  (such  as  0E),  and

there  must  be  a 220 ohm resistor in series with U52 pin 6 (to

delay PWR) and a 400pf capacitor from U52 pin 6 to  ground  (pin

8).

CDOS 2.17 can be used on  the  CCS  2422  controller  in  single

density  if  one  uses  the  1.07  bootstrap  loader.   The 2.17

bootstrap will not work without a hardware change  on  the  2422

board,  and  it is somewhat lengthy.  There will be a persistent

problem of read errors on track 2 sector 1, but this is  due  to

the  WD1793 chip.  Just Retry the operation and it will clear up

every time.

CDOS 2.17 has a command, VERIFY, with  one  of  three  operands.

VERIFY ON will enable read-after-write verification on the disk.

VERIFY  OFF will disable that, and just VERIFY will indicate the

status of the feature.  VERIFY is not present in CDOS 2.36.

CDOS 2.17 and up has  several  undocumented  commands:  REM  and

ATTR.   ATTR  is  the  same  as  ATRIB, and REM is for inserting

remarks into your batch file, because the REM line is ignored.

Although  it  isnt  currently  stated  in  any   Cromemco   doc-

umentation  that  I have seen (and I have written Cromemco twice

about it with no answer),  all  versions  of  Cromemco  software

shipped  since  about  February 1980 come with the system genned

for 64K.  If you have a copy of CDOS 1.07, you can run the  2.17

or  2.36  versions  of CDOSGEN under 1.07 in order to generate a

smaller system.  The new versions of CDOSGEN will not run  in  a

32K system, tho, they are quite a bit bigger.

For users of non-Cromemco memory boards that want to get a "full

house"  (64K),  here is how to add a Phantom signal to the 4FDC.

Add a jumper from IC46(74367) pin 15 to IC31(2708) pin 20.   Add

a  jumper from IC46 pin 12 to pin 8 (ground).  Add a jumper from

IC46 pin 11 to S100 pin 67 (Phantom output).  The  memory  board

addressed  at  C000H  must  respond  to  the  Phantom signal, of

course.  The RES switch (SW2) must be set on.  This way, you can

use the 4FDC RDOS program until CDOS is booted, then  the  EPROM

will  be turned off and the memory board "behind" it can then be

used.  This method must be used even in  many  boards  that  are

advertised  as  "Cromemco banking compatible" because the boards

do not have the feature of being bank-disabled upon power-up.

CDOS 2.17 now takes about 11K minimum for itself.  CDOS 2.36 now

takes 14K minimum.

STAT for 2.36 has several undocumented switches: /M, /N, /E, and

/EZ.   STAT/M  allows  you to change the "master" drive (the one

that CDOS looks at if it cant find  a  program  on  the  current

drive).   STAT/N  gives  a  5-up directory display.  STAT/E is a

directory erase, with prompting.  STAT/EZ erases, and no prompts

as I recall.

Jordan Siedband tells me that 2.36  will  not  read  CP/M  disks

correctly,  cause  unknown.   Keep  a copy of 2.17 around if you

want to continue to use CPMUG stuff.  There is one thing that is

obvious to anyone who has looked: CP/M and CDOS directories  are

compatable  only in  "1.4 mode", that is, no system flags of any

kind set in the directories,  either  CP/M  2.2  flags  or  CDOS

flags. When they  are  not  compatable, the one operating system

will usually clobber directory entries in the  other  format  or

refuse to work.

CDOS 1.07 and up have an  undocumented  error  message:  LOGICAL

DISK ERROR n.  This message is produced if you do something like

request  a  track that does not exist, or a sector that does not

exist, or try an operation that is impossible for the  drive  to

perform.   You will generally get these messages only if you are

trying to do disk I/O thru the "BIOS" directly.

There is a small bug in the Divide Integers CDOS call  (8AH)  in

2.17  -  the BC register will be altered.  No problem in 1.07 or

2.36 as far as I can tell.

All  versions  of  CDOS have the CP/M "BIOS" jump table in them,

but none of them use it.  That is, you can jump to  the  entries

in this table, but cannot modify the jump addresses in the table

and  expect it to work.  The first entry in the table, the "cold

start" jump, is a jump to itself, because CDOS never  uses  this

entry,  and  the  user  is not supposed to use it, either.  This

makes routines like FAST rather difficult  to  use  under  CDOS.

Some CPMUG programs set themselves below the operating system by

changing the address at location 6, but that will not work under

CDOS.   You  can use the CDOS call 97H to do the same thing, see

the manual for details.

All  versions of CDOS I have used  try  to initialize all  TUART

ports in the system.  That is, they will output to ports 12, 13,

22,  23,  52,  53,  62, ...  F2, and F3.  If you have some other

hardware at those addresses, good luck.  CDOS will also zero out

the Dazzler port at 0E every time it starts disk I/O.

CDOS 2.17 and up will disable all interrupts when doing disk I/O

and never  re-enable.   This  means  you  can't  use  the  timer

interrupts available in the TMS5501.

If anyone figures out what attributes S and U are, I would like

to know.  They can be set, reset, and listed,  but dont seem to

make any difference to the operating system. Could they be used

for Hard disk files?

##########################################