AN EPROM PROGRAMMER FOR THE BIG BOARD

written by Jim Monesmith

438 Patterson Road

DAYTON, OHIO 45419

If you ever have a need to program a 2708, 2716, or 2732 eprom then

this article is for you. The article is presented in four parts. they are

construction, bigboard jumpers, software, and operation documentation. I am

noting at this time that the object code listing presented in the software

section is for a 4 MHz. CPU clock bigboard. If you are using a 2.5 MHz. CPU

bigboard you will have to change the sofware timing loops for the one mille

second and the 50 millesecond programming pulses. See the source listing for

these changes. Another option is to modify your slow bigboard. If you chose

this second option then be sure to use the 4 MHz. PIO, SIO, CTC, and CPU

devices. The second issue of this publication will show you more details

on this modifaction.

PART I     PROGRAMMER AND POWER SUPPLY CONSTRUCTION

Using the eprom programmer parts list, and the accompaning schematic

construct the programmer on a wire wrap board of your choice. Don't forget to

use decoupling capacitors between Vcc and ground (.01 micro farad cap. per ic)

even though they are not shown on the schematic. Also use three 15 micro farad

tantalum electrolytic capacitors between the plus five volt supply and ground.

Observe polarity with the + lead connected to Vcc and the other lead connected

to ground. Using connectors of your choice construct a cable to attach the

programmer to the bigboard connector J5 with the following pin designations:

PROGRAMMER SIGNAL NAME		  BIGBOARD PIN NUMBER

DATA BIT 0-----------------------J5 PIN 6

DATA BIT 1-----------------------J5 PIN 8

DATA BIT 2-----------------------J5 PIN 10

DATA BIT 3-----------------------J5 PIN 12

PORT A	    DATA BIT 4-----------------------J5 PIN 14

DATA BIT 5-----------------------J5 PIN 16

DATA BIT 6-----------------------J5 PIN 18

DATA BIT 7-----------------------J5 PIN 20

CLOCK----------------------------J5 PIN 26

PGM------------------------------J5 PIN 30

PORT B	    PROG-----------------------------J5 PIN 32

VERIFY---------------------------J5 PIN 34

RESET----------------------------J5 PIN 36

GROUND---------------------------J5 PIN 1

GROUND---------------------------J5 PIN 3

Using the power supply schematic and the power supply parts list,

construct the power supply on a prototype card of your choice. Be sure to

observe polarity of the electrolytic capacitors. Correct polarity for the

positive supplies is for the + lead to go to the positive side of the supply,

and the other lead to ground. For the -5 volt supply the + lead should go to

ground, and the other lead to the -5 volt supply. Be sure and mount LM340T-5

plus five volt regulator and the LM317T adjustable positive voltage regulator

on heat sinks, using heat sink grease. The LM340T-12 twelve volt regulator

and the LM320T-5 minus five volt regulator do not require heat sinks.

Now you need to fabricate the personality modules. On separate 16 pin

dip headers make the following jumper connections for the personality modules:

pin 1 to pin 16

2716 PERSONALITY MODULE		pin 2 to pin 15

pin 3 to pin 14

ON A SEPARATE 16 PIN HEADER:

pin 4 to pin 13

2732 PERSONALITY MODULE		pin 6 to pin 11

pin 7 to pin 10

Next mount the 24 pin textool zero insertion force sockets to two

24 pin dip headers. This will make their attachment to the rest of the circuit

simpler.

The final step in the construction of the programmer is to adjust the

programming voltage. This is accomplished by adjusting R19 (the pot in the

power supply section) while the program is running menu selection number 6

WITH NO EPROMS IN THE PROGRAMMING SOCKETS !!!!!!!!!  R19 is adjusted until

the voltage at test point TP-1 is exactly 25.5 volts. Be sure to use a very

accurate high impedance meter for this adjustment.

This completes the construction section as the 2708 eprom does not

require a personality module.

I might add at this point that with the voltage set at 25.5 volts

the programmer will program Intel 2708's, Intel 2716's, and Intel 2732's, or

any device that meets their respective specifications.

NOTE THIS WELL !!!!!!!!!!!!!!!!!!!!!

IF YOU WANT TO PROGRAM AN INTEL 2732A EPROM YOU WILL HAVE TO LOWER

THE PROGRAMMING VOLTAGE AT TP-1 TO 21 VOLTS. INTEL CHANGED THE

SPEC. ON THE 2732A PROGRAMMING VOLTAGE FROM 25 VOLTS TO 21 VOLTS.

THE 2732A WILL BE DAMAGED IF YOU TRY TO PROGRAM IT AT 25.5 VOLTS !

YOU HAVE BEEN WARNED.

EPROM PROGRAMMER PART LIST

NUMBER		DESCRIPTION

1		  PROTOTYPE BOARD OF YOUR CHOICE

1		  74LS00  2 INPUT NAND GATE

1		  7407    OPEN COLLECTOR BUFFER WITH HIGH VOLTAGE OUTPUTS (30V)

1		  74LS04  HEX INVERTER

1		  MC14040B 12 BIT BINARY COUNTER (MOTOROLA)

1		  8286     BI-DIRECTONAL BUFFER (INTEL)

2		  2N2222A  NPN TRANSISTOR

2		  2N3906   PNP TRANSISTOR

3		  2N3904   NPN TRANSISTOR

1		  1N914    DIODE

2  	   	  24 PIN ZERO INSERTION FORCE SOCKET (TEXTOOL 224-3344)

2		  24 PIN WIRE WRAP SOCKET

2		  24 PIN DIP HEADER

1		  20 PIN WIRE WRAP SOCKET

5		  16 PIN WIRE WRAP SOCKET

2		  14 PIN WIRE WRAP SOCKET

4		  16 PIN DIP HEADER

2		  3K   OHM RESISTOR 1/4 WATT  5%

1  		  27K  OHM RESISTOR 1/4 WATT  5%

1		  620  OHM RESISTOR 1/4 WATT  5%

1		  1K   OHM RESISTOR 1/4 WATT  5%

1		  2.7K OHM RESISTOR 1/4 WATT  5%

3		  10K  OHM RESISTOR 1/4 WATT  5%

2		  50   OHM RESISTOR 1/4 WATT  5%

1		  5.1K OHM RESISTOR 1/4 WATT  5%

1		  11K  OHM RESISTOR 1/4 WATT  5%

1		  3.6K OHM RESISTOR 1/4 WATT  5%

2		  2.2K OHM RESISTOR 1/4 WATT  5%

2		  0.1  MICRO FARAD CAPACITOR  50 VOLT CERAMIC

10		  0.01 MICRO FARAD CAPACITOR  50 VOLT CERAMIC

2		  1.0  MICRO FARAD CAPACITOR  50 VOLT  TANTALUM ELECTROLYTIC

1		  .004 MICRO FARAD CAPACITOR  50 VOLT CERAMIC

POWER SUPPLY PART LIST

NUMBER		DESCRIPTION

1		  PROTOTYPE BOARD OF YOUR CHOICE

1		  25.2 VOLT TRANSFORMER RADIO SHACK NUMBER RS-273-1505

1		  12.6 VOLT TRANSFORMER RADIO SHACK NUMBER RS-273-1480

8		  1N4005 DIODE

3		  100 MICRO FARAD CAPACITOR ELECTROLYTIC 50 volt

1		  1000 MICRO FARAD CAPACITOR ELECTROLYTIC 50 volt

4		  1.0 MICRO FARAD CAPACITOR TANTALUM ELECTROLYTIC 50 volt

1		  5K OHM POT

1		  200 OHM RESISTOR 1/4 WATT  5%

1		  LM317T ADJUSTABLE VOLTAGE REGULATOR

1		  LM340T-5   +5 VOLT REGULATOR

1		  LM340T-12  +12 VOLT REGULATOR

1		  LM320T-5   -5 VOLT REGULATOR

PART II		BIGBOARD JUMPERS

I also assume that your bigboard has the PIO parallel option installed

in it. You will also need two 74LS243 I.C.s installed in sockets U90 and U92 of

the PIO section. ON two separte 14 pin headers jumper the following pins, and

install the headers in sockets U91, and U93:

pin 6 to pin 8

pin 5 to pin 9

pin 4 to pin 10

pin 3 to pin 11

also jumper JB3 pin 16 to JB3 pin 14 now, jumper JB3 pin 14 to J5 pin 40. This

last connection uses bit 7 on PIO port B to control the direction of the bi-

directional buffers in U90 and U92. The eprom data is output and input on port

A of the general purpose PIO. The general purpose PIO port B is used to control

the programmer, with the following control signal designations:

BIT 0		CLOCK (ACTIVE HIGH)

BIT 1		NOT USED

BIT 2		PROGRAMMING PULSE (ACTIVE LOW)

BIT 3		PROGRAMMING MODE (ACTIVE LOW)

BIT 4		VERIFY MODE (ACTIVE LOW)

BIT 5		RESET (ACTIVE LOW)

BIT 6		NOT USED

BIT 7		BUFFER DIRECTION CONTROL (0=OUTPUT 1=INPUT)

PART III	SOFTWARE

Refer to the program listing. The program is very heavily

commented and should be self-explanatory. The program has been in use

for quite some time now, and I believe it to be completely bug free.

At this time I will present an object code dump of the program

for quick entry of the program into your machine. The code entry should

start at 0100 hex. After you have completed the entry, using DDT.COM,

type ^C and use the SAVE command to place the program onto your disk.

EXAMPLE:

DDT.COM

S100

ENTER THE DATA FROM THE OBJECT CODE LISTING

^C

SAVE 11 PROMPIO.COM

If your typing is accurate you will now have a working copy of the

eprom programmer sofware on disk.

If you are lazy, like myself, and do not want to type all of that

code in by hand, send me $5.00 for handling and an IBM compatable 8"

single density disk, and I will send you an assembled copy of the

program along with source code for the program. Don't forget to

include your return address and postage for the return trip. I can't

return your disk unless you include the postage for the return trip.

The package used to send the disk to me must be usable for the return

trip also.

PART IV 	OPERATION DOCUMENTATION

2708 READ:

Place the 2708 eprom in socket number one, then

type 1. The program will then read the contents of the eprom into

memory starting at location 4000 hex.

2708 COMPARE:

Place the 2708 eprom in socket number one and type 2. The program

will read the contents of the eprom into memory, starting at location

3000 hex. After the eprom is read into memory a comparison is made with

data starting at memory location 4000 hex. If any differences are found

the program outputs the location in the eprom, that dosen't compare

with memory, the data that was read from the eprom, and what the eprom

data should have actually been.

2708 PROGRAM:

Place the 2708 eprom in socket number one and type 3. The routine will

will program the 2708 with memory data starting at memory location 4000

hex. When the programming is complete (about two minutes) the routine

will enter the verify mode and read the eprom data into location 3000

hex. A byte by byte comparision will then be made and any differences

will be reported to the console. If the data in the eprom compares with

memory, this will also be reported to the console.

2716 READ:

First place the 2716 personality module in the personality socket. Then

place the 2716 eprom in socket number two and type 4. The program will

then tell you to make sure that the 2716 personality module is in the

personality socket and wait for a space to be typed at the console.

When the program receives a space, it will proceed to read the 2716

data into memory starting at location 4000 hex.

2716 COMPARE:

First place the 2716 personality module in the personality socket. Then

place the 2716 eprom in socket number two and type 5. The program will

then tell you to make sure that the 2716 personality module is in the

personality socket and wait for a space to be typed at the console.

When the program receives a space, it will proceed to read the 2716

data into memory starting at location 3000 hex. After the eprom data

is read into memory, a byte by byte comparision will be made with data

starting at memory location 4000 hex. If any differences are found the

program will report them to the console. If the eprom data compares

with the memory data, this will also be reported to the console.

2716 PROGRAM:

First place the 2716 personality module in the personality socket. Then

place the 2716 eprom in socket number two and type 6. The program will

then tell you to make sure that the 2716 personality module is in the

personality socket and wait for a space to be typed at the console.

When the program receives a space, it will tell you to turn on the

programming voltage, and wait for another space. When the program

receives the second space, it will proceed to program the eprom with

memory data starting at location 4000 hex. After the 2716 is completly

programmed, the routine will tell you to turn off the programming

voltage, and wait for a space to be typed at the console. After a space

is received, the routine will enter the verify mode and read the eprom

data into memory starting at location 3000 hex. Finally, a byte by byte

comparision is made with memory starting at location  4000 hex, and any

differences reported to the console. If the data read from the eprom

compares with memory data at location 4000 hex, this will also be

reported to the console.

2732 READ:

First place the 2732 personality module in the personality socket. Then

place the 2732 eprom in socket number two and type 7. The program will

then tell you to make sure that the 2732 personality module is in the

personality socket and wait for a space to be typed at the console.

When the program receives a space, it will proceed to read the 2732

data into memory starting at location 4000 hex.

2732 COMPARE:

First place the 2732 personality module in the personality socket. Then

place the 2732 eprom in socket number two and type 8. The program will

then tell you to make sure that the 2732 personality module is in the

personality socket and wait for a space to be typed at the console.

When the program receives a space, it will proceed to read the 2732

data into memory starting at location 3000 hex. After the eprom data

is read into memory, a byte by byte comparision will be made with data

starting at memory location 4000 hex. If any differences are found the

program will report them to the console. If the eprom data compares

with the memory data, this will also be reported to the console.

2732 PROGRAM:

First place the 2732 personality module in the personality socket. Then

place the 2732 eprom in socket number two and type 9. The program will

then tell you to make sure that the 2732 personality module is in the

personality socket and wait for a space to be typed at the console.

When the program receives a space, it will tell you to turn on the

programming voltage, and wait for another space. When the program

receives the second space, it will proceed to program the eprom with

memory data starting at location 4000 hex. After the 2732 is completly

programmed, the routine will tell you to turn off the programming

voltage, and wait for a space to be typed at the console. After a space

is received, the routine will enter the verify mode and read the eprom

data into memory starting at location 3000 hex. Finally, a byte by byte

comparision is made with memory starting at location  4000 hex, and any

differences reported to the console. If the data read from the eprom

compares with memory data at location 4000 hex, this will also be

reported to the console.

READ A .COM FILE INTO MEMORY AT LOCATION 4000 HEX:

Type a : at the console. The program will then ask you for the file

name. Enter the file name, with optional drive indicator. If no drive

is specified the routine will attempt to read the file from the

currently logged drive. If a drive is specified the routine will read

the file from the specified drive. If the file is not found, an error

message will be typed at the console. The file must be of type .COM.

The file also must be small enough to fit in the file buffer. The file

buffer starts at memory location 4000 hex and continues up to location

D3FF hex. The top address is just before the console command processor

(CCP). If the file is to large to fit in the file buffer, it must be

split into two, or more files and read into the buffer one at a time.

Use the DDT.COM program supplied by DIGITAL RESEARCH INC. with the

CP/M (tm) disk and the SAVE command to split the file.

EXAMPLE:

DDT FILENAME.COM

^C

SAVE 144 FILE1.COM

DDT FILENAME.COM

M9000,D3FF,0100

^C

SAVE 68 FILE2.COM

MOVE NEXT BLOCK OF DATA INTO POSITION TO BE PROGRAMMED:

Type a 0 at the console. The program will then ask what size block

of data if to be moved. The size of the block will depend on the eprom

being used. Type 1 for a 2708 eprom, 2 for a 2716 eprom, and 3 for a

2732 eprom. After receiving a 1,2, or 3 the routine will add the block

size to 4000 hex to calculate the starting address of the block move.

The routine will then do a block move using the calculated address

as the source address for the move, and 4000 hex as the destination

address for the move. Control will then be passed back to the main

menu.

SAVING THE CONTENTS OF AN EPROM:

Read the eprom into memory using the programmer and the appropriate

menu selection. Next type ^C. This will cause an exit from the program.

Now type DDT.COM. This will invoke DDT.COM into action. Next use DDT to

move the data at 4000 hex down to 0100 hex. After moving the eprom data

from 4000 hex to 0100 hex type ^C and use the SAVE command to put the

eprom image on disk.

EXAMPLE:

DDT

M4000,5000,0100

^C

SAVE XX EPROM.COM

XX depends on the size of the eprom read

2708 XX = 4

2716 XX = 8

2732 XX = 16

######################################################################################################