SDIAG PG. 1

SDIAG

VERSION 2.02

APRIL 11, 1989

SCSI DIAGNOSTIC SOFTWARE

WESTERN DIGITAL CORPORATION

SDIAG DOCUMENT BY,  JOHN LAUBE

SDIAG PROGRAM BY, JOHN SPONGR & TOM DEVLIN

SDIAG PG. 2

TABLE OF CONTENTS

1.)   OVERVIEW

2.)   INSTALLATION

3.)   MENU ORGANIZATION

4.)   DISCONNECT MENU

5.)   INITIATOR MENU

6.)   TARGET MENU

7.)   SBIC REGISTERS MENU

8.)   SCSI SETUP MENU

9.)   DATA DISPLAY MENU

10.)  COMMAND EXECUTION

11.)  IMP0RTANT POINTS

12.)  PROGRAMMING EXAMPLES

SDIAG PG. 3

S D I A G

(SCSI DIAGNOSTIC SOFTWARE)

1. OVERVIEW

The SDIAG software package can be used in conjunction with the WD

SCS-XTS or WD SCS-ATXT to provide assistance to the system

designers in debugging and developing products.  By combining two

WD SCS-ATXT boards with the SDIAG software the new designer may

enhance his knowledge of the SCSI standard ANSI x3.131 1986 and

minimize the learning curve for SCSI protocol.

Not only is SDIAG menu driven and easy to use, it provides direct

register access, automated command programming-using batch files

and on screen data display.  The menus provide all command

options available for each state and enable direct programming of

registers.

The SDIAG software package includes: SDIAG.EXE, CONFIG.SYS, I1,

T1.  SDIAG should be run under a DOS language. I1 and T1 are

sample batch files for the initiator and target modes operation.

2. INSTALLATION

"INSTALL" board(s) into computers with power-off.  Power-up and

load DOS program. Then place SDIAG software in drive and run

SDIAG.  There should be plenty of space on SDIAG disk to save

personalized batch files if not, they can be saved on a separate

disk.  The screen will now show:

----------------------- BY ------------------------

--------------- WESTERN DIGITAL CORP ---------------

Strike any key to proceed. The program will now display:

Enter RT10/XTS base address in hex (or <cr> for 300H):

The I/O address of either the RT10/WDSCSXTS or the WDSCSATXT is

needed.  The WDSCSXTS usually uses address 300.  Some XTS Bios

run from 320.  The WDSCSATXT FASST Bios runs at 340.  If you

receive a 'e7' or 'ff' int. or see  e7's in the registers, most

likely the wrong I/O port has been programmed. Next, the SDIAG

software will display:

Enter DMA channel 0. <1..3> (or 'Return' for 3):

SDIAG PG. 4

At this time the proper DMA channel must be selected.  Both the

WDSCSXTS and the WDSCSATXT are set on ch3 when shipped and should

work on this channel.  If not, channel 1 or 2 can be set. Well,

that's the basics for installation.

3. MENU ORGANIZATION

After SDIAG has been installed with proper I/O port and DMA

channel, the designer will have access to six menus.

The Disconnect, Initiator and Target menus must be used in the

respective applications and are used to implement an operation.

The Modify Set Up and Register Access menus can be accessed from

all three previously mentioned menus.  The Display Data menu can

only be accessed from the Initiator or Target menus.  The

following diagram shows menu access in SDIAG:

___________________________________

|                                 |

|                                 |

v                                 |

-------------                          |

------->  | INITIATOR | <-----------             |

|         -------------            |             v

|              ^                   |        ------------

|              |                   |        |  DISPLAY |

|              |                   |        |   DATA   |

|              |                   |        ------------

v              v                   v              ^

-----------     ------------       ----------          |

|REGISTERS|<--->|DISCONNECT| <---> | MODIFY |          |

-----------     ------------       | SET UP |          |

^               ^              ----------          |

|               |                   ^              |

|               v                   |              |

|           ----------              |              |

|---------> | TARGET | <------------|              |

----------                             |

^                                 |

|_________________________________|

The next couple of pages show the different menus and the

applicable commands and functions of each.

SDIAG PG. 5

The screen is split up into four sections. The top area is used

to display the available options. The smaller box areas on the

right are used to report bus phase conditions and interrupt

requests.  The bottom area is used for command query questions

and keyboard entry.

The phase box generally identifies the next requested phase.

The Interrupt box provides information on the interrupt received

and sometimes contains the number of bytes transferred if

applicable.

The dialog portion where keyboard entry of commands and data

takes place is at the bottom of the screen/menu.

SDIAG PG. 6

4. DISCONNECT MENU

- SBIC Commands -                     - Other Commands -

0.   Reset SBIC                         R.  Read/Write SBIC Reg.

1.   Abort Current SBIC Command         M.  Modify SBIC Setup

5.   Reselect an Initiator              S.  Read Status

6.   Select a Target With ATN           I.  Goto Initiator Menu

7.   Select a Target W/O ATN            T.  Goto Target Menu

8.   Select With ATN and Transfer       L.  Load and run file

9.   Select W/O ATN and Transfer        Q.  Enable interrupts

A.   Reselect and Receive Data          X.  Exit

B.   Reselect and Send Data          -------------- PHASE ------

C.   Wait for Select and Receive    |

F.   Set IDI bit                    |

18.  Translate Address              --------------- INTERRUPT ---

H.   Hard SCSI Bus Reset            |

|

|

-----------------------------------------------------------------

The Disconnect menu assumes that the SBIC is in a SCSI Bus free

state.  This will be the first menu that is seen and a Reset

command (0) must be entered along with the desired SCSI ID. All

other parameters under the reset command can be changed or the

default parameter selected, depending on the SBIC and features

that are needed.

Commands 1,5-9,A,B,C,F and 18 are all direct SBIC commands and

correspond to those seen on pg. 23 of the WD33C93A Data Sheet.

Command H is used for a hard reset and is implemented externally.

The WDSCSATXT does not implement a hard reset.

The "other commands" are used for a variety of things. R,M,I,T

give direct access to there respective menus.

Command X will exit from SDIAG to DOS.

Command S simply reads the Auxiliary Status Reg. and the SCSI

Status Reg.

The 'L' option (Load batch file and execute) asks for the name of

the file and executes the character command options as it would

from a keyboard. The 'batch' file is simply a list of keystrokes

that the user would normally type into the keyboard. It is a text

file and can be edited with DOS EDLIN or any other editor that

doesn't introduce special characters.

SDIAG PG. 7

Again, two examples of these batch files are included on the

diskette (I1 and T1).

The enable interrupts option in the Disconnect menu ('Q'), allows

the interrupt signals generated by the SBIC to reach the host PC

system.  Since SDIAG polls the SBIC Aux Status register for

interrupt information, it does not need the hardware interrupts.

The hardware interrupt signal may be used in conjunction with a

logic analyzer for performance/timing measurements.

SDIAG PG. 8

5. INITIATOR MENU

- SBIC Commands -                      - Other Commands -

0.   Reset SBIC                        R. Read/Write SBIC Reg.

1.   Abort Current SBIC Command        M. Modify SBIC Setup

2.   Assert ATN                        P. Poll Data Register

3.   Negate ACK                        S. Read Status

4.   Disconnect SBIC From SCSI Bus     Z. Display Data

F.   Set IDI bit                       L. Load and run file

20.   Transfer Info                     X. Exit

21.   Transfer Pad                  ---------------  PHASE  -----

|

|

--------------- INTERRUPT ---

|

|

|

-----------------------------------------------------------------

The Initiator and Target menus are used when an initiator or

target command is executed, except for those combinational

commands and initial commands which are executed from the

disconnect menu.  For SAT commands the disconnect menu will

remain until the command is complete.

In certain situations, the user may need to go to the Initiator

or Target menu. A warning is shown on the screen that the SBIC

may or may not be connected that way. Disregard this if the menu

coincides with the correct mode.

Although command "0" appears in this menu, it is not common to

issue a reset when in this mode. If a reset is to be implemented

from the Initiator menu it would usually occur in a hang

condition.

Command 21, Transfer pad, can not be implemented with the

WD33C93A SBIC but will function properly with the WD33C93. it

simply repeats the value in the Data Reg. until the transfer is

complete.

Commands R,M,S,L,X are the same as discribed in the Disconnect

Menu.

Command P accesses the Data Register and displays the value held

in that register.

Command Z will access the Data Display Menu and allow several

options for data handling.

SDIAG PG. 9

6. TARGET MENU

- SBIC commands -                      - Other Commands -

0.  Reset SBIC                          R.  Read/Write SBIC Reg.

1.  Abort Current SBIC Command          M.  Modify SBIC Setup

4.  Disconnect SBIC From SCSI Bus       P.  Poll Data Register

D.  Send Status & Command Complete      S.  Read Status

E.  Send Disconnect Message             Z.  Display Data

F.  Set IDI bit                         L.  load and run file

10.  Receive Command                     X.  Exit

11.  Receive Data

12.  Receive Message Out            ---------------  PHASE  -----

13.  Receive Unspecified Info Out   |

14.  Send Status                    |

15.  Send Data                      --------------- INTERRUPT ---

16.  Send Message In                |

17.  Send Unspecified Info In       |

18.  Translate Address              |

-----------------------------------------------------------------

Command "0" should only be issued under a reset or hang condition

from the Target Menu.

Commands 1,4, D-F, 10-18 are all typical SBIC commands and

correspond to pg. 23 in the WD33C93A Data Sheet.

Commands D, E, F can only be implemented in the WD33C93A.

All other commands operate as described in the Disconnect and

Initiator Menus.

SDIAG PG. 10

7. SBIC REGISTERS TO READ/WRITE

0. SBIC ID register        = 00  D. Cyl Number (9MSB) /CDB11 = 00

1. Control register        = 00  E. Cyl Number (LSB) /CDB12  = 00

2. Timeout period          = 00  F. Target LUN               = 00

3. Total sectors /CDB1     = 00  10. Command Phase Register  = 00

4. Total heads /CDB2       = 00  11. Sync. Transfer Reg.     = 00

5. Total cyls (MSB) /CDB3  = 00  12. Transfer Count (MSB)    = 00

6. Total cyls (LSB) /CDB4  = 00  13. Transfer Count (2nd)    = 00

7. Logical adr (MSB) /CDB5 = 00  14. Transfer Count (LSB)    = 00

8. Logical adr (2nd) /CDB6 = 00  15. Destination ID          = 00

9. Logical adr (3rd) /CDB7 = 00  16. Source ID               = 00

A. Logical adr (LSB) /CDB8 = 00  17. SCSI Status (RD ONLY)   = 00

B. Sector number /CDB9     = 00  18. Command Register        = 00

C. Head number /CDB10      = 00  R. Read Data Registers

S. Aux. status (RD ONLY)   = 00  W. Write Data Register

X. Exit read/write menu

Enter selection number (hex):

----------------------------------------------------------------

The Registers menu can be accessed from the Disconnect, Initiator

and Target menus and can be used to actually enter commands for

more explicit operation.

For more indepth operation, a SBIC command can be initiated and

finished in the SBIC Register Menu.  This is done by entering the

correct value in each register necessary to implement the desired

command.  This operation takes much longer, but supplies a step-

by-step operation with great detail. There are, however, some

conditions which should be noted.  First, when the chip is in

operation, all registers except Status Reg. will contain (FF)h.

After the Interrupt bit is set in the Aux Status Register, the

pending interrupt will be seen as an XX in the SCSI Status

Register. Once the return key is entered the pending interrupt

will be seen in the SCSI Status Reg.

All registers correspond to the registers in the SBIC and can be

seen in the table on pg.6 in the WD33C93A Data sheet.

To program a register, simply enter the register number then

enter the desired value. You must use returns for each value.

Command X will return the program to the menu from which the

register menu was entered.

SDIAG PG. 11

8. SCSI SETUP OPTIONS

1. DMx  - Polled I/O

2. EDI  - Ending Disconnect Interrupt enabled.

3. IDI  - Intermediate Disconnect Interrupt enabled.

4. HA   - Halt on Attention disabled.

5. HSP  - Halt on SCSI Parity Error enabled.

6. HHP  - Halt on Host Parity Error enabled.

7. TPx  - Transfer period is X cycles.

8. OFx  - Offset is X.

9. ER   - Reselection is disabled.

A. ES   - Selection is enabled.

B. DSP  - Selection/Reselection parity is disabled.

C. Timeout Period is X (hex) or X msec

Enter selection # to change setup or x to exit

----------------------------------------------------------------

The Modify Setup menu is used to change various bits in the SBIC

registers.  This could be done in the Register Access menu, but

it is easiest for the user to use the Modify Setup menu, since

one does not have to remember the exact bit position.

The setup options menu, which is accessed via the Modify SCSI

Setup option in the 3 main menus, can be used to implement

specific characteristic in the testing scenario.

Option 1, DMx, allows the user to change the host transfer mode

from polled I/O to DMA.  This option also allows WD-Bus and Burst

DMA, but these options were not implemented on the ATXT nor the

WDSCSXTS. They can, however, be implemented in a custom design.

Options 2-6 and 9-B toggle upon selecting the corresponding

number (hexadecimal).

TPx and OFx program the Sync. Transfer Register with the period

and offset, respectively.

Option C programs the timeout period.

SDIAG PG. 12

9. DATA DISPLAY OPTIONS

1. Display receive data

2. Display transmit data

3. Compare receive and transmit data

4. Clear receive data buffer

5. Specify data buffer length

x. Exit

Enter selection:

-----------------------------------------------------------------

In the Data Display menu, an option exists (option 5) to specify

the buffer length. Normally, this is done automatically from the

information entered in the other menus, but situations may exist

when the user wants to alter the buffer length, such as, in the

clear receive buffer option.

Generally, the user will use command - #1, however, the other

options can be used as well.

Command X returns the program to the menu from which the Data

menu was entered.

SDIAG PG. 13

10. COMMAND EXECUTION

When a command is entered, the information that the SDIAG program

needs to run the command at the chip level, is asked in a

scrolling manner in the dialog portion of the menu. It appears

at the bottom of the screen and moves upward as the next question

appears.  The following example illustrates the typical data

entry for a Select and Transfer command.  As the questions are

answered they scroll up and disappear.

Enter command:  8

Enter destination ID:  0

Enter target LUN {0} :

Enter # of command bytes to transmit <6,10 or 12>: {6}

Enter command byte 0: 08

Enter command byte 1: 00

Enter command byte 2: 00

Enter command byte 3: 00

Enter command byte 4: 00

Enter command byte 5: 01

Is data transfer required <y,n> {n} : y

Is data bytes to be transmitted <y, n> {n} : y

Enter number of data bytes to be transmitted: 512

Transmit data pattern options:

1. 0's           5. 55's

2. FF's          6. 6DB pattern

3. Increment     7. Received data

4. AA's          8. User Specified data

Enter selection: 3

Is continuous loop required <y,n> {n} : n

Once the disconnect menu on the screen is viewed it will be

obvious that this entire command sequence will not fit on the

allowed portion of the screen.

On some commands, a continuous loop option is available as seen

on the last line of the previous example.  This allows the

command to be repeated until 'ESC' is pressed. Generally, the

user would not enable this option. It can be ignored by pressing

the Return key to select the default value: 'NO'.

To exit the program or abort any selection, the 'ESC' key may be

pressed before the final carriage return.

SDIAG PG. 14

11. IMPORTANT POINTS

SDIAG is menu driven and therefore sometimes requires a return

keystroke entered to provide an interrupt on screen.  If it seems

to hang, enter a return.

Don't be afraid to try new things, a power down and back up

should reset the system if a hang up condition occurs.  Most

illegal possibilities are denoted so via SDIAG when issued.

There could be some programming errors in SDIAG, so if you think

you may have found one please contact W.D. Apps. department at

714-474-2033 x 37862.

JLSDIAG

SDIAG PG.15

12.PROGRAMMING EXAMPLES

Keystrokes for a read using Select and Transfer w/ATN at Initiator and

Wait for Select and Receive at Target. Both cases, Disconnect-

Reselection and No Reselection are provided.

> represents carriage return keystroke, I=Initiator, T=Target

----------------------------------------------------------------------

Both Cases|No Reselection|Reselection |Reselection |

|              |            |   Cont.    |

I     T   | I        T   | I       T  | I       T  |

|FROM POINT A  |FROM POINT A|FROM POINT B|

M         |          15  |        16  |  13 @      |@ENTER 14 HERE FOR

>         |           >  |         >  |   >        | 2 BYTE TRANSFER

9         |          512 |        02  |  02        | INSTEAD OF 512.

>         |           >  |         >  |   >        |

X         |           3  |   >        |  18        |

>         |           >  |   3        |   >        |

C   |           >  |   >     >  |  09        |

>   |           14 |        16  |   >        |

>   |           >  |         >  |         15 |

8         |           00 |        04  |          > |

>         |           >  |         >  |        512 |*THE DATA TRANSFER

0         |           >  |   >        |          > |SIZE CAN BE

>         |           16 |   >        |          3 |REDUCED TO A

>         |           >  |  20        |          > |SMALLER VALUE, SO

>         |           00 |   >        |   >        |THAT THE R,>,>

08        |           >  |   >        |   R *      |DOESN'T HAVE TO

>         |           >  |   >        |   > *      |BE REPEATED 512

00        |   >          |   3        |   > *      |TIMES. ALL VALUES

>         |   1          |   >        |         14 |OF 512 CAN BE

00        |   >          |         >  |          > |CHANGED TO 02, FOR

>         |   >          |         4  |         00 |EXAMPLE.

00        |   X          |         >  |          > |

>         |   >          |   >        |          > |

01        |              |   >        |         16 |

>         |           4  |         5  |          > |

00        |           >  |         >  |         00 |

>         |   >          |         7  |          > |

Y         |   >          |         >  |          > |

>         |   DISCONNECT |         >  |  >         |

>         |              |  >         |  >         |

512       |              |  >         |          4 |

>         |              |  R         |          > |

>         |              |  >         |  >         |

>  |              | 10         |  >         |

>  |              |  >         | DISCONNECT |

--------- |              | 41         |            |

POINT A  |              |  >         |            |

--------- |              |--------    |            |

|              |POINT B     |            |

|              |--------    |            |