/----------------------------------------------------------------------------\

|									     |

|		         ---------------------------- 			     |

|		         IRWIN MAGNETIC SYSTEMS, INC.			     |

|		         ----------------------------			     |

|									     |

|   File  : EXTAPE.DOC						             |

|									     |

|   Title : Notes on the Tape Exercise Program (4.87)                        |

|									     |

|   By    : Applications Engineering Dept.                                   |

|									     |

|   Date  : February 8, 1985         				             |

|									     |

|									     |

\----------------------------------------------------------------------------/

----------------------------

|         Purpose          |--------------------------------------------------

----------------------------

This program "exercises" the tape and provides a means of testing the Irwin

Magnetics tape drive. Test areas include data transfer, tape motion and

positioning, cartridge status, and other hardware functions.

----------------------------

| Operational Instructions |--------------------------------------------------

----------------------------

To run the exerciser, enter the MS-DOS command:   C>EXTAPE

The exerciser will print an identifying message and display the prompt ">> ",

indicating that it is ready to accept commands.

Commands to the exerciser consist of SINGLE words or their numerical

abbreviations.  Additional parameters will be called with a message. Upper

case and lower case are treated as equivalent. A string of commands may be

given, separated by blanks.

------------------------------------------------------------------------------

The commands currently recognized are:                 pulses

(1) ISBUSY          return busy status               0

(2)	STOP		stop tape                        2

(3)	PAUSE		stop, reverse 4 blocks, stop     3

(4)	LOADPT		seek load point                  4

(5)	EOT		move physically forward          5

(6) BOT		move physically reverse          6

(7)	ISNORMAL	report normal completion         7

(8)	ISDRIVE		report drive presence            8

(9)	ISEOT		report EOT status                9

(10)	ISBOT		report BOT status                10

(11)	ISCART  	report cartridge presence        11

(12)	ISTRACK		report track found               12

(13)	ISNEWCART	report new cartridge             13

(14) REVERSE         move logically reverse           14

(15)	FORWARD		move logically forward		 15

(16)	ONPULSE         turn on 2nd pulse                16

(17)	OFFPULSE        turn off 2nd pulse               17

(20)	SEEK 		seek track n (0<= n <=7)         20 + n

(31) SERVO           write servo                      31

(32) RECAL		Send recal to controller.	 32

(38) PULSE           Send n pulses to controller      n  (0<= n <=255)

(The above commands all invoke tp0comm to send the requisite number

of stepping pulses.)

(18)	ID              Read cylinder and sector id from block (b).

(19)	LOCATE	        Move logically forward and read first available id.

(21) FORMAT          Format track (k). Writes fake badblock map if track 0

is formatted.

(22) ISFORMAT	Report format status of cartridge.

(23)	ISPROT		Report write protect status of cartridge.

(24) ISSERVO		Report servo status of cartridge.

(25)	WRITE 		Write block (b). Offers 2 filler styles.

(26)	READ            Read block (b). Can be used with PEEK (n) (s).

(n = 64 displays an entire block.)

(27)	PEEK		Display RAM. Calls for number (n) of 128-byte records

to display, and starting record (s).  Display in ASCII

and hex.

(28) WRITESECT       Write block (b), sector (n). Calls for a digit to

write.

(29) READSECT        Read block (b), sector (n). Calls for a digit to

verify read. Used with WRITESECT.  Can be used with

PEEK.

(30)	SHOWSECT        Display contents of block (b), sector (n) on the

screen. Choice of display format.

(33)	REREAD		Read block (b). Calls for the number of repetitions.

(34)	RETRY 		Read track (n), report bad blocks.

(35) WRITEONE  	Write one byte.

(36)	SETDRIVE	Set tape drive line number.

(37)	READMAP	        Read badblock map from a formatted cartridge. Return

bad block locations.

(39) WAIT		Wait until drive not busy (for use with a string of

commands).

HELP            List commands and their numerical abbreviations.

PARAM		Set parameters (tracks/tape, blocks/track, etc.)

EXIT            Return to MS-DOS.

----------------------------

| Error and Info. Messages |--------------------------------------------------

----------------------------

TP0COMM return codes

0:     O.K.

1:     Command not accepted

2:	Receive timeout

3:	Send timeout

4:	Bad NEC chip

5:	Record not found

6:	CRC error

7:	DMA error

8:	Write protect

9:	ID mark error

10:	Interrupt timeout

11:	DMA boundary error

##################################################################