#ident "@(#)snmp.doc	13.3	90/08/08"

1. Introduction

The following describes a Management Information Base (MIB) tailored

to Network Computing Devices (NCD) X Display Stations. (Although,

much of the MIB could be applied to non-NCD devices.)  This MIB

conforms to the standards set down in RFC1155, and is written in the

style of RFC1158. This MIB is an adjunct to the one specified in

RFC1158. It was written to provide the same level of network

management information via SNMP (or CMOT) as that level which is

currently available on an NCD Display Station via local mechanisms.

2. Object Definitions

2.1 System Group

The System Group contains basic information on the unit.

OBJECT:

-------

ncdSysMemTotal { ncdSystem 1 }

Syntax:

INTEGER

Definition:

The total amount of RAM installed in the system. This does

not include special purpose RAM such as video memory.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSysMemAvail { ncdSystem 2 }

Syntax:

Gauge

Definition:

The amount of RAM memory which is currently unallocated.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSysMemFrags { ncdSystem 3 }

Syntax:

Gauge

Definition:

The number of discontiguous blocks of unallocated memory.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSysIdleTime { ncdSystem 4 }

Syntax:

TimeTicks

Definition:

The estimated amount of time since boot that the system

was not executing any useful task.

Access:

read-only

Status:

optional

2.2 Mbuf Group

The Mbuf Group provides information on the memory buffers used by

the network protocol stacks for dynamic storage needs. Note that

protocol stacks other than TCP/IP may be running on the unit and

will consume mbuf resources. These are the same type of buffers

as used by the BSD TCP/IP implementation.

OBJECT:

-------

ncdMbufTotal { ncdMbuf 1 }

Syntax:

INTEGER

Definition:

The total number of mbufs that currently exist in the system.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufFree { ncdMbuf 2 }

Syntax:

Gauge

Definition:

The number of mbufs which are unallocated.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufClusterTotal { ncdMbuf 3 }

Syntax:

INTEGER

Definition:

The total number of clusters that currently exist in the

system.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufClusterFree { ncdMbuf 4 }

Syntax:

Gauge

Definition:

The number of clusters which are unallocated.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufReserved { ncdMbuf 5 }

Syntax:

Gauge

Definition:

The number of mbufs that are reserved, but as yet

unallocated, for storing user data within the protocol

stacks.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufDrops { ncdMbuf 6 }

Syntax:

Counter

Definition:

The number of times the networking code was unable to

allocate an mbuf, and initiated error recovery procedures

due to this failure.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufWaits { ncdMbuf 7 }

Syntax:

Counter

Definition:

The number of times the networking code had to block waiting

for mbufs to become available.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufData { ncdMbuf 8 }

Syntax:

Gauge

Definition:

The number of mbufs currently allocated to storing user data

within the protocol stacks.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufHeader { ncdMbuf 9 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing protocol header

information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufSocket { ncdMbuf 10 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing BSD socket

information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufProtoCtrlBlock { ncdMbuf 11 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing internal

protocol information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufRouteTable { ncdMbuf 12 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing routing table

information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufAtTable { ncdMbuf 13 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing address

translation information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufSocketName { ncdMbuf 14 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing BSD socket name

information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufSocketOptions { ncdMbuf 15 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing BSD socket

option information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufFrags { ncdMbuf 16 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing IP fragments.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufInterfaceAddr { ncdMbuf 17 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing network

interface address information.

Access:

read-only

Status:

optional

2.3 Ethernet Group

The Ethernet Group contains information on the unit's ethernet

option card which uses the AMD LANCE controller.

OBJECT:

-------

ncdEtherTable { ncdEther 1 }

Syntax:

SEQUENCE OF NcdEtherEntry

Definition:

A table containing Ethernet interface information.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdEtherEntry { ncdEtherTable 1 }

Syntax:

NcdEtherEntry ::= SEQUENCE {

ncdEtherInCrcErrors,

Counter,

ncdEtherInMissed,

Counter,

ncdEtherInFrameErrors,

Counter,

ncdEtherSingleCollisions,

Counter,

ncdEtherMultipleCollisions,

Counter,

ncdEtherLateCollisions,

Counter,

ncdEtherOutDeferred,

Counter,

ncdEtherOutRetryErrors,

Counter,

ncdEtherLostCarrier,

Counter,

ncdEtherRestarts,

Counter

}

Definition:

Information about an Ethernet interface.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdEtherInCrcErrors { ncdEtherEntry 1 }

Syntax:

Counter

Definition:

The number of times a incoming frame failed the

Cyclic Redundancy Check (CRC).

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherInMissed { ncdEtherEntry 2 }

Syntax:

Counter

Definition:

The number of times an incoming frame was not received

due to a lack of buffers.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherInFrameErrors { ncdEtherEntry 3 }

Syntax:

Counter

Definition:

The number of times an incoming frame had a bad bit

alignment.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherSingleCollisions { ncdEtherEntry 4 }

Syntax:

Counter

Definition:

The number of times an outbound frame had a single

collision occur during transmission.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherMultipleCollisions { ncdEtherEntry 5 }

Syntax:

Counter

Definition:

The number of times an outbound frame had multiple

collisions occur during transmission.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherLateCollisions { ncdEtherEntry 6 }

Syntax:

Counter

Definition:

The number of times the collision signal was asserted

after the preamble of a frame was transmitted.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherOutDeferred { ncdEtherEntry 7 }

Syntax:

Counter

Definition:

The number of times an outbound frame could not be

transmitted directly due to the Ethernet being busy,

and the Ethernet exponentail backoff algorithm was

invoked.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherOutRetryErrors { ncdEtherEntry 8 }

Syntax:

Counter

Definition:

The number of times an outbound frame could not be

transmitted after 16 backed-off attempts due to

repeated collisions on the Ethernet.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherLostCarrier { ncdEtherEntry 9 }

Syntax:

Counter

Definition:

The number of times carrier (Receiver Enable) was lost

during the transmission of a frame.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherRestarts { ncdEtherEntry 10 }

Syntax:

Counter

Definition:

The number of times the software has restarted the

Ethernet controller, possibly due to a heuristic for

determining an inoperable controller.

Access:

read-only

Status:

optional

2.4 Serial Group

The Serial Group contains information on the unit's auxilary

RS-232 port(s) which can be used for a terminal session, a network

interface, a debug session, or a printer daemon.

OBJECT:

-------

ncdSerialTable { ncdSerial 1 }

Syntax:

SEQUENCE OF NcdSerialEntry

Definition:

A table containing serial interface information.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdSerialEntry { ncdSerialTable 1 }

Syntax:

NcdSerialEntry ::= SEQUENCE {

ncdSerialInOverruns,

Counter,

ncdSerialInFrameErrors,

Counter,

ncdSerialInParityErrors,

Counter,

ncdSerialInBreakErrors,

Counter

}

Definition:

Information about a serial interface.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdSerialInOverruns { ncdSerialEntry 1 }

Syntax:

Counter

Definition:

The number of times an incoming octet overwrote a

previously received octet in the UART's buffer.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSerialInFrameErrors { ncdSerialEntry 2 }

Syntax:

Counter

Definition:

The number of times an incoming octet had bad bit

alignment.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSerialInParityErrors { ncdSerialEntry 3 }

Syntax:

Counter

Definition:

The number of times the parity bit of an incoming

octet was incorrect.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSerialInBreakErrors { ncdSerialEntry 4 }

Syntax:

Counter

Definition:

The number of times a stop bit was not received at the

expected time after a start bit, and the intervening

bits were all zero.

Access:

read-only

Status:

optional

2.5 TCP Group

The TCP Group contains additional information on TCP beyond that

defined by RFC 1066.

OBJECT:

-------

ncdTcpRetransDrops { ncdTcp 1 }

Syntax:

Counter

Definition:

The number of connections progressing directly to the

CLOSED state due to too many retransmissions being sent.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTcpOutKeepAlives { ncdTcp 2 }

Syntax:

Counter

Definition:

The number of TCP Keep Alive packets transmitted.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTcpKeepAliveDrops { ncdTcp 3 }

Syntax:

Counter

Definition:

The number of connections progressing directly to the

CLOSED state due to too many keep alives being sent.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTcpInAcks { ncdTcp 4 }

Syntax:

Counter

Definition:

The number of TCP packets received which acknowledged

1 or more octets of unacknowledged data.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTcpOutBareAcks { ncdTcp 5 }

Syntax:

Counter

Definition:

The number of TCP packets transmitted which contained

no data, and only the ACK flag turned on.

Access:

read-only

Status:

optional

2.6 TFTP Group

The TFTP Group contains information on the TFTP file transfer

client that is used to download fonts, and read or write remote

configuration files.

OBJECT:

-------

ncdTftpClientReadReqs { ncdTftp 1 }

Syntax:

Counter

Definition:

The number of file read requests made by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientWriteReqs { ncdTftp 2 }

Syntax:

Counter

Definition:

The number of file write requests made by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientInDataPkts { ncdTftp 3 }

Syntax:

Counter

Definition:

The number of TFTP Data packets received by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientInAcks { ncdTftp 4 }

Syntax:

Counter

Definition:

The number of TFTP Ack packets received by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientInNoFileErrors { ncdTftp 5 }

Syntax:

Counter

Definition:

The number of times an TFTP Error packet was received

by the TFTP client with the error code set to

"File not found".

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientInAccessErrors { ncdTftp 6 }

Syntax:

Counter

Definition:

The number of times an TFTP Error packet was received

by the TFTP client with the error code set to

"Access violation".

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientInDiskFullErrors { ncdTftp 7 }

Syntax:

Counter

Definition:

The number of times an TFTP Error packet was received

by the TFTP client with the error code set to "Disk full".

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientNoBufs { ncdTftp 8 }

Syntax:

Counter

Definition:

The number of times the TFTP client failed to allocate

a buffer.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientOutDataPkts { ncdTftp 9 }

Syntax:

Counter

Definition:

The number of TFTP Data packets sent by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientOutAcks { ncdTftp 10 }

Syntax:

Counter

Definition:

The number of TFTP Ack packets sent by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientRetrans { ncdTftp 11 }

Syntax:

Counter

Definition:

The number of times a TFTP packet was retransmitted by

the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientRetries { ncdTftp 12 }

Syntax:

INTEGER

Definition:

The number of retransmissions of a TFTP packet sent

before an error situation is declared.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdTftpClientTimeout { ncdTftp 13 }

Syntax:

TimeTicks

Definition:

The amount of time between retransmissions of a TFTP

packet.

Access:

read-write

Status:

optional

2.7 NFS Group

The NFS Group contains information on the NFS file transfer

client that is used to download fonts, and read or write remote

configuration files.

OBJECT:

-------

ncdNfsClientNulls { ncdNfs 1 }

Syntax:

Counter

Definition:

The number of NFS Null operations requested by the

NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientGetAttributes { ncdNfs 2 }

Syntax:

Counter

Definition:

The number of NFS Get Attribute operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientSetAttributes { ncdNfs 3 }

Syntax:

Counter

Definition:

The number of NFS Set Attribute operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientGetRoots { ncdNfs 4 }

Syntax:

Counter

Definition:

The number of NFS Get File System Root operations

requested by the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientLookups { ncdNfs 5 }

Syntax:

Counter

Definition:

The number of NFS File Name Component Lookup

operations requested by the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientReadLinks { ncdNfs 6 }

Syntax:

Counter

Definition:

The number of NFS Read Link operations requested

by the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientReads { ncdNfs 7 }

Syntax:

Counter

Definition:

The number of NFS Read operations requested by the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientWriteCaches { ncdNfs 8 }

Syntax:

Counter

Definition:

The number of NFS Write Cache operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientWrites { ncdNfs 9 }

Syntax:

Counter

Definition:

The number of NFS Write operations requested by the NFS

client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientCreates { ncdNfs 10 }

Syntax:

Counter

Definition:

The number of NFS File Create operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientRemoves { ncdNfs 11 }

Syntax:

Counter

Definition:

The number of NFS File Remove operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientRenames { ncdNfs 12 }

Syntax:

Counter

Definition:

The number of NFS File Rename operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientCreateLinks { ncdNfs 13 }

Syntax:

Counter

Definition:

The number of NFS Create Link operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientCreateSymlinks { ncdNfs 14 }

Syntax:

Counter

Definition:

The number of NFS Create Symbolic Link operations

requested by the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientMakeDir { ncdNfs 15 }

Syntax:

Counter

Definition:

The number of NFS Create Directory operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientRemoveDir { ncdNfs 16 }

Syntax:

Counter

Definition:

The number of NFS Remove Directory operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientReadDir { ncdNfs 17 }

Syntax:

Counter

Definition:

The number of NFS Read Directory operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientStatFileSys { ncdNfs 18 }

Syntax:

Counter

Definition:

The number of NFS Stat File System operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsMountTable { ncdNfs 19 }

Syntax:

SEQUENCE OF NfsMountEntry

Definition:

A table containing NFS Client file system mounting

information.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdNfsMountEntry { ncdNfsMountTable 1 }

Syntax:

NcdNfsMountEntry ::= SEQUENCE {

ncdNfsMountEntryIndex

INTEGER,

ncdNfsMountEntryLocalDir

OCTET STRING,

ncdNfsMountEntryServerAddr

IpAddress,

ncdNfsMountEntryServerDir

OCTET STRING,

ncdNfsMountEntryReadSize

INTEGER (1..8192),

ncdNfsMountEntryWriteSize

INTEGER (1..8192),

ncdNfsMountEntryRetries

INTEGER,

ncdNfsMountEntryTimeout

INTEGER

}

Definition:

Information about an NFS mounted file system on the

NFS Client. An object of this type is transient, in

that the file system can be unmounted.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdNfsMountEntryIndex { ncdNfsMountEntry 1 }

Syntax:

INTEGER

Definition:

A unique value for each mounted file system. The value

must remain constant during the period of time that the

file system remains mounted.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsMountEntryLocalDir { ncdNfsMountEntry 2 }

Syntax:

OCTET STRING

Definition:

The local directory name of the mount point of the NFS

file system represented by this entry.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsMountEntryServerAddr { ncdNfsMountEntry 3 }

Syntax:

NetworkAddress

Definition:

The IP address of the NFS file server for this entry.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsMountEntryServerDir { ncdNfsMountEntry 4 }

Syntax:

OCTET STRING

Definition:

The directory name of the file system on the file server

for this entry.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsMountEntryReadSize { ncdNfsMountEntry 5 }

Syntax:

INTEGER

Definition:

The preferred size of read requests sent by the client

to the NFS server.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdNfsMountEntryWriteSize { ncdNfsMountEntry 6 }

Syntax:

INTEGER

Definition:

The preferred size of write requests sent by the client

to the NFS server.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdNfsMountEntryRetries { ncdNfsMountEntry 7 }

Syntax:

INTEGER

Definition:

The number of retransmissions of an NFS request sent before

an error situation is declared by the RPC layer.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdNfsMountEntryTimeout { ncdNfsMountEntry 8 }

Syntax:

TimeTicks

Definition:

The amount of time between retransmissions of an NFS

request used by the RPC layer.

Access:

read-write

Status:

optional

3. Definitions

NCD-MIB-1 {iso org(3) dod(6) internet(1) private(4)

enterprises(1) ncd(82) ncd-mibs(2) 1 }

DEFINITIONS ::= BEGIN

IMPORTS

private, enterprises, OBJECT-TYPE, NetworkAddress,

IpAddress, Counter, Gauge, TimeTicks

FROM RFC1065-SMI;

ncd OBJECT IDENTIFIER ::= { enterprises 82 }

ncd-products OBJECT IDENTIFIER ::= { ncd 1 }

ncd16 OBJECT IDENTIFIER ::= { ncd-products 1 }

ncd19 OBJECT IDENTIFIER ::= { ncd-products 2 }

ncd17c OBJECT IDENTIFIER ::= { ncd-products 3 }

ncd16e OBJECT IDENTIFIER ::= { ncd-products 4 }

ncd-mibs OBJECT IDENTIFIER ::= { ncd 2 }

ncd-mibs-mib1 OBJECT IDENTIFIER ::= { ncd-mibs 1 }

ncdSystem OBJECT IDENTIFIER ::= { ncd-mibs-mib1 1 }

ncdMbuf OBJECT IDENTIFIER ::= { ncd-mibs-mib1 2 }

ncdEther OBJECT IDENTIFIER ::= { ncd-mibs-mib1 3 }

ncdSerial OBJECT IDENTIFIER ::= { ncd-mibs-mib1 4 }

ncdTcp OBJECT IDENTIFIER ::= { ncd-mibs-mib1 5 }

-- ncdUdp OBJECT IDENTIFIER ::= { ncd-mibs-mib1 6 }

ncdTftp OBJECT IDENTIFIER ::= { ncd-mibs-mib1 7 }

ncdNfs OBJECT IDENTIFIER ::= { ncd-mibs-mib1 8 }

-- NCD System Group

ncdSysMemTotal OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  optional

::= { ncdSystem 1 }

ncdSysMemAvail OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdSystem 2 }

ncdSysMemFrags OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdSystem 3 }

ncdSysIdleTime OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-only

STATUS  optional

::= { ncdSystem 4 }

-- NCD Mbuf Group

ncdMbufTotal OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 1 }

ncdMbufFree OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 2 }

ncdMbufClusterTotal OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 3 }

ncdMbufClusterFree OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 4 }

ncdMbufReserved OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 5 }

ncdMbufDrops OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 6 }

ncdMbufWaits OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 7 }

ncdMbufData OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 8 }

ncdMbufHeader OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 9 }

ncdMbufSocket OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 10 }

ncdMbufProtoCtrlBlock OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 11 }

ncdMbufRouteTable OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 12 }

ncdMbufAtTable OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 13 }

ncdMbufSocketName OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 14 }

ncdMbufSocketOptions OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 15 }

ncdMbufFrags OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 16 }

ncdMbufInterfaceAddr OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 17 }

-- NCD Ether Group

-- the ether interface table

ncdEtherTable OBJECT-TYPE

SYNTAX  SEQUENCE OF NcdEtherEntry

ACCESS  read-write

STATUS  optional

::= { ncdEther 1 }

ncdEtherEntry OBJECT-TYPE

SYNTAX  NcdEtherEntry

ACCESS  read-write

STATUS  optional

::= { ncdEtherTable 1 }

NcdEtherEntry ::= SEQUENCE {

ncdEtherInCrcErrors,

Counter,

ncdEtherInMissed,

Counter,

ncdEtherInFrameErrors,

Counter,

ncdEtherSingleCollisions,

Counter,

ncdEtherMultipleCollisions,

Counter,

ncdEtherLateCollisions,

Counter,

ncdEtherOutDeferred,

Counter,

ncdEtherOutRetryErrors,

Counter,

ncdEtherLostCarrier,

Counter,

ncdEtherRestarts,

Counter

}

ncdEtherInCrcErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 1 }

ncdEtherInMissed OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 2 }

ncdEtherInFrameErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 3 }

ncdEtherSingleCollisions OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 4 }

ncdEtherMultipleCollisions OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 5 }

ncdEtherLateCollisions OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 6 }

ncdEtherOutDeferred OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 7 }

ncdEtherOutRetryErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 8 }

ncdEtherLostCarrier OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 9 }

ncdEtherRestarts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 10 }

-- NCD Serial Group

-- the serial interface table

ncdSerialTable OBJECT-TYPE

SYNTAX  SEQUENCE OF NcdSerialEntry

ACCESS  read-write

STATUS  optional

::= { ncdSerial 1 }

ncdSerialEntry OBJECT-TYPE

SYNTAX  NcdSerialEntry

ACCESS  read-write

STATUS  optional

::= { ncdSerialTable 1 }

NcdSerialEntry ::= SEQUENCE {

ncdSerialInOverruns,

Counter,

ncdSerialInFrameErrors,

Counter,

ncdSerialInParityErrors,

Counter,

ncdSerialInBreakErrors,

Counter

}

ncdSerialInOverruns OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialEntry 1 }

ncdSerialInFrameErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialEntry 2 }

ncdSerialInParityErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialEntry 3 }

ncdSerialInBreakErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialEntry 4 }

-- NCD TCP Group

ncdTcpRetransDrops OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTcp 1 }

ncdTcpOutKeepAlives OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTcp 2 }

ncdTcpKeepAliveDrops OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTcp 3 }

ncdTcpInAcks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTcp 4 }

ncdTcpOutBareAcks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTcp 5 }

-- NCD TFTP Group

ncdTftpClientReadReqs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 1 }

ncdTftpClientWriteReqs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 2 }

ncdTftpClientInDataPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 3 }

ncdTftpClientInAcks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 4 }

ncdTftpClientInNoFileErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 5 }

ncdTftpClientInAccessErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 6 }

ncdTftpClientInDiskFullErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 7 }

ncdTftpClientNoBufs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 8 }

ncdTftpClientOutDataPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 9 }

ncdTftpClientOutAcks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 10 }

ncdTftpClientRetrans OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 11 }

ncdTftpClientRetries OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  optional

::= { ncdTftp 12 }

ncdTftpClientTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdTftp 13 }

-- NCD NFS Group

ncdNfsClientNulls OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 1 }

ncdNfsClientGetAttributes OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 2 }

ncdNfsClientSetAttributes OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 3 }

ncdNfsClientGetRoots OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 4 }

ncdNfsClientLookups OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 5 }

ncdNfsClientReadLinks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 6 }

ncdNfsClientReads OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 7 }

ncdNfsClientWriteCaches OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 8 }

ncdNfsClientWrites OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 9 }

ncdNfsClientCreates OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 10 }

ncdNfsClientRemoves OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 11 }

ncdNfsClientRenames OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 12 }

ncdNfsClientCreateLinks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 13 }

ncdNfsClientCreateSymlinks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 14 }

ncdNfsClientMakeDir OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 15 }

ncdNfsClientRemoveDir OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 16 }

ncdNfsClientReadDir OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 17 }

ncdNfsClientStatFileSys OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 18 }

-- the NFS client mount table

ncdNfsMountTable OBJECT-TYPE

SYNTAX  SEQUENCE OF NcdNfsMountEntry

ACCESS  read-write

STATUS  optional

::= { ncdNfs 19 }

ncdNfsMountEntry OBJECT-TYPE

SYNTAX  NcdNfsMountEntry

ACCESS  read-write

STATUS  optional

::= { ncdNfsMountTable 1 }

NcdNfsMountEntry ::= SEQUENCE {

ncdNfsMountEntryIndex

INTEGER,

ncdNfsMountEntryLocalDir

OCTET STRING,

ncdNfsMountEntryServerAddr

IpAddress,

ncdNfsMountEntryServerDir

OCTET STRING,

ncdNfsMountEntryReadSize

INTEGER (1..8192),

ncdNfsMountEntryWriteSize

INTEGER (1..8192),

ncdNfsMountEntryRetries

INTEGER,

ncdNfsMountEntryTimeout

INTEGER

}

ncdNfsMountEntryIndex OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  optional

::= { ncdNfsMountEntry 1 }

ncdNfsMountEntryLocalDir OBJECT-TYPE

SYNTAX  OCTET STRING

ACCESS  read-only

STATUS  optional

::= { ncdNfsMountEntry 2 }

ncdNfsMountEntryServerAddr OBJECT-TYPE

SYNTAX  NetworkAddress

ACCESS  read-only

STATUS  optional

::= { ncdNfsMountEntry 3 }

ncdNfsMountEntryServerDir OBJECT-TYPE

SYNTAX  OCTET STRING

ACCESS  read-only

STATUS  optional

::= { ncdNfsMountEntry 4 }

ncdNfsMountEntryReadSize OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  optional

::= { ncdNfsMountEntry 5 }

ncdNfsMountEntryWriteSize OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  optional

::= { ncdNfsMountEntry 6 }

ncdNfsMountEntryRetrans OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  optional

::= { ncdNfsMountEntry 7 }

ncdNfsMountEntryTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdNfsMountEntry 8 }

END

4. Identification of OBJECT instances for use with SNMP

/* TBD */

4.1 ncdEtherTable Object Type Names

The name of an ethernet interface entry, 'x', is an OBJECT IDENTIFIER

of the form 'a' such that 'a' is the value of that instance of ifIndex

object type associated with 'x'.

For each object type, 't', for which the defined name, 'n', has a

prefix of ncdEther, an instance, 'i', of 't' is named by

an OBJECT IDENTIFIER of the form 'n.y', where 'y' is the name of

the interface entry about which 'i' represents information.

For example, suppose one wanted to find the number of Ethernet late

collisions associated with the second interface. Accordingly,

ncdEtherEntry.ncdEtherLateCollisions.2 would identify the desired

instance.

4.2 ncdSerialTable Object Type Names

The name of an serial interface entry, 'x', is an OBJECT IDENTIFIER

of the form 'a' such that 'a' is the value of that instance of ifIndex

object type associated with 'x'.

For each object type, 't', for which the defined name, 'n', has a

prefix of ncdSerial, an instance, 'i', of 't' is named by

an OBJECT IDENTIFIER of the form 'n.y', where 'y' is the name of

the interface entry about which 'i' represents information.

For example, suppose one wanted to find the number of serial parity

errors associated with the third interface. Accordingly,

ncdSerialEntry.ncdSerialInParityErrors.3 would identify the desired

instance.

4.3 ncdNfsMountTable Object Type Names

The name of an NFS mount entry, 'x', is an OBJECT IDENTIFIER of the

form 'a' such that 'a' is the value of that instance of

ncdNfsMountEntryIndex object type associated with 'x'.

For each object type, 't', for which the defined name, 'n', has a

prefix of ncdNfsMountEntry, an instance, 'i', of 't' is named by

an OBJECT IDENTIFIER of the form 'n.y', where 'y' is the name of

the NFS mount entry about which 'i' represents information.

For example, suppose one wanted to find the IP address of the NFS

server associated with the second NFS mount table entry. Accordingly,

ncdNfsMountEntryServerAddr.2 would identify the desired instance.

5. SNMP Traps Generated

5.1 Cold Start Trap

This trap is generated when the SNMP agent in the NCD unit is first

initialized.

5.2 Authentication Failure Trap

This trap is generated when the SNMP agent detects that the

authentication information in an SNMP request packet is invalid.

Generation of this trap can be controlled through the

"snmpEnableAuthTraps" object.