/* $NCDId: @(#)snmp.doc,v 14.10 1992/11/17 09:59:37 mkm Exp $ */

1. Introduction

The following describes a Management Information Base (MIB) tailored

to Network Computing Devices (NCD) X Display Stations. (Although,

much of the MIB could be applied to non-NCD devices.)  This MIB

conforms to the standards set down in RFC-1155, and is written in the

style of RFC-1158. This MIB is an adjunct to the one specified in

RFC-1158.

This is the third version of the NCD MIB. The only differences from

the second version is that about twenty objects have been rendered

obsolete due to changes in the underlying services in the 3.0

version of NCDware. The STATUS field of these objects has been

changed from 'optional' to 'obsolete'.

Note that SNMP specifies that all time related values are

represented as the type 'TimeTicks' whose units are one-hundredths

(1/100) of a second. However, the timers as actually implemented

in the NCD software have differing resolutions ranging from one

(1) minute to one-seventieth (1/70) of a second. When a timer is

set via SNMP the NCD software automatically rounds down the new

value to the resolution of the timer. Each timer object definition

specifies the resolution of that timer.

2. Object Definitions

2.1 System Group

The System Group contains basic information on the unit.

OBJECT:

-------

ncdSysMemTotal { ncdSystem 1 }

Syntax:

INTEGER

Definition:

The total amount of RAM installed in the system. This does

not include special purpose RAM such as video memory.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSysMemAvail { ncdSystem 2 }

Syntax:

Gauge

Definition:

The amount of RAM memory which is currently unallocated.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSysMemFrags { ncdSystem 3 }

Syntax:

Gauge

Definition:

The number of discontiguous blocks of unallocated memory.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSysIdleTime { ncdSystem 4 }

Syntax:

TimeTicks

Definition:

The estimated amount of time since boot that the system

was not executing any useful task.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSysBootPromVersion { ncdSystem 5 }

Syntax:

OCTET STRING

Definition:

A textual description of the boot PROM's installed

in the unit.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSysKeyboardControllerVersion { ncdSystem 6 }

Syntax:

OCTET STRING

Definition:

A textual description of the keyboard controller

installed in the unit.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSysAdminStatus { ncdSystem 7 }

Syntax:

INTEGER {

running(1),

resetSession(2),

resetUnitOnLastClientClose(3),

resetUnit(4)

}

Definition:

The desired state of the system.

The normal state of the system is 'running'.

Setting the state to 'resetSession' causes all the unit's

current X.11 connections to be closed, and returns the

unit's X.11 server to it's initial state. The state of

the system will return to 'running' at this time.

Setting the state to 'resetUnitOnLastClientClose' will

cause the unit to wait until all its X.11 client

connections close, and then it will reset. If no X.11

client connections exist, the unit is reset immediately.

Once this state is set, in cannot be unset. This state

is useful for gracefully installing software upgrades.

Setting the state to 'resetUnit' will cause an immediate

reset after the SNMP response to the set is generated.

Once this state is set, in cannot be unset. This state

provides no notification to the unit's user, and therefore

should be used with extreme caution.

N.B. The ability to write this object is controlled by

the normal SNMP community string AND a switch in the

unit's configuration file. By default, this object is

rendered read-only. Enabling the SNMP reset feature

requires specific configuration by the unit's administrator.

Access:

read-write

Status:

optional

2.2 Mbuf Group

The Mbuf Group provides information on the memory buffers used by

the network protocol stacks for dynamic storage needs. Note that

protocol stacks other than TCP/IP may be running on the unit and

will consume mbuf resources. These are the same type of buffers

as used by the BSD TCP/IP implementation.

OBJECT:

-------

ncdMbufTotal { ncdMbuf 1 }

Syntax:

INTEGER

Definition:

The total number of mbufs that currently exist in the system.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufFree { ncdMbuf 2 }

Syntax:

Gauge

Definition:

The number of mbufs which are unallocated.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufClusterTotal { ncdMbuf 3 }

Syntax:

INTEGER

Definition:

The total number of clusters that currently exist in the

system.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufClusterFree { ncdMbuf 4 }

Syntax:

Gauge

Definition:

The number of clusters which are unallocated.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufReserved { ncdMbuf 5 }

Syntax:

Gauge

Definition:

The number of mbufs that are reserved, but as yet

unallocated, for storing user data within the protocol

stacks.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufDrops { ncdMbuf 6 }

Syntax:

Counter

Definition:

The number of times the networking code was unable to

allocate an mbuf, and initiated error recovery procedures

due to this failure.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufWaits { ncdMbuf 7 }

Syntax:

Counter

Definition:

The number of times the networking code had to block waiting

for mbufs to become available.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufData { ncdMbuf 8 }

Syntax:

Gauge

Definition:

The number of mbufs currently allocated to storing user data

within the protocol stacks.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufHeader { ncdMbuf 9 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing protocol header

information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufSocket { ncdMbuf 10 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing BSD socket

information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufProtoCtrlBlock { ncdMbuf 11 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing internal

protocol information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufRouteTable { ncdMbuf 12 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing routing table

information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufAtTable { ncdMbuf 13 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing address

translation information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufSocketName { ncdMbuf 14 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing BSD socket name

information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufSocketOptions { ncdMbuf 15 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing BSD socket

option information.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufFrags { ncdMbuf 16 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing IP fragments.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdMbufInterfaceAddr { ncdMbuf 17 }

Syntax:

Gauge

Definition:

The number of mbufs allocated to storing network

interface address information.

Access:

read-only

Status:

optional

2.3 Ethernet Group

The Ethernet Group contains information on the unit's ethernet

option card which uses the AMD LANCE controller.

OBJECT:

-------

ncdEtherTable { ncdEther 1 }

Syntax:

SEQUENCE OF NcdEtherEntry

Definition:

A table containing Ethernet interface information.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdEtherEntry { ncdEtherTable 1 }

Syntax:

NcdEtherEntry ::= SEQUENCE {

ncdEtherInCrcErrors

Counter,

ncdEtherInMissed

Counter,

ncdEtherInFrameErrors

Counter,

ncdEtherSingleCollisions

Counter,

ncdEtherMultipleCollisions

Counter,

ncdEtherLateCollisions

Counter,

ncdEtherOutDeferred

Counter,

ncdEtherOutRetryErrors

Counter,

ncdEtherLostCarrier

Counter,

ncdEtherRestarts

Counter

}

Definition:

Information about an Ethernet interface.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdEtherInCrcErrors { ncdEtherEntry 1 }

Syntax:

Counter

Definition:

The number of times a incoming frame failed the

Cyclic Redundancy Check (CRC).

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherInMissed { ncdEtherEntry 2 }

Syntax:

Counter

Definition:

The number of times an incoming frame was not received

due to a lack of buffers.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherInFrameErrors { ncdEtherEntry 3 }

Syntax:

Counter

Definition:

The number of times an incoming frame had a bad bit

alignment.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherSingleCollisions { ncdEtherEntry 4 }

Syntax:

Counter

Definition:

The number of times an outbound frame had a single

collision occur during transmission.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherMultipleCollisions { ncdEtherEntry 5 }

Syntax:

Counter

Definition:

The number of times an outbound frame had multiple

collisions occur during transmission.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherLateCollisions { ncdEtherEntry 6 }

Syntax:

Counter

Definition:

The number of times the collision signal was asserted

after the preamble of a frame was transmitted.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherOutDeferred { ncdEtherEntry 7 }

Syntax:

Counter

Definition:

The number of times an outbound frame could not be

transmitted directly due to the Ethernet being busy,

and the Ethernet exponentail backoff algorithm was

invoked.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherOutRetryErrors { ncdEtherEntry 8 }

Syntax:

Counter

Definition:

The number of times an outbound frame could not be

transmitted after 16 backed-off attempts due to

repeated collisions on the Ethernet.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherLostCarrier { ncdEtherEntry 9 }

Syntax:

Counter

Definition:

The number of times carrier (Receiver Enable) was lost

during the transmission of a frame.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdEtherRestarts { ncdEtherEntry 10 }

Syntax:

Counter

Definition:

The number of times the software has restarted the

Ethernet controller, possibly due to a heuristic for

determining an inoperable controller.

Access:

read-only

Status:

optional

2.4 Serial Group

The Serial Group contains information on the unit's auxilary

RS-232 port(s) which can be used for a terminal session, a network

interface, a debug session, or a printer daemon.

OBJECT:

-------

ncdSerialTable { ncdSerial 1 }

Syntax:

SEQUENCE OF NcdSerialEntry

Definition:

A table containing serial interface information.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdSerialEntry { ncdSerialTable 1 }

Syntax:

NcdSerialEntry ::= SEQUENCE {

ncdSerialInOverruns

Counter,

ncdSerialInFrameErrors

Counter,

ncdSerialInParityErrors

Counter,

ncdSerialInBreakErrors

Counter

}

Definition:

Information about a serial interface.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdSerialInOverruns { ncdSerialEntry 1 }

Syntax:

Counter

Definition:

The number of times an incoming octet overwrote a

previously received octet in the UART's buffer.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSerialInFrameErrors { ncdSerialEntry 2 }

Syntax:

Counter

Definition:

The number of times an incoming octet had bad bit

alignment.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSerialInParityErrors { ncdSerialEntry 3 }

Syntax:

Counter

Definition:

The number of times the parity bit of an incoming

octet was incorrect.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSerialInBreakErrors { ncdSerialEntry 4 }

Syntax:

Counter

Definition:

The number of times a stop bit was not received at the

expected time after a start bit, and the intervening

bits were all zero.

Access:

read-only

Status:

optional

2.5 TCP Group

The TCP Group contains additional information on TCP beyond that

defined by RFC 1066.

OBJECT:

-------

ncdTcpRetransDrops { ncdTcp 1 }

Syntax:

Counter

Definition:

The number of connections progressing directly to the

CLOSED state due to too many retransmissions being sent.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTcpOutKeepAlives { ncdTcp 2 }

Syntax:

Counter

Definition:

The number of TCP Keep Alive packets transmitted.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTcpKeepAliveDrops { ncdTcp 3 }

Syntax:

Counter

Definition:

The number of connections progressing directly to the

CLOSED state due to too many keep alives being sent.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTcpInAcks { ncdTcp 4 }

Syntax:

Counter

Definition:

The number of TCP packets received which acknowledged

1 or more octets of unacknowledged data.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTcpOutBareAcks { ncdTcp 5 }

Syntax:

Counter

Definition:

The number of TCP packets transmitted which contained

no data, and only the ACK flag turned on.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTcpConnectTimeout { ncdTcp 6 }

Syntax:

TimeTicks

Definition:

The time that must elapse between a TCP connection

attempt and a lack of a response for a failure situation

to be declared. The resolution of this timer is one-half

(1/2) of a second.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdTcpMaxRetrans { ncdTcp 7 }

Syntax:

INTEGER

Definition:

The maximum number of times TCP will retransmit data

on an established connection before a failure situation

is declared.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdTcpKeepAliveIdleTime { ncdTcp 8 }

Syntax:

TimeTicks

Definition:

The time that must elapse on an established TCP connection

with no activity before a TCP keepalive packet is generated.

The resolution of this timer is one-half (1/2) of a second.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdTcpKeepAliveInterval { ncdTcp 9 }

Syntax:

TimeTicks

Definition:

The time between succesive transmissions of TCP keepalive

packets when no response to the keepalive packets has been

received. The resolution of this timer is one-half (1/2)

of a second.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdTcpKeepAliveMaxIdle { ncdTcp 10 }

Syntax:

TimeTicks

Definition:

The maximum amount of time that TCP keepalives are

transmitted with no response before a failure situation

is declared. The resolution of this timer is one-half

(1/2) of a second.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdTcpDfltLingerTime { ncdTcp 11 }

Syntax:

TimeTicks

Definition:

The default time TCP continues to attempt reliable

transmission of outstanding data on the connection's

transmit queue after local software closes a connection.

This timer is optional and can be configure on a per

connection basis by the software by higher level software.

The resolution of this timer is one (1) second.

Access:

read-write

Status:

optional

2.6 TFTP Group

The TFTP Group contains information on the TFTP file transfer

client that is used to download fonts, and read or write remote

configuration files.

OBJECT:

-------

ncdTftpClientReadReqs { ncdTftp 1 }

Syntax:

Counter

Definition:

The number of file read requests made by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientWriteReqs { ncdTftp 2 }

Syntax:

Counter

Definition:

The number of file write requests made by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientInDataPkts { ncdTftp 3 }

Syntax:

Counter

Definition:

The number of TFTP Data packets received by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientInAcks { ncdTftp 4 }

Syntax:

Counter

Definition:

The number of TFTP Ack packets received by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientInNoFileErrors { ncdTftp 5 }

Syntax:

Counter

Definition:

The number of times an TFTP Error packet was received

by the TFTP client with the error code set to

"File not found".

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientInAccessErrors { ncdTftp 6 }

Syntax:

Counter

Definition:

The number of times an TFTP Error packet was received

by the TFTP client with the error code set to

"Access violation".

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientInDiskFullErrors { ncdTftp 7 }

Syntax:

Counter

Definition:

The number of times an TFTP Error packet was received

by the TFTP client with the error code set to "Disk full".

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientNoBufs { ncdTftp 8 }

Syntax:

Counter

Definition:

The number of times the TFTP client failed to allocate

a buffer.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientOutDataPkts { ncdTftp 9 }

Syntax:

Counter

Definition:

The number of TFTP Data packets sent by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientOutAcks { ncdTftp 10 }

Syntax:

Counter

Definition:

The number of TFTP Ack packets sent by the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientRetrans { ncdTftp 11 }

Syntax:

Counter

Definition:

The number of times a TFTP packet was retransmitted by

the TFTP client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdTftpClientRetries { ncdTftp 12 }

Syntax:

INTEGER

Definition:

The number of retransmissions of a TFTP packet sent

before an error situation is declared.

Access:

read-write

Status:

obsolete

OBJECT:

-------

ncdTftpClientTimeout { ncdTftp 13 }

Syntax:

TimeTicks

Definition:

The amount of time between retransmissions of a TFTP

packet.

Access:

read-write

Status:

obsolete

2.7 NFS Group

The NFS Group contains information on the NFS file transfer

client that is used to download fonts, and read or write remote

configuration files. Note that a specific version of NCD software

may not use all possible types of NFS protocol operations and

will not report the counters for those operation types.

OBJECT:

-------

ncdNfsClientNulls { ncdNfs 1 }

Syntax:

Counter

Definition:

The number of NFS Null operations requested by the

NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientGetAttributes { ncdNfs 2 }

Syntax:

Counter

Definition:

The number of NFS Get Attribute operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientSetAttributes { ncdNfs 3 }

Syntax:

Counter

Definition:

The number of NFS Set Attribute operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientGetRoots { ncdNfs 4 }

Syntax:

Counter

Definition:

The number of NFS Get File System Root operations

requested by the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientLookups { ncdNfs 5 }

Syntax:

Counter

Definition:

The number of NFS File Name Component Lookup

operations requested by the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientReadLinks { ncdNfs 6 }

Syntax:

Counter

Definition:

The number of NFS Read Link operations requested

by the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientReads { ncdNfs 7 }

Syntax:

Counter

Definition:

The number of NFS Read operations requested by the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientWriteCaches { ncdNfs 8 }

Syntax:

Counter

Definition:

The number of NFS Write Cache operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientWrites { ncdNfs 9 }

Syntax:

Counter

Definition:

The number of NFS Write operations requested by the NFS

client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientCreates { ncdNfs 10 }

Syntax:

Counter

Definition:

The number of NFS File Create operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientRemoves { ncdNfs 11 }

Syntax:

Counter

Definition:

The number of NFS File Remove operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientRenames { ncdNfs 12 }

Syntax:

Counter

Definition:

The number of NFS File Rename operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientCreateLinks { ncdNfs 13 }

Syntax:

Counter

Definition:

The number of NFS Create Link operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientCreateSymlinks { ncdNfs 14 }

Syntax:

Counter

Definition:

The number of NFS Create Symbolic Link operations

requested by the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientMakeDir { ncdNfs 15 }

Syntax:

Counter

Definition:

The number of NFS Create Directory operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientRemoveDir { ncdNfs 16 }

Syntax:

Counter

Definition:

The number of NFS Remove Directory operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientReadDir { ncdNfs 17 }

Syntax:

Counter

Definition:

The number of NFS Read Directory operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsClientStatFileSys { ncdNfs 18 }

Syntax:

Counter

Definition:

The number of NFS Stat File System operations requested by

the NFS client.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdNfsMountTable { ncdNfs 19 }

Syntax:

SEQUENCE OF NfsMountEntry

Definition:

A table containing NFS Client file system mounting

information.

Access:

read-write

Status:

obsolete

OBJECT:

-------

ncdNfsMountEntry { ncdNfsMountTable 1 }

Syntax:

NcdNfsMountEntry ::= SEQUENCE {

ncdNfsMountEntryIndex

INTEGER,

ncdNfsMountEntryLocalDir

OCTET STRING,

ncdNfsMountEntryServerAddr

IpAddress,

ncdNfsMountEntryServerDir

OCTET STRING,

ncdNfsMountEntryReadSize

INTEGER (1..8192),

ncdNfsMountEntryWriteSize

INTEGER (1..8192),

ncdNfsMountEntryRetries

INTEGER,

ncdNfsMountEntryTimeout

INTEGER

}

Definition:

Information about an NFS mounted file system on the

NFS Client. An object of this type is transient, in

that the file system can be unmounted.

Access:

read-write

Status:

obsolete

OBJECT:

-------

ncdNfsMountEntryIndex { ncdNfsMountEntry 1 }

Syntax:

INTEGER

Definition:

A unique value for each mounted file system. The value

must remain constant during the period of time that the

file system remains mounted.

Access:

read-only

Status:

obsolete

OBJECT:

-------

ncdNfsMountEntryLocalDir { ncdNfsMountEntry 2 }

Syntax:

OCTET STRING

Definition:

The local directory name of the mount point of the NFS

file system represented by this entry.

Access:

read-only

Status:

obsolete

OBJECT:

-------

ncdNfsMountEntryServerAddr { ncdNfsMountEntry 3 }

Syntax:

NetworkAddress

Definition:

The IP address of the NFS file server for this entry.

Access:

read-only

Status:

obsolete

OBJECT:

-------

ncdNfsMountEntryServerDir { ncdNfsMountEntry 4 }

Syntax:

OCTET STRING

Definition:

The directory name of the file system on the file server

for this entry.

Access:

read-only

Status:

obsolete

OBJECT:

-------

ncdNfsMountEntryReadSize { ncdNfsMountEntry 5 }

Syntax:

INTEGER

Definition:

The preferred size of read requests sent by the client

to the NFS server.

Access:

read-write

Status:

obsolete

OBJECT:

-------

ncdNfsMountEntryWriteSize { ncdNfsMountEntry 6 }

Syntax:

INTEGER

Definition:

The preferred size of write requests sent by the client

to the NFS server.

Access:

read-write

Status:

obsolete

OBJECT:

-------

ncdNfsMountEntryRetries { ncdNfsMountEntry 7 }

Syntax:

INTEGER

Definition:

The number of retransmissions of an NFS request sent before

an error situation is declared by the RPC layer.

Access:

read-write

Status:

obsolete

OBJECT:

-------

ncdNfsMountEntryTimeout { ncdNfsMountEntry 8 }

Syntax:

TimeTicks

Definition:

The amount of time between retransmissions of an NFS

request used by the RPC layer.

Access:

read-write

Status:

obsolete

OBJECT:

-------

ncdNfsMountRpcTimeout { ncdNfs 20 }

Syntax:

TimeTicks

Definition:

The amount of time between retransmissions of a MOUNT

request used by the RPC layer.

Access:

read-write

Status:

obsolete

OBJECT:

-------

ncdNfsMountRpcRetries { ncdNfs 21 }

Syntax:

INTEGER

Definition:

The number of retransmissions of a MOUNT request sent

before an error situation is declared by the RPC layer.

Access:

read-write

Status:

obsolete

2.8 RPC Port Mapper Group

The RPC Port Mapper Group contains information on the SUN RPC

service which maps RPC services to specific network addresses.

OBJECT:

-------

ncdRpcPortMapperTimeout { ncdRpcPortMapper 1 }

Syntax:

TimeTicks

Definition:

The time between successive retransmissions of RPC

port mapper requests. The resolution of this timer is

one-seventieth (1/70) of a second.

Access:

read-write

Status:

obsolete

OBJECT:

-------

ncdRpcPortMapperRetries { ncdRpcPortMapper 2 }

Syntax:

INTEGER

Definition:

The maximum number of retransmissions of an RPC

port mapper request before a failure situation is

declared.

Access:

read-write

Status:

obsolete

2.9 X.11 Server Group

The X.11 Server Group contains information on the X.11 display

server running in the X display station.

OBJECT:

-------

ncdX11EmbryonicConnTimeout { ncdX11 1 }

Syntax:

TimeTicks

Definition:

The time allowed to elapse between the receipt of an

X.11 connection attempt and the delivery of a complete

X.11 Connection Request packet before an error situation

is declared and the embryonic connection is discarded.

Access:

read-write

Status:

optional

2.10 XDMCP Group

The XDMCP Group contains information on the X Display Manager

Control Protocol which is used by the X Display Station to

contact a host that can provide an X.11 session.

OBJECT:

-------

ncdXdmcpKeepAliveIdleTime { ncdXdmcp 1 }

Syntax:

TimeTicks

Definition:

The time that must elapse of no X.11 activity before a

XDMCP keepalive packet is generated. The resolution of

this timer is one (1) minute.

Access:

read-write

OBJECT:

-------

ncdXdmcpKeepAliveMaxIdleTime { ncdXdmcp 2 }

Syntax:

TimeTicks

Definition:

The maximum amount of time that XDMCP keepalives are

transmitted with no response before a failure situation

is declared. The resolution of this timer is one (1)

second.

Access:

read-write

2.11 DECnet Group

The DECnet Group contains information on the DECnet Phase IV

transport, and network protocols. The MOP based periodic

identification feature is also included in this group.

OBJECT:

-------

ncdDecnetConnectTimeout { ncdDecnet 1 }

Syntax:

TimeTicks

Definition:

The time that must elapse between a DECnet connection

attempt and a lack of a response for a failure situation

to be declared. The resolution of this timer is one-half

(1/2) a second.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdDecnetMaxRetrans { ncdDecnet 2 }

Syntax:

INTEGER

Definition:

The maximum number of times DECnet will retransmit data

on an established connection before a failure situation

is declared.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdDecnetKeepAliveInterval { ncdDecnet 3 }

Syntax:

TimeTicks

Definition:

The amount of time of inactivity on an established DECnet

connection that causes a DECnet keepalive packet to be sent.

The resolution of this timer is one-fifth (1/5) of a second.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdDecnetHelloInterval { ncdDecnet 4 }

Syntax:

TimeTicks

Definition:

The time between the successive transmissions of DECnet Hello

packets. The resolution of this timer is one-fifth (1/5) of

a second.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdDecnetDefaultRouterTimeout { ncdDecnet 5 }

Syntax:

TimeTicks

Definition:

The amount of time after which a Router Hello message is

received that the sending router is used as the default

router. The resolution of this timer is one-fifth (1/5) of

a second.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdDecnetMopIdentifyInterval { ncdDecnet 6 }

Syntax:

TimeTicks

Definition:

The time between the successive transmissions of MOP Identify

packets. The resolution of this timer is one-fifth (1/5) of

a second.

Access:

read-write

Status:

optional

2.12 LAT Group

The LAT Group contains information on the unit's implementation of

DEC's Local Area Transport.

OBJECT:

-------

ncdLatConnectTimeout { ncdLat 1 }

Syntax:

TimeTicks

Definition:

The time that must elapse between a LAT connection

attempt and a lack of a response for a failure situation

to be declared. The resolution of this timer is one (1)

second.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdLatCircuitFlushTime { ncdLat 2 }

Syntax:

TimeTicks

Definition:

The time LAT waits between receiving user data and

transmitting this data on the network to the remote

peer. This delay period is used to collect further

user data so that it can be bundled into the same

LAT packet. The resolution of this timer is

one-seventieth (1/70) of a second.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdLatCircuitRetransInterval { ncdLat 3 }

Syntax:

TimeTicks

Definition:

The time between consecutive retransmissions of

LAT data packets on an established connection.

The resolution of this timer is one-seventieth

(1/70) of a second.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdLatCircuitMaxRetrans { ncdLat 4 }

Syntax:

INTEGER

Definition:

The maximum number of times LAT will retransmit data

on an established connection before a failure situation

is declared.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdLatKeepAliveInterval { ncdLat 5 }

Syntax:

TimeTicks

Definition:

The amount of time of inactivity on an established LAT

connection that causes a LAT keepalive packet to be sent.

The resolution of this timer is one (1) second.

Access:

read-write

Status:

optional

2.13 Name Resolver Group

The Name Resolver Group contains information on the Internet

name resolution functions which currently include support for

the Domain Name System, and IEN-116.

OBJECT:

-------

ncdNameResolverTimeout { ncdNameResolver 1 }

Syntax:

TimeTicks

Definition:

The time between successive retransmissions of name

resolver requests. The resolution of this timer is

one (1) second.

Access:

read-write

Status:

obsolete

OBJECT:

-------

ncdNameResolverRetries { ncdNameResolver 2 }

Syntax:

INTEGER

Definition:

The maximum number of retransmissions of a name resolver

request before a failure situation is declared.

Access:

read-write

Status:

obsolete

2.14 Font Service Group

The Font Service Group contains information on the unit's X.11

font retrieval functions. An NCD can currently use three different

transports to retrieve fonts: TFTP, NFS, and DECnet. The X.11

Server shares font information among X.11 clients. Additionally,

the font service code caches a font when the last X.11 client

reference to the font is removed. This is beneficial if another

X.11 client later requests this font.

OBJECT:

-------

ncdFontServiceRecuperation { ncdFontService 1 }

Syntax:

TimeTicks

Definition:

The time after a font server fails to respond to a

request that the font server is assumed to be inoperable.

During this period of assumed inoperablity, the unit's

font service will not attempt any further requests to

the font server. The resolution of this timer is one (1)

second.

N.B. The amount of time for a font server to be declared

inoperable is dependent on the transport mechanism used.

For example, if TFTP is used, the TFTP timeout and retry

parameters control the length of the font server timeout.

Access:

read-write

Status:

obsolete

OBJECT:

-------

ncdFontServiceCacheSize { ncdFontService 2 }

Syntax:

INTEGER (0..4294967295)

Definition:

The maximum possible size in bytes of the font service

cache for unreferenced fonts.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdFontServiceCacheUsed { ncdFontService 3 }

Syntax:

Gauge

Definition:

The current size in bytes of all the unreferenced

fonts in the font service cache.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdFontServiceNumCachedFonts { ncdFontService 4 }

Syntax:

Gauge

Definition:

The current number of unreferenced fonts in the font

service cache.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdFontServiceNetLoads { ncdFontService 5 }

Syntax:

Counter

Definition:

The number of times the font service was able to

successfully satisfy a font request by the X.11

server only by loading a font from a font server

over the network.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdFontServiceReferenceIncs { ncdFontService 6 }

Syntax:

Counter

Definition:

The number of times the font service was able to

successfully satisfy a font request by the X.11

server from already referenced fonts.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdFontServiceCacheHits { ncdFontService 7 }

Syntax:

Counter

Definition:

The number of times the font service was able to

successfully satisfy a font request by the X.11

server from the unreferenced fonts in the font

service cache.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdFontServiceFontReferences { ncdFontService 8 }

Syntax:

Gauge

Definition:

The total number of references to fonts by the X.11

server.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdFontServiceFontsInUse { ncdFontService 9 }

Syntax:

Gauge

Definition:

The total number of fonts to which the X.11 server has

at least one reference.

Access:

read-only

Status:

optional

2.15 Serial Daemon Group

The Serial Daemon Group contains information on the network

daemon in NCD units that allows access to the auxilary RS-232

port of the X Display Station. Potentially, this daemon is

listening on both an TCP Port and a DECnet object simultaneously.

OBJECT:

-------

ncdSerialDaemonState { ncdSerialDaemon 1 }

Syntax:

INTEGER {

listening(1),	-- waiting for a connection

active(2)		-- connected and transferring data

}

Definition:

The current state of the serial daemon. If the state is

'active', it can be set to 'listening' which will cause

the current connection to be closed, and the daemon to

return to waiting for a new connection. The state cannot

be set from 'listening' to 'active.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdSerialDaemonConnAccepted { ncdSerialDaemon 2 }

Syntax:

Counter

Definition:

The number of connections made to the serial daemon.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSerialDaemonConnBadAccess { ncdSerialDaemon 3 }

Syntax:

Counter

Definition:

The number of connections that were closed by the

serial daemon due to the remote address not being

present in the serial daemon's access control list.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSerialDaemonConnConfError { ncdSerialDaemon 4 }

Syntax:

Counter

Definition:

The number of connections that were closed by the

serial daemon due to the a local configuration error.

For example, the serial port not being enabled for use

by the serial daemon.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSerialDaemonConnIOError { ncdSerialDaemon 5 }

Syntax:

Counter

Definition:

The number of connections that were closed by the

serial daemon due to the a local I/O error. This

includes the user aborting the connection via local

mechanisms.

Access:

read-only

Status:

optional

OBJECT:

-------

ncdSerialDaemonConnEofClose { ncdSerialDaemon 6 }

Syntax:

Counter

Definition:

The number of connections that were closed by the

remote host. This is the normal mode of ending a

connection to the serial daemon.

Access:

read-only

Status:

optional

2.16 Telnet Group

The Telnet Group contains information on the Internet network

virtual terminal protocol.

OBJECT:

-------

ncdTelnetConnectTimeout { ncdTelnet 1 }

Syntax:

TimeTicks

Definition:

The time that must elapse between a Telnet connection

attempt and a lack of a response for a failure situation

to be declared. The resolution of this timer is one (1)

second.

N.B. If this timer is set to be larger than the connect

timeout for TCP, then TCP will declare a failure situation

first.

Access:

read-write

Status:

obsolete

2.17 Cterm Group

The Cterm Group contains information on the DECnet network

virtual terminal protocol.

OBJECT:

-------

ncdCtermConnectTimeout { ncdCterm 1 }

Syntax:

TimeTicks

Definition:

The time that must elapse between a Cterm connection

attempt and a lack of a response for a failure situation

to be declared. The resolution of this timer is one (1)

second.

N.B. If this timer is set to be larger than the connect

timeout for DECnet, then DECnet will declare a failure

situation first.

Access:

read-write

Status:

obsolete

2.18 ARP Group

The ARP Group contains information on the Ethernet Address

Resolution Protocol used to map Internet addresses to Ethernet

addresses.

OBJECT:

-------

ncdArpIncompleteEntryTimeout { ncdArp 1 }

Syntax:

TimeTicks

Definition:

The amount of time an incomplete ARP entry created

by an ARP request is kept awaiting a response. The

resolution of this timer is one (1) minute.

Access:

read-write

Status:

optional

OBJECT:

-------

ncdArpCompleteEntryTimeout { ncdArp 2 }

Syntax:

TimeTicks

Definition:

The amount of time an completed ARP entry is kept

after it's last usage. The resolution of this timer

is one (1) minute.

Access:

read-write

Status:

optional

3. Definitions

NCD-MIBS {iso org(3) dod(6) internet(1) private(4)

enterprises(1) ncd(82) ncd-mibs(2) }

NCD-MIB DEFINITIONS ::= BEGIN

IMPORTS

enterprises, NetworkAddress, IpAddress, Counter,

Gauge, TimeTicks, OBJECT-TYPE

FROM RFC1155-SMI;

ncd OBJECT IDENTIFIER ::= { enterprises 82 }

ncd-products OBJECT IDENTIFIER ::= { ncd 1 }

ncd16 OBJECT IDENTIFIER ::= { ncd-products 1 }

ncd19 OBJECT IDENTIFIER ::= { ncd-products 2 }

ncd17c OBJECT IDENTIFIER ::= { ncd-products 3 }

ncd16e OBJECT IDENTIFIER ::= { ncd-products 4 }

ncd19b OBJECT IDENTIFIER ::= { ncd-products 5 }

ncd15b OBJECT IDENTIFIER ::= { ncd-products 6 }

ncd14c OBJECT IDENTIFIER ::= { ncd-products 7 }

ncd19c OBJECT IDENTIFIER ::= { ncd-products 8 }

ncd19r OBJECT IDENTIFIER ::= { ncd-products 9 }

ncd-mibs OBJECT IDENTIFIER ::= { ncd 2 }

ncd-mibs-mib1 OBJECT IDENTIFIER ::= { ncd-mibs 1 }

ncd-mibs-mib2 OBJECT IDENTIFIER ::= { ncd-mibs 2 }

-- NCD MIB I Objects

ncdSystem OBJECT IDENTIFIER ::= { ncd-mibs-mib1 1 }

ncdMbuf OBJECT IDENTIFIER ::= { ncd-mibs-mib1 2 }

ncdEther OBJECT IDENTIFIER ::= { ncd-mibs-mib1 3 }

ncdSerial OBJECT IDENTIFIER ::= { ncd-mibs-mib1 4 }

ncdTcp OBJECT IDENTIFIER ::= { ncd-mibs-mib1 5 }

ncdUdp OBJECT IDENTIFIER ::= { ncd-mibs-mib1 6 }

ncdTftp OBJECT IDENTIFIER ::= { ncd-mibs-mib1 7 }

ncdNfs OBJECT IDENTIFIER ::= { ncd-mibs-mib1 8 }

-- NCD MIB II Objects

ncdRpcPortMapper OBJECT IDENTIFIER ::= { ncd-mibs-mib2 9 }

ncdX11 OBJECT IDENTIFIER ::= { ncd-mibs-mib2 10 }

ncdXdmcp OBJECT IDENTIFIER ::= { ncd-mibs-mib2 11 }

ncdDecnet OBJECT IDENTIFIER ::= { ncd-mibs-mib2 12 }

ncdLat OBJECT IDENTIFIER ::= { ncd-mibs-mib2 13 }

ncdNameResolver OBJECT IDENTIFIER ::= { ncd-mibs-mib2 14 }

ncdFontService OBJECT IDENTIFIER ::= { ncd-mibs-mib2 15 }

ncdSerialDaemon OBJECT IDENTIFIER ::= { ncd-mibs-mib2 16 }

ncdTelnet OBJECT IDENTIFIER ::= { ncd-mibs-mib2 17 }

ncdCterm OBJECT IDENTIFIER ::= { ncd-mibs-mib2 18 }

ncdArp OBJECT IDENTIFIER ::= { ncd-mibs-mib2 19 }

-- NCD System Group

ncdSysMemTotal OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  optional

::= { ncdSystem 1 }

ncdSysMemAvail OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdSystem 2 }

ncdSysMemFrags OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdSystem 3 }

ncdSysIdleTime OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-only

STATUS  optional

::= { ncdSystem 4 }

ncdSysBootPromVersion OBJECT-TYPE

SYNTAX  OCTET STRING

ACCESS  read-only

STATUS  optional

::= { ncdSystem 5 }

ncdSysKeyboardControllerVersion OBJECT-TYPE

SYNTAX  OCTET STRING

ACCESS  read-only

STATUS  optional

::= { ncdSystem 6 }

ncdSysAdminStatus OBJECT-TYPE

SYNTAX  INTEGER {

running(1),

resetSession(2),

resetUnitOnLastClientClose(3),

resetUnit(4)

}

ACCESS  read-write

STATUS  optional

::= { ncdSystem 7 }

-- NCD Mbuf Group

ncdMbufTotal OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 1 }

ncdMbufFree OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 2 }

ncdMbufClusterTotal OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 3 }

ncdMbufClusterFree OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 4 }

ncdMbufReserved OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 5 }

ncdMbufDrops OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 6 }

ncdMbufWaits OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 7 }

ncdMbufData OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 8 }

ncdMbufHeader OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 9 }

ncdMbufSocket OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 10 }

ncdMbufProtoCtrlBlock OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 11 }

ncdMbufRouteTable OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 12 }

ncdMbufAtTable OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 13 }

ncdMbufSocketName OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 14 }

ncdMbufSocketOptions OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 15 }

ncdMbufFrags OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 16 }

ncdMbufInterfaceAddr OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdMbuf 17 }

-- NCD Ether Group

-- the ether interface table

ncdEtherTable OBJECT-TYPE

SYNTAX  SEQUENCE OF NcdEtherEntry

ACCESS  read-write

STATUS  optional

::= { ncdEther 1 }

ncdEtherEntry OBJECT-TYPE

SYNTAX  NcdEtherEntry

ACCESS  read-write

STATUS  optional

::= { ncdEtherTable 1 }

NcdEtherEntry ::= SEQUENCE {

ncdEtherInCrcErrors

Counter,

ncdEtherInMissed

Counter,

ncdEtherInFrameErrors

Counter,

ncdEtherSingleCollisions

Counter,

ncdEtherMultipleCollisions

Counter,

ncdEtherLateCollisions

Counter,

ncdEtherOutDeferred

Counter,

ncdEtherOutRetryErrors

Counter,

ncdEtherLostCarrier

Counter,

ncdEtherRestarts

Counter

}

ncdEtherInCrcErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 1 }

ncdEtherInMissed OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 2 }

ncdEtherInFrameErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 3 }

ncdEtherSingleCollisions OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 4 }

ncdEtherMultipleCollisions OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 5 }

ncdEtherLateCollisions OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 6 }

ncdEtherOutDeferred OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 7 }

ncdEtherOutRetryErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 8 }

ncdEtherLostCarrier OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 9 }

ncdEtherRestarts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdEtherEntry 10 }

-- NCD Serial Group

-- the serial interface table

ncdSerialTable OBJECT-TYPE

SYNTAX  SEQUENCE OF NcdSerialEntry

ACCESS  read-write

STATUS  optional

::= { ncdSerial 1 }

ncdSerialEntry OBJECT-TYPE

SYNTAX  NcdSerialEntry

ACCESS  read-write

STATUS  optional

::= { ncdSerialTable 1 }

NcdSerialEntry ::= SEQUENCE {

ncdSerialInOverruns

Counter,

ncdSerialInFrameErrors

Counter,

ncdSerialInParityErrors

Counter,

ncdSerialInBreakErrors

Counter

}

ncdSerialInOverruns OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialEntry 1 }

ncdSerialInFrameErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialEntry 2 }

ncdSerialInParityErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialEntry 3 }

ncdSerialInBreakErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialEntry 4 }

-- NCD TCP Group

ncdTcpRetransDrops OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTcp 1 }

ncdTcpOutKeepAlives OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTcp 2 }

ncdTcpKeepAliveDrops OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTcp 3 }

ncdTcpInAcks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTcp 4 }

ncdTcpOutBareAcks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTcp 5 }

ncdTcpConnectTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdTcp 6 }

ncdTcpMaxRetrans OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  optional

::= { ncdTcp 7 }

ncdTcpKeepAliveIdleTime OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdTcp 8 }

ncdTcpKeepAliveInterval OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdTcp 9 }

ncdTcpKeepAliveMaxIdle OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdTcp 10 }

ncdTcpDfltLingerTime OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdTcp 11 }

-- NCD TFTP Group

ncdTftpClientReadReqs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 1 }

ncdTftpClientWriteReqs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 2 }

ncdTftpClientInDataPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 3 }

ncdTftpClientInAcks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 4 }

ncdTftpClientInNoFileErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 5 }

ncdTftpClientInAccessErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 6 }

ncdTftpClientInDiskFullErrors OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 7 }

ncdTftpClientNoBufs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 8 }

ncdTftpClientOutDataPkts OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 9 }

ncdTftpClientOutAcks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 10 }

ncdTftpClientRetrans OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdTftp 11 }

ncdTftpClientRetries OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  obsolete

::= { ncdTftp 12 }

ncdTftpClientTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  obsolete

::= { ncdTftp 13 }

-- NCD NFS Group

ncdNfsClientNulls OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 1 }

ncdNfsClientGetAttributes OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 2 }

ncdNfsClientSetAttributes OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 3 }

ncdNfsClientGetRoots OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 4 }

ncdNfsClientLookups OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 5 }

ncdNfsClientReadLinks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 6 }

ncdNfsClientReads OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 7 }

ncdNfsClientWriteCaches OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 8 }

ncdNfsClientWrites OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 9 }

ncdNfsClientCreates OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 10 }

ncdNfsClientRemoves OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 11 }

ncdNfsClientRenames OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 12 }

ncdNfsClientCreateLinks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 13 }

ncdNfsClientCreateSymlinks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 14 }

ncdNfsClientMakeDir OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 15 }

ncdNfsClientRemoveDir OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 16 }

ncdNfsClientReadDir OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 17 }

ncdNfsClientStatFileSys OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdNfs 18 }

ncdNfsMountRpcTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  obsolete

::= { ncdNfs 20 }

ncdNfsMountRpcRetries OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  obsolete

::= { ncdNfs 21 }

-- the NFS client mount table

ncdNfsMountTable OBJECT-TYPE

SYNTAX  SEQUENCE OF NcdNfsMountEntry

ACCESS  read-write

STATUS  obsolete

::= { ncdNfs 19 }

ncdNfsMountEntry OBJECT-TYPE

SYNTAX  NcdNfsMountEntry

ACCESS  read-write

STATUS  obsolete

::= { ncdNfsMountTable 1 }

NcdNfsMountEntry ::= SEQUENCE {

ncdNfsMountEntryIndex

INTEGER,

ncdNfsMountEntryLocalDir

OCTET STRING,

ncdNfsMountEntryServerAddr

IpAddress,

ncdNfsMountEntryServerDir

OCTET STRING,

ncdNfsMountEntryReadSize

INTEGER (1..8192),

ncdNfsMountEntryWriteSize

INTEGER (1..8192),

ncdNfsMountEntryRetries

INTEGER,

ncdNfsMountEntryTimeout

INTEGER

}

ncdNfsMountEntryIndex OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  obsolete

::= { ncdNfsMountEntry 1 }

ncdNfsMountEntryLocalDir OBJECT-TYPE

SYNTAX  OCTET STRING

ACCESS  read-only

STATUS  obsolete

::= { ncdNfsMountEntry 2 }

ncdNfsMountEntryServerAddr OBJECT-TYPE

SYNTAX  NetworkAddress

ACCESS  read-only

STATUS  obsolete

::= { ncdNfsMountEntry 3 }

ncdNfsMountEntryServerDir OBJECT-TYPE

SYNTAX  OCTET STRING

ACCESS  read-only

STATUS  obsolete

::= { ncdNfsMountEntry 4 }

ncdNfsMountEntryReadSize OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  obsolete

::= { ncdNfsMountEntry 5 }

ncdNfsMountEntryWriteSize OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  obsolete

::= { ncdNfsMountEntry 6 }

ncdNfsMountEntryRetrans OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  obsolete

::= { ncdNfsMountEntry 7 }

ncdNfsMountEntryTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  obsolete

::= { ncdNfsMountEntry 8 }

-- RPC Port Mapper

ncdRpcPortMapperTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  obsolete

::= { ncdRpcPortMapper 1 }

ncdRpcPortMapperRetries OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  obsolete

::= { ncdRpcPortMapper 2 }

-- X.11 Server Group

ncdX11EmbryonicConnTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdX11 1 }

-- XDMCP Group

ncdXdmcpKeepAliveIdleTime OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdXdmcp 1 }

ncdXdmcpKeepAliveMaxIdleTime OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdXdmcp 2 }

-- DECnet Group

ncdDecnetConnectTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdDecnet 1 }

ncdDecnetMaxRetrans OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  optional

::= { ncdDecnet 2 }

ncdDecnetKeepAliveInterval OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdDecnet 3 }

ncdDecnetHelloInterval OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdDecnet 4 }

ncdDecnetDefaultRouterTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdDecnet 5 }

ncdDecnetMopIdentifyInterval OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdDecnet 6 }

-- LAT Group

ncdLatConnectTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdLat 1 }

ncdLatCircuitFlushTime OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdLat 2 }

ncdLatCircuitRetransInterval OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdLat 3 }

ncdLatCircuitMaxRetrans OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  optional

::= { ncdLat 4 }

ncdLatKeepAliveInterval OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdLat 5 }

-- Name Resolver Group

ncdNameResolverTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  obsolete

::= { ncdNameResolver 1 }

ncdNameResolverRetries OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  obsolete

::= { ncdNameResolver 2 }

-- Font Service Group

ncdFontServiceRecuperation OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  obsolete

::= { ncdFontService 1 }

ncdFontServiceCacheSize OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-write

STATUS  optional

::= { ncdFontService 2 }

ncdFontServiceCacheUsed OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdFontService 3 }

ncdFontServiceNumCachedFonts OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdFontService 4 }

ncdFontServiceFontLoads OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdFontService 5 }

ncdFontServiceReferenceIncs OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdFontService 6 }

ncdFontServiceCacheHits OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdFontService 7 }

ncdFontServiceFontReferences OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdFontService 8 }

ncdFontServiceFontsInUse OBJECT-TYPE

SYNTAX  Gauge

ACCESS  read-only

STATUS  optional

::= { ncdFontService 9 }

-- Serial Daemon Group

ncdSerialDaemonState OBJECT-TYPE

SYNTAX  INTEGER {

listening(1),

active(2)

}

ACCESS  read-write

STATUS  optional

::= { ncdSerialDaemon 1 }

ncdSerialDaemonConnAccepted OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialDaemon 2 }

ncdSerialDaemonConnBadAccess OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialDaemon 3 }

ncdSerialDaemonConnConfError OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialDaemon 4 }

ncdSerialDaemonConnIOError OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialDaemon 5 }

ncdSerialDaemonConnEofClose OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  optional

::= { ncdSerialDaemon 6 }

-- Telnet Group

ncdTelnetConnectTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  obsolete

::= { ncdTelnet 1 }

-- Cterm Group

ncdCtermConnectTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  obsolete

::= { ncdCterm 1 }

-- ARP Group

ncdArpIncompleteEntryTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdArp 1 }

ncdArpCompleteEntryTimeout OBJECT-TYPE

SYNTAX  TimeTicks

ACCESS  read-write

STATUS  optional

::= { ncdArp 2 }

END

4. Identification of OBJECT instances for use with SNMP

4.1 ncdEtherTable Object Type Names

The name of an ethernet interface entry, 'x', is an OBJECT IDENTIFIER

of the form 'a' such that 'a' is the value of that instance of ifIndex

object type associated with 'x'.

For each object type, 't', for which the defined name, 'n', has a

prefix of ncdEther, an instance, 'i', of 't' is named by

an OBJECT IDENTIFIER of the form 'n.y', where 'y' is the name of

the interface entry about which 'i' represents information.

For example, suppose one wanted to find the number of Ethernet late

collisions associated with the second interface. Accordingly,

ncdEtherEntry.ncdEtherLateCollisions.2 would identify the desired

instance.

4.2 ncdSerialTable Object Type Names

The name of an serial interface entry, 'x', is an OBJECT IDENTIFIER

of the form 'a' such that 'a' is the value of that instance of ifIndex

object type associated with 'x'.

For each object type, 't', for which the defined name, 'n', has a

prefix of ncdSerial, an instance, 'i', of 't' is named by

an OBJECT IDENTIFIER of the form 'n.y', where 'y' is the name of

the interface entry about which 'i' represents information.

For example, suppose one wanted to find the number of serial parity

errors associated with the third interface. Accordingly,

ncdSerialEntry.ncdSerialInParityErrors.3 would identify the desired

instance.

4.3 ncdNfsMountTable Object Type Names

The name of an NFS mount entry, 'x', is an OBJECT IDENTIFIER of the

form 'a' such that 'a' is the value of that instance of

ncdNfsMountEntryIndex object type associated with 'x'.

For each object type, 't', for which the defined name, 'n', has a

prefix of ncdNfsMountEntry, an instance, 'i', of 't' is named by

an OBJECT IDENTIFIER of the form 'n.y', where 'y' is the name of

the NFS mount entry about which 'i' represents information.

For example, suppose one wanted to find the IP address of the NFS

server associated with the second NFS mount table entry. Accordingly,

ncdNfsMountEntryServerAddr.2 would identify the desired instance.

5. SNMP Traps Generated

5.1 Cold Start Trap

This trap is generated when the SNMP agent in the NCD unit is first

initialized.

5.2 Authentication Failure Trap

This trap is generated when the SNMP agent detects that the

authentication information in an SNMP request packet is invalid.

Generation of this trap can be controlled through the

"snmpEnableAuthTraps" object.

6. Trademarks

NCD is a trademark of Network Computing Devices, Inc.

Ethernet is a trademark of Xerox Corporation.

DEC, DECnet, and LAT are trademarks of Digital Equipment Corporation.