USING THE TMS320 XDS WITH PROCOMM PLUS (tm)

James M. Patterson

Texas Instruments, Inc.

370 South North Lake Boulevard

Suite 1008

Altamonte Springs, Florida   32701

December 21, 1989

OVERVIEW

This report describes the use of an IBM (tm) or compatible personal

computer (PC) with Datastorm Technologies' PROCOMM PLUS (tm) communications

software as a host for an XDS/22 emulator for the TMS320 digital signal

processors (DSP).  Configuration of the emulator for use with the PC is

described, and several scripts are provided for automating frequently performed

procedures, such as initialization of the emulator and loading

programs.

INTRODUCTION

The XDS\22 for the TMS320 family of digital signal processors (DSP) is a

host-independent emulator which may be configured to support any of Texas

Instruments' first (TMS320C1x) or second generation (TMS320C2x) DSPs. The

configuration of the emulator and the communications parameters set for the

communications software should work with any of these emulators.  Some of the

scripts are specifically tailored for first or second generation processors.

The scripts were written to work with PROCOMM PLUS (tm), version 1.1B or later.

Please note that they are not compatible with ProComm v2.4.2.

GETTING STARTED

In order to follow this procedure, you will need an IBM  (tm)  or

compatible PC, a TMS320 family XDS/22, PROCOMM PLUS (tm) communications

software, and an RS-232C data communications cable.

1.  Configure XDS/22:

The communications board for the XDS allows it to be configured as either a

terminal (DTE) or modem (DCE).	For use with a PC as a host, the "user" port of

the XDS, marked port A on the rear panel of the emulator, should be configured

as a DCE.  This s done by removing the front panel of the XDS, disconnecting

the ribbon cables from the communications card, which is the uppermost card in

the emulator card cage, and setting DIP switches S4 and S3 as shown in table 1

below.

Table 1

XDS/22 Communications Switch Settings

SWITCH	   DIP SWITCH

POSITION	   S4	   S3

1		  ON	  OFF

2		  ON	  OFF

3		  OFF	  ON

4		  OFF	  ON

5		  ON	  OFF

6		  ON	  ON

7		  OFF	  OFF

After setting the appropriate switch settings,  replace the communications

board in the XDS/22 card cage, reconnect the ribbon cables, and replace the

front cover of the XDS.

Install PROCOMM PLUS (tm) and script files:

The installation procedure for PROCOMM PLUS (tm) is described in the

PROCOMM PLUS (tm) User's Manual.

Script files are programs written in the ASPECT script language, and which

allow frequently used procedures to be automated.  Script files, which have a

file name extension of .ASP, may be placed either in the directory specified by

the PCPLUS environment variable, or in the directory which will be your current

working directory when you are using the XDS.  The following script files are

provided:

Table 2

TMS320 XDS/22 Script Files

Script file	       Description

INIT1.ASP		  Initializes TMS320C1x XDS

INIT2.ASP		  Initialized TMS320C2x XDS

LOAD1.ASP		  Loads program to TMS320C1x XDS

LOAD2.ASP		  Loads program to TMS320C2x XDS

XDS1.ASP		  TMS320C1x XDS power-on initialization

XDS2.ASP		  TMS320C2x XDS power-on initialization

If you will be working only with a first or second generation TMS320

processor, you may wish to install only the files associated with that series

of processor.  The script files will also create several parameter files in the

current working directory.  These files contain information on XDS

configuration and the most recently loaded program.

Connect the XDS and the PC:

The XDS/22 is connected to the PC via a cable.  Depending on the PC you are

using, the PC end of this cable will have either a 25 conductor female "D"

(DB-25S) connector or a 9 conductor female "D" (DB-9S) connector.  In general,

IBM (tm) PCs and PC-XTs and compatibles will have 25 pin RS232 connectors, and

PC-ATs and compatibles will have 9 pin RS232 connectors. The XDS/22 end of the

cable will have a 25 pin male "D" (DP-25P) connector. Determine which cable you

need.  Cables are available ready-made at retail computer stores or electronics

upply shops.  If you wish to fabricate your own cable, it should be wired as

shown in Table 3 or Table 4.

Table 3

XDS/22 to PC RS232 Communications Cable

IBM (tm) PC, PC-XT, and compatibles

XDS/22 Port A	       Signal		   PC Serial Port

Pin			 Name			 Pin

(DB-25P)					      (DB-25S)

1			 Shield 		  1

2			 TD			  2

3			 RD			  3

4			 RTS*			  4

5			 CTS*			  5

6			 DSR*			  6

7			 SG			  7

8			 DCD*			  8

20			 DTR*			 20

* Signal not essential for XDS operation

Table 4

XDS/22 to PC RS232C Communications Cable

IBM (tm) PC-AT and compatibles

XDS/22 Port A		  Signal	    PC Serial Port

Pin			   Name 		   Pin

(DB-25P)					 (DB-9S)

1			   Shield		  n/c

2			   TD			    3

3			   RD			    2

4			   RTS* 		    7

5			   CTS* 		    8

6			   DSR* 		    6

7			   SG			    5

8			   DCD* 		    1

20			   DTR* 		    4

* Signal not essential for XDS operation

Connect the DB-25P connector to Port A of the XDS\22, and the other end to the

serial port connector of your PC.  Start your PC, and power up your XDS, and

you will be ready to start.  Invoke PROCOMM PLUS (tm), and go to the terminal

mode screen.

OPERATING THE XDS WITH PROCOMM PLUS (tm)

The first time:

As a confidence check, you should first manually configure PROCOMM PLUS

(tm) to communicate with the XDS. First, press alt-P on the PC keyboard. The

line setup screen should appear on the PC monitor screen.  If it does not, you

have a problem with PROCOMM PLUS (tm).	If the screen does appear, set BAUD

RATE to 9600, PARITY to EVEN, DATA BITS to 7, STOP BITS to 2, and select the

COM port to which you have connected the XDS.  In many cases, this will be

COM1, although the communications software will support as many as 8 serial

ports.	After checking your settings, press the ESC key to return to terminal

mode.

Next, press alt-S to invoke the PROCOMM PLUS (tm) setup utility.  The setup

screen main menu should appear.  Select the TERMINAL OPTIONS setup.  The

TERMINAL OPTIONS setup screen should appear.  On this screen, the following

options should be set:

Terminal emulation			TVI920

Duplex					FULL

Software flow control			OFF

Hardware flow control			OFF

Line wrap				ON

Screen scroll				ON

CR translation				CR

BS translation				DESTRUCTIVE

Break length				xxx*

Enquiry					xxx*

*indicates a "don't care" setting

Now press the ESC key to return to the setup utility main menu.  Press ESC once

more to return to the terminal screen.	You should be ready to start XDS

operation.

First, press the "Enter" or "Return" key twice.  The XDS will determine the

correct data rate, and after a pause of around one second, should respond with

a revision identification message, a list of commands, and finally a "?"

prompt.  If you get this far, the hardware is working OK, and you are ready to

proceed to the section on runnning script files.  If you don't get the command

list and prompt, you probably have a problem with the PROCOMM PLUS (tm) setup,

the XDS, or the communications cable.  Go back and check these and try again.

Some of the most frequent errors are listed below.

Common XDS-PC Communications Problems

Symptom			    Possible cause(es)

No response from XDS		    XDS not powered up

XDS not connected

Parity mismatch

Cabling error

ERROR 1160 PROCESSOR SYNC LOST	    Signals shorted at

emulator target cable

Garbage characters on screen	    Parity mismatch

RUNNING SCRIPT FILES

Script files are provided which will automate the frequently performed

procedures of initializing the emulator at power up, changing emulator

paramters, and loading programs.

Power-up initialization

When you first power up the XDS, execute the script file XDSx.ASP (

XDS1.ASP for TMS320C1x, XDS2.ASP for TMS320C2x).  This script will configure

PROCOMM PLUS (tm) to communicate with the XDS.	It will also issue the

following instructions to the XDS:

INIT - sets emulator clock source, etc.

MAP - defines TMS320C1x program memory map (XDS1.ASP)

ITR - selects configuration of emulator "target" memory

(XDS2.ASP)

ICC - initializes cursor control characters

This file may be invoked by using the /f command line switch when running

PRO COMM PLUS (tm), or from the PROCOMM PLUS (tm) terminal screen by pressing

alt-F5.  The script will first look for a parameter file named XDSxINIT.PRM. If

the file exists, you will be asked if you want to use the initialization

paramters from the file.  If you answer NO, or if the file does not exist, you

will then be prompted for each of the parameters.  The new parameters will be

written to the parameter file, and the initialization will be performed.  If

the file executes correctly, a box will be displayed at the top of the screen

with the message "XDS READY."

Changing XDS paramters

The files INIT1.ASP and INIT2.ASP issue the INIT command to the XDS from

para meters stored in a parameter file, but do not change the communications

settings.  Like the XDSx.ASP scripts, the INITx scripts will first look for a

file to read parameters from, but allow keyboard entry of the parameters.

Loading files

LOAD1 and LOAD2 script files load an object file in TI tagged object format

from the PC to the XDS.  These scripts are identical except for the name of the

parameter file used.  When the LOADx script is invoked, it will first try to

open a file named XDSx.PRG, which contains the pathname of the last file

loaded.  If the file is found, you will be prompted to change the download

pathname before the object file is loaded.  If the parameter file doesn't

exist, you will be prompted to enter the name of the object file to be loaded.

If the named file doesn't exist, you will be given the opportunity to specify a

different file to be loaded, or to return to terminal mode.  When the object

file is found, the script will initiate an ASCII file transfer to the XDS.

Notes on load errors:

TI tagged object format is an ASCII format which breaks the object file into

lines of ASCII characters terminated by a carriage return and line feed.  The

only error checking performed by the XDS when a file is loaded is a checksum

comparison on each line of the file.  If the XDS detects an error during the

download, an error message will be displayed on the terminal screen.  When this

occurs, the only remedy is to abort the download and try again.  If, during the

download, an error message appears on the screen, you may abort the download by

pressing the ESC key once. You will then be prompted to try again.  Pressing

the ESC key twice will abort execution of the script.

Occasional errors during downloads are inevitable.  If you have persistant

problems with errors during downloads, you may wish to decrease the data rate,

or change the PROCOMM PLUS (tm) ASCII file transfer parameters.  Increasing the

line pacing time or the character pacing time may improve the error

performance, although this is done at the cost of increasing the time required

to download programs to the XDS.

Listings of the script files are included as appendices to this report.  You

may wish to modify these for your particular PC and emulator configuration.

Happy debugging!