Logarithmic Differential Compression (LDC)  - Patent Pending

------------------------------------------------------------

A patent application has been filed covering LDC as a new technology

--------------------------------------------------------------------

LDC is a simple, low overhead method of compressing audio and natural

signals with minimal impact to quality.  LDC takes advantage of several

basic principles and allows an easy tradeoff of quality and bits.  LDC can

also be combined with other compression techniques to achieve much higher

compression ratios.  This article covers the native format of LDC, how

it works, and a brief description of how the demo works.

Demonstration Code -> LDC.EXE

-----------------------------

The code provided has been compiled using the TI C compiler to create a

COFF file output (LDCDEMO.OUT).  This file can be directly loaded and run

using either a C31 DSK or C30 EVM.

The demo is essentiualy an analog loopback where one of three methods are

used in succession for several seconds each.  The loopback method can be

evaluated either by an actual analog loopback or by using a signal generator

and oscilliscope.

Loopback - ADC->DAC

u-law    - ADC->ulaw_compress->ulaw_decompress->DAC

LDC      - ADC-> LDC_compress-> LDC_decompress->DAC

Evaluating LDC or the simulated ulaw compressors with different bit masks

can be done by simply changing these values in the source file and

recompiling or by modifying them directly in the executable after it has

been loaded into the target.  In particular, try removing all of the mantissa

bits in the LDC and u-law formats to see what effect you have.

This demo can also be ported to your own C3x/C4x system by changing the

ADC/DAC and serial port set up routines.

Overview

--------

LDC uses the floating point format of the TMS320C3x/C4x number as a quick

logarithm approximation.  A simple differentiation then provides the

compression speech and video signals.  Unlike straight u-law or a-law

compression, LDC maintains the dynamic nature of the signal, which preserves

what is most important to human perception.  The result is a significant

improvement in perceptual quality for a given number of bits used.

With LDC, playback of high quality audio is possible using only 8 bits, but

additional bits will greatly improve the quality.  The TMS320C32, which is

capable of external 16 bit short float formats, becomes very cost affective

in this case.

In addition to being a simple compressor and decompressor, LDC is also a

linear process.  This allows a signal which is LDC compressed to be filtered

or processed while it is in the LDC compressed format.  A floating point

processor would be desired since on a fixed point processor the required

dynamic range and resolution would likely exceed 16 bits.

Masking Effects of the Ear

--------------------------

The human ear is logarithmic by nature, tending to respond to the loudest

sounds, or those that are significantly far apart in frequency.  The ear

also tends to readily notice low frequency distortions while high frequency

errors are masked, especially if they are transient.  This would indicate

that the signal to noise ratio for low frequencies needs to be fairly high,

but is not critical at high frequencies.

For a telephone, the listener will have difficulty in hearing the difference

between the orginal linear or ulaw/alaw companded signal.  'Companding' the

signal yields a signal to noise ratio (SNR) that is only about 20 dB, which

may seem to be unacceptable.  However, the SNR of simple log compression is

constant over all frequencies, and the resulting distortions for low

frequency signals end up in the mid frequency bands where they are most

easily recognized.  In addition, if two signals are present, one at the

low and the other at the high frequency bands, they will become badly

intermodulated with each other, which is most undesirable.

The reason we dont hear this in a phone is that the phone system is bandwidth

limited, and only interested in reproducing speech at a quality level good

enough to get the job done.

By using step sizes that are logarithmic, a telephone compander is able to

cover a large dynamic range, and still be able to preserve enough harmonic

content to be legible. Looking more closely, conversion errors are seen to

be proportional to the signal amplitude.  The result is that the noise

level remains a constant 20 dB below the signal, regardless of the signal

amplitude.  In other words, a small amplitude signal is passed with roughly

the same quality as a larger signal.

Another useful fact is that u-law and a-law representations are roughly

equivalent to a floating point format with only 4 exponent, 3 mantissa

and one sign bits as significant.  In telephone quality systems, full

precision, 32 bit floating point numbers are not be required.

Differential Compression

------------------------

For speech or music signals, the harmonic content tends to follow a 1/F

rule where lower frequency signals (bass) tend to have larger magnitudes as

compared to high frequencies.  Another way of looking at this is that the

slew rate demands (first derivative of the signal) are somewhat constant

over the audio band.

A number format that fits

-------------------------

The logarithmic form of the first derivative of an audio signal would

therefor have many benefits.  And, as mentioned, a floating point

representation would be even more useful since since this format is available

in low cost DSPs.  For more information of how to use the floating point

format as a good approximation of a base 2 logarithm, see Designer Notebook

Page (DNP) 22.

LDC Benefits and Tradeoffs

--------------------------

The bit trade off for precision and quality exhibits a surprisingly high

quality for the number of bits used.  In addition, the effect is compounded

by the log nature of the data format, which matches the ears heuristic

behavior.  The precision and dynamic range are simply a trade off of how

many bits you are willing to use in the Exponent/Sign/Mantissa.

E/S/M   Quality

--------------------------

4/1/0 - Telephone

4/1/1 - AM broadcast

4/1/2 - FM broadcast

4/1/3 - CD Audio

: : :

8/1/7 - C32 external short float format

Oversampling reconstruction filters, like those used in CD players, tend to

have high Qs and sharp cutoffs.  This tends to average higher frequency

amplitude modulations for signals whose frequencies are close to the Nyquist

limit.  For LDC, an added benefit is that the intermodulation noise tends to

be whitened by the error feedback making signal reconstruction filters

particularly effective.

The 'difference' operation which is used to form the LDC, is simply the

linear first derivative of the signal.  Since linear operations do not place

constraints on data processing, processing can be done while in the LDC or

truncated floating point format, simplifying both hardware and software

designs.

Low frequency signals are reconstructed with extremely small errors

(large SNR) since the LDC 'delta' value covers a very wide dynamic range.

In addition, the 'dynamic' delta coding allows high frequency signals in the

presence of low frequency signals without significant modulation artifacts

that are common in traditional linear differential techniques.

Refinements to the basic LDC compressor

---------------------------------------

By localy decompressing the LDC output stream from the compressor, which is

nothing more than an integration, an error feedback can be introduced which

will further reduce errors and improve quality.

LDC decompression uses a leaky integrator as a simple high pass filter.

This allows the LDC data stream to be connected and disconnected without

requiring a DC restoration.

LDC compression and decompression is particularly efficient on the TMS320

family of floating point processors.  A TMS320C3x/C4x assembly optimized

version of the LDC compressor and decompressor (not C callable) consumes

about 10 program words and 2 data words.  Execution requires 5 cycles in

either direction.

Enjoy, Keith Larson