File:           GSPMAC.DOC

Description:    Documentation for functions in gspmac.lib macro library

The following macros are available in the macro library gspmac.lib, contained

in the \tiga\include directory:

divl    Divide by power-of-two value

intfix  Perform fix for pre-3.0 TMS34010 silicon PIXBLT interrupt bug

mpyl    Multiply by power-of-two value

pop     Pop a value off the system stack (SP) into a register

popc    Pop a value off the C stack (A14 or STK) into a register

push    Push a register value on the system stack

pushc   Push a register value on the C stack (A14 or STK)

repc    Replicate COLOR0 and COLOR1 values based on PSIZE

repc0   Replicate COLOR0 value based on PSIZE

repc1   Replicate COLOR1 value based on PSIZE

rmo     Calculate bit position of right-most one

swap    Swap values in registers

-----------------------------------[ divl ]----------------------------------

Macro           divl    Rs,Rd

Registers       On entry:   Rs, Rd

Altered:    Rs, Rd

Exit:       Rs, Rd

Description     Divide the value in Rd by the power-of-two value in Rs (1, 2,

4, 8, 16, etc), and store the result (quotient) in Rd.  Rs

and Rd must be in the same register file.

Example         movk    2,a1

movk    8,a2

divl    a1,a2

After:  a1 = destroyed

a2 = 4

----------------------------------[ intfix ]---------------------------------

Macro           intfix

Registers       On entry:   None

Altered:    B0, B2, B4, B10, B13, B14

Exit:       None

Description     Check for interrupt of special case of PIXBLTB and PIXBLT.

Due to a bug in pre-3.0 silicon, this macro has to be the

last call in any interrupt service routine.

-----------------------------------[ mpyl ]----------------------------------

Macro           mpyl    Rs,Rd

Registers       On entry:   Rs, Rd

Altered:    Rs, Rd

Exit:       Rs, Rd

Description     Multiply the value in Rd by the power-of-two value in Rs (1,

2, 4, 8, 16, etc), and store the result (quotient) in Rd.  Rs

and Rd must be in the same register file.

Example         movk    4,a1

movk    3,a2

mpyl    a1,a2

After:  a1 = destroyed

a2 = 12

-----------------------------------[ pop ]-----------------------------------

macro           pop     Rd

Registers       On entry:   Rd, SP

Altered:    Rd, SP

Exit:       Rd

Description     Pop the value off the system stack into Rd and increment the

stack pointer register (SP=SP+32).

Example         movk    5,a1

push    a1

movk    3,a1

pop     a1

After:  a1 = 5

-----------------------------------[ popc ]----------------------------------

macro           popc    Rd

Registers       On entry:   Rd, A14

Altered:    Rd, A14

Exit:       Rd

Description     Decrement the C stack pointer (A14 or STK) and pop the value

into Rd.  Register Rd must be in the A register file.

Example         movk    7,a1

pushc   a1

movk    5,a1

popc    a1

After:  a1 = 7

-----------------------------------[ push ]----------------------------------

macro           push    Rs

Registers       On entry:   Rs, SP

Altered:    SP

Exit:       None

Description     Decrement the stack pointer register (SP=SP-32), and push the

source register onto the system stack.

Example         Before: SP = FFC00020

movk    5,a1

push    a1

After:  a1 = 5

SP = FFC00000

-----------------------------------[ pushc ]---------------------------------

macro           pushc   Rs

Registers       On entry:   Rs, A14

Altered:    A14

Exit:       None

Description     Pushes the source register onto the C stack (A14 or STK) and

inrements the C stack pointer (STK=STK+32).  Rs must be in

the A register file.

Example         Before: STK = FFC00020

movk    5,a1

pushc   a1

After:  a1 = 5

STK = FFC00040

-----------------------------------[ repc ]----------------------------------

Macro           repc

Registers       On entry:   COLOR0 = color value in ls psize bits

COLOR1 = color value in ls psize bits

Altered:    COLOR0, COLOR1, B12, B13, B14

Exit:       COLOR0 = replicated pixel value

COLOR1 = replicated pixel value

Description     Replicate color values that are currently in the COLOR0 and

COLOR1 registers throughout the COLOR0 and COLOR1 registers

(based on the current pixel size).

Example         Before: PSIZE = 4

movk    2,COLOR0

movk    7,COLOR1

repc

After:  COLOR0 = 22222222

COLOR1 = 77777777

----------------------------------[ repc0 ]----------------------------------

Macro           repc0

Registers       On entry:   COLOR0 = color value in ls psize bits

Altered:    COLOR0, B12, B13, B14

Exit:       COLOR0 = replicated pixel value

Description     Replicate color value that is currently in the COLOR0

register throughout COLOR0 register (based on the current

pixel size).

Example         Before: PSIZE  = 8

movk    4,COLOR0

repc0

After:  COLOR0 = 04040404

----------------------------------[ repc1 ]----------------------------------

Macro           repc1

Registers       On entry:   COLOR1 = color value in ls psize bits

Altered:    COLOR1, B12, B13, B14

Exit:       COLOR1 = replicated pixel value

Description     Replicate color value that is currently in the COLOR1

register throughout COLOR1 register (based on the current

pixel size).

Example         Before: PSIZE  = 4

movk    3,COLOR1

repc1

After:  COLOR1 = 33333333

-----------------------------------[ rmo ]-----------------------------------

Macro           rmo     Rs, Rd

Registers       On entry:   Rs, Rd

Altered:    Rs, Rd

Exit:       Rs, Rd

Description     Find bit number of the right-most one in Rs and place result

in Rd.   Registers Rs and Rd  must be in the same register

file.

Example         movk    2,a1

movk    12,a2

rmo     a1,a2

After:  a1 = destroyed

a2 = 1

-----------------------------------[ swap ]----------------------------------

Macro           swap    Rs, Rd

Registers       On entry:   Rs, Rd

Altered:    Rs, Rd

Exit:       Rs, Rd

Description     Swap the contents of the source and destination registers.

Registers must be in the same register file.

Example         movk    5,a1

movk    23,a2

swap    a1,a2

After:  a1 = 23

a2 = 5

- END OF FILE -