Background:

I - Current Synthesis Library Development Flow:

Library development is based on VML - Viewlogic Master Library format.

A VML file is the source for multiple Viewlogic tools, not only

ViewSynthesis.  All other ViewSynthesis library files are

automatically derived from VML as follows:

library.VML     (Created by ASIC vendors, Viewlogic Consulting

|             Services, independent consultants or Synthesis end

|             users.)

v

library.CVM     (Compiled by ASIC vendor for general distribution.

|             This embodies all the data from the VML file.  Some

|             consistency checks have been done for phase

|             correctness, completeness of timing information,

|             consistency with symbols, etc.)

v

library.SML     (Distributed by Viewlogic with ViewSynthesis in 2.2.1.

|             Can be compiled by the end user from a CVM file using:

|                    cvm2sml

|             which is distributed with ViewSynthesis 2.2.1 and up.

v

Used in ViewSynthesis

library.SML files are keyed to a specific release of ViewSynthesis,

and may not be usable with other ViewSynthesis releases.  However,

the cvm2sml executable which accompanies a ViewSynthesis release will

transform any released CVM file into SML format appropriate for the

concurrent ViewSynthesis release.

II - Previous ViewSynthesis Library Development Flow:

Beginning with ViewSynthesis version 2.0, there was no officially

approved method of creating synthesis libraries other than through a

VCS contract.  The ViewSynthesis libraries were distributed in a

compiled format as library.LIB.  In the summer of 1992, Viewlogic did

have a Beta program for developing Synthesis libraries in what was

then called the SML format.  This format has no direct relation to the

current library.SML format described above.  Although all officially

supported Beta sites for this program have been converted to VML,

Viewlogic will make every effort to ensure a successful transition for

any remaining library developers.  Please contact ViewLogic ASAP if

you have developed Beta-SML libraries.

III - How to Use cvm2sml

cvm2sml {library}

input files:

requires "library.cvm" file in current directory.

format of "library.cvm" must be compiled VML.

requires viewdraw.ini file in current directory.

the viewdraw.ini file must contain the directories (with alias tags)

of where the "library" design kit is located.

design kit SYM & WIR files.

output file:

writes compiled SML to "library.sml" in current directory.

other requirements:

none - no security code is necessary.

possible errors:

can not find/read "library.cvm" file.

alias required by library is not defined in viewdraw.ini file.

can not find/read design kit directory as specified in viewdraw.ini.

design kit and cvm file are not consistent.

can not write in current directory.

warning messages may be generated, these can be ignored.