# @(#)vj.doc	30.2 3/14/91 AppEng/SCCS  Interphase

#

#

# for vi set ts=4

#

# REVISION HISTORY

# DATE       ID    VER    COMMENT

# 03/13/91 rbrant 30.1    New development/distribution format.

# 10/01/90  jqo   21.3    Renamed README to vj.doc and bumped to same level.

# 08/30/90  JQO   21.3    bumped to new SBU/SCCS level.

# 06/06/90  jqo    1.3    bumped to match RCS version 1.3.

1. SAMPLE 4210 CONFIGURATION LINES:

# Interphase V/SCSI 4210 Jaguar driver

# Install Interphase Jaguar board and daughter card (optional) at SCSI ID#7

# (flags & 0x01) indicates secondary SCSI bus.

# (flags & 0x02) indicates suppressed sync xfer.

# (flags & 0x04) indicates fixed block size for tape

# (flags & 0x08) indicates a forced tape device, maily for MT-02

# (flags & 0x20) indicates a GEMS Genesis Digital Audio Tape device

# (flags & 0x80) indicates a Dorofile device

# (flags & 0xc0) indicates a Dorofile control device

controller vjc0 at vme16d16 ? csr 0x8800 priority 2  vector vjnint 0x56 vjeint 0x57 vjqint 0x58

disk	vj0 at vjc0 drive 0  flags 0	#SCSI_ADDR 0 on bus 0 LUN 0

disk	vj1 at vjc0 drive 8  flags 0	#SCSI_ADDR 1 on bus 0 LUN 0

disk	vj2 at vjc0 drive 9  flags 0	#SCSI_ADDR 1 on bus 0 LUN 1

disk	vj3 at vjc0 drive 16 flags 0	#SCSI_ADDR 2 on bus 0 LUN 0

tape	vj4 at vjc0 drive 32 flags 0	#SCSI_ADDR 4 on bus 0 LUN 0

disk	vj5 at vjc0 drive 1  flags 0x01	#SCSI_ADDR 0 on bus 1 LUN 1

NOTE:

- 'vj??' where ?? serves as a unit (index) number with values from

0, 1, 2, ...

- 'drive ##' where ## equals to (scsi_addr * 8 + lun).

- 'flags @@' where if (@@ & 0x1) is true it indicates secondary scsi

bus.  Also, checking (@@ & 0xc0) and (@@ & 0x80) are for Dorofile.

2. MINOR NO. OF SPECIAL DEV ENTRIES:

- 7 6 5 4 3 2 1 0

+++++++++ ^^^^^

|       |__ Disk partitions from 'a' to 'h'

Unit no. indexes into configuration file vj?? entries.

- For example:

`/dev/vj0a ~ /dev/vj0h` with unit no. 0 should have minor no.

from 0 to 7.

`/dev/vj5a ~ /dev/vj5a` with unit no. 5 should have minor no.

from 40 to 47.

3. MULTIPLE CONTROLLERS:

- This option is set when MULT_MAJOR = 1 (default = 0)

- Allows the use of any number of controllers with any number of devices.

Changes To The Normal vj Installation (do the normal install and then

execute these steps):

- Edit your configuration file in /usr/sys/sun[3 or 4]/conf directory.

- Add an entry for each vj controller (and all the devices hooked

up to it) to be used.  The vjcx entry must be different for each

controller (vjc0, vjc1, vjc2 ...).  The address must be different

for each controller (0x8800, 0x9000, 0x9800 ...).  The vectors

used for each controller must be unique (vjnint 0x56 vjeint 0x57

vjqint 0x58, vjnint 0x58 vjeint 0x59 vjqint 0x5a ...).  The device

names must be unique (controller 0 = vj0 - vj13, controller 1 =

vj14 - vj27 ...).

- Edit the conf.c file in /usr/sys/sun directory.  Each controller

uses a seperate block and character major number defined in the

conf.c file.

- Go to the last entry in the bdevsw table.

- The vj entry should be repeated for each vj controller defined in

the configuration file (these entries must be in sequential order).

- Go to the last entry in the cdevsw table.

- The vj entry should be repeated for each vj controller defined in

the configuration file (these entries must be in sequential order).

NOTE: The last block major number used must be less than the

first character major number used.

- The file vj_major.h must be created in the /usr/sys/sundev directory

and contain the following lines:

#define VJBLKMAJOR xx      xx = first vj blk major number (from conf.c)

#define VJCHRMAJOR xx      xx = first vj chr major number (from conf.c)

- The program vjmknode.c will create all of the dev entries needed for

the vj devices on the system.  Do the following to create the

dev entries after the configuration file has been updated with the

vj entries:

cd /usr/sys/sun[3 or 4]/conf

cp /usr/sys/interphase/drivers/vjmknode.c .

cc -o vjmknode vjmknode.c

vjmknode GENERIC			; replace GENERIC with your configuration

; name

chmod 777 jagmknode

cp jagmknode /dev

cd /dev

jagmknode

- Minor and major numbers have the following meaning.

MINOR NO. OF SPECIAL DEV ENTRIES:

- 7 6 5 4 3 2 1 0

_ +++++++ ^^^^^

|  |       |__ Disk partitions from 'a' to 'h'

| Unit no. indexes into configuration file vj?? entries.

|_Special bit for writing a label.

ex.

controller 1 = blk major no 50   chr major no 104

this is mapped to the first controller in the configuration file

drive 0's minor numbers will go from 0 to 7

...

drive 13's minor numbers will go from 104 to 111

controller 2 = blk major no 51   chr major no 105

this is mapped to the second controller in the configuration file

drive 0's minor numbers will go from 0 to 7

...

drive 13's minor numbers will go from 104 to 111

controller 3 = blk major no 52   chr major no 106

this is mapped to the third controller in the configuration file

drive 0's minor numbers will go from 0 to 7

...

drive 13's minor numbers will go from 104 to 111

controller 4 = blk major no 53   chr major no 107

this is mapped to the fourth controller in the configuration file

drive 0's minor numbers will go from 0 to 7

...

drive 13's minor numbers will go from 104 to 111

...

4. vjutil:

- for SunOs 3.X compile with

cc -O -DOS3 -o vjutil vjutil.c -ltermlib

- for SunOs 4.X compile with

cc -O -o vjutil vjutil.c -ltermlib